3GPP TSG-RAN WG3 Meeting #126	R3-247536

Orlando, USA, 18th–22nd November, 2024

Title:	(TP to TS38.423) - AI/ML support for NR-DC

Source:	Ericsson

Agenda item:	11.4

Document Type:	Discussion and agreement

# 1. Introduction

In this paper, we present a TP to TS 38.423 for AI/ML assisted mobility optimization for NR-DC, based on the proposals in R3-247535.

# 2. TP to XnAP - AI/ML assisted mobility opt. for NR-DC

-------------------------------------- START OF CHANGES ----------------------------------

### 8.3.1	S-NG-RAN node Addition Preparation

#### 8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

-------------------------------------- SKIP UNCHANGED TEXT ----------------------------------

If the S-NODE ADDITION REQUEST message contains the IAB Authorization status IE, the S-NG-RAN node shall, if supported, store it and use it as defined in TS 38.401[2].

If the M-NG-RAN Node PSCell Assumption IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node should take it into account as an indication of the PSCell the M-NG-RAN node assumed when selecting the S-NG-RAN node as an SN.

If the M-NG-RAN Node QoS Flow Cell Group Assumption IE is present in the PDU Session Resource Setup Info – SN terminated IE included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node should take it into account as an indication of the SCGs the M-NG-RAN node assumed when selecting the S-NG-RAN node as an SN.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, report to the M-NG-RAN node after successful SN addition, via the Data Collection Reporting procedure, the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the NG-RAN node1 Measurement ID IE, allocated by the M-NG-RAN node, and the NG-RAN node2 Measurement ID IE, allocated by the S-NG-RAN node.

#### 8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: S-NG-RAN node Addition Preparation, unsuccessful operation

If the S-NG-RAN node is not able to accept any of the bearers or a failure occurs during the S-NG-RAN node Addition Preparation, the S-NG-RAN node sends the S-NODE ADDITION REQUEST REJECT message with an appropriate cause value to the M-NG-RAN node.

If the CHO Information SN Addition IE is included in the S-NODE ADDITION REQUEST message and the PCell ID IE is also included, but the S-NG-RAN node is not able to accept any of the bearers or a failure occurs during the S-NG-RAN node Addition Preparation, the S-NG-RAN node shall, if supported, include the PCell ID IE in the S-NODE ADDITION REQUEST REJECT message.

-------------------------------------- NEXT CHANGE ----------------------------------

### 8.4.13	Data Collection Reporting Initiation

#### 8.4.13.1	General

This procedure is used by an NG-RAN node to request from another NG-RAN node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.4.13.2	Successful Operation

Figure 8.4.13.2-1: Data Collection Reporting Initiation, successful operation

NG-RAN node1 initiates the procedure by sending the DATA COLLECTION REQUEST message to NG-RAN node2 to start information reporting or to stop information reporting. Upon receipt, NG-RAN node2:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and predictions and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and the Report Characteristics for Data Collection IE indicates cell-specific information reporting, the Cell To Report List for Data Collection IE shall be included.

If NG-RAN node2 is capable of providing all of the requested information, it shall initiate the information reporting as requested by NG-RAN node1 and respond with the DATA COLLECTION RESPONSE message.

If NG-RAN node2 is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE or the Cell Measurement Initiation Result List IE or both in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The NG-RAN node2 shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Requested Prediction Time IE in the DATA COLLECTION REQUEST message is present, it indicates the specific point in time to which the prediction of the requested information applies. The NG-RAN node2 shall take it into account when generating the requested predicted information.

If the UE Trajectory Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE trajectory collection and reporting. NG-RAN node2 shall report the UE trajectory only once. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Trajectory IE;

-	the number of visited cells within NG-RAN node2 is equal to the value of the Number of Visited Cells IE, if included;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to a cell belonging to an NG-RAN node different from NG-RAN node2.

The result of the UE trajectory collection is reported at the next available DATA COLLECTION UPDATE message.

If the UE Performance Collection Configuration IE is present in the DATA COLLECTION REQUEST message, the NG-RAN node2 shall take it into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 or the time since the successful addition of NG-RAN node2 as an SN is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell;

-	NG-RAN node2 is released as an SN serving the UE.

The result of the UE performance collection is reported at the next available DATA COLLECTION UPDATE message.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects NG-RAN node2 performs measurements or predictions on. NG-RAN node2 shall include in the DATA COLLECTION UPDATE message:

-	the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE, included in the Predicted Radio Resource Status IE, if the first bit, "Predicted Radio Resource Status" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted Number of Active UEs IE, if the second bit, "Predicted Number of Active UEs" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Predicted RRC Connections IE, if the third bit, "Predicted RRC Connections" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput DL IE, if the fourth bit, "Average UE Throughput DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average UE Throughput UL IE, if the fifth bit, "Average UE Throughput UL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Delay IE, if the sixth bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Average Packet Loss DL IE, if the seventh bit, "Average Packet Loss DL" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Energy Cost IE, if the eighth bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the Measured UE Trajectory IE, if the ninth bit, "Measured UE Trajectory" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

-	the F1 Propagation Delay IE, if the tenth bit, "F1 Propagation Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by NG-RAN node2.

#### 8.4.13.3	Unsuccessful Operation

Figure 8.4.13.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, NG-RAN node2 shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

-------------------------------------- NEXT CHANGE ----------------------------------

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Selected PLMN | O |  | PLMN Identity 9.2.2.4 | The selected PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Mobility Restriction List | O |  | 9.2.3.53 |  | YES | ignore |
| Index to RAT/Frequency Selection Priority | O |  | 9.2.3.23 |  | YES | reject |
| PDU Session Resources To Be Added List |  | 1 |  |  | YES | reject |
| >PDU Session Resources To Be Added Item |  | 1 .. <maxnoofPDUSessions> |  | NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE nor the PDU Session Resource Setup Info – MN terminated IE is present in a PDU Session Resources To Be Added Item IE, abnormal conditions as specified in clause 8.3.1.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>S-NG-RAN node PDU Session Aggregate Maximum Bit Rate | O |  | PDU Session Aggregate Maximum Bit Rate 9.2.3.69 |  | – |  |
| >>PDU Session Resource Setup Info – SN terminated | O |  | 9.2.1.5 |  | – |  |
| >>PDU Session Resource Setup Info – MN terminated | O |  | 9.2.1.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-ConfigInfo message as defined in subclause 11.2.2 of TS 38.331 [10] | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | 9.2.3.81 |  | YES | ignore |
| Requested Split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for Split SRBs are requested. | YES | reject |
| PCell ID | O |  | Global NG-RAN Cell Identity 9.2.2.27 |  | YES | reject |
| Desired Activity Notification Level | O |  | 9.2.3.77 |  | YES | ignore |
| Available DRB IDs | C-ifSNterminated |  | DRB List 9.2.1.29 | Indicates the list of DRB IDs that the S-NG-RAN node may use for SN-terminated bearers. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Uplink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Uplink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Uplink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. If the S-NG-RAN node Maximum Integrity Protected Data Rate Downlink IE is not present, this IE applies to both UL and DL. | YES | reject |
| S-NG-RAN node Maximum Integrity Protected Data Rate Downlink | O |  | Bit Rate 9.2.3.4 | The S-NG-RAN node Maximum Integrity Protected Data Rate Downlink is a portion of the UE’s Maximum Integrity Protected Data Rate in the Downlink, which is enforced by the S-NG-RAN node for the UE’s SN terminated PDU sessions. | YES | reject |
| Location Information at S-NODE reporting | O |  | ENUMERATED (pscell, ...) | Indicates that the user’s Location Information at S-NODE is to be provided. | YES | ignore |
| MR-DC Resource Coordination Information | O |  | 9.2.2.33 | Information used to coordinate resource utilisation between M-NG-RAN node and S-NG-RAN node. | YES | ignore |
| Masked IMEISV | O |  | 9.2.3.32 |  | YES | ignore |
| NE-DC TDM Pattern | O |  | 9.2.2.38 |  | YES | ignore |
| SN Addition Trigger Indication | O |  | ENUMERATED (SN change, inter-MN HO, intra-MN HO, ...) | This IE indicates the trigger for S-NG-RAN node Addition Preparation procedure | YES | reject |
| Trace Activation | O |  | 9.2.3.55 |  | YES | ignore |
| Requested Fast MCG recovery via SRB3 | O |  | ENUMERATED (true, ...) | Indicates that the resources for fast MCG recovery via SRB3 are requested. | YES | ignore |
| UE Radio Capability ID | O |  | 9.2.3.138 |  | YES | reject |
| Source NG-RAN Node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source NG-RAN node, or the source SN in e.g. NR-DC to NR-DC (conditional) handover. | YES | ignore |
| Management Based MDT PLMN List | O |  | MDT PLMN List 9.2.3.133 |  | YES | ignore |
| UE History Information | O |  | 9.2.3.64 |  | YES | ignore |
| UE History Information from the UE | O |  | 9.2.3.110 |  | YES | ignore |
| PSCell Change History | O |  | ENUMERATED (reporting full history, ...) |  | YES | ignore |
| IAB Node Indication | O |  | ENUMERATED (true, ...) |  | YES | reject |
| No PDU Session Indication | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | ignore |
| CHO Information SN Addition | O |  |  |  | YES | reject |
| >Source M-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >Source M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source M-NG-RAN node | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  | YES | ignore |
| Conditional PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, ...) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >S-CPAC Request Information | O |  | 9.2.3.192 | Indicates that SN addition is for S-CPAC preparation. | YES | reject |
| >S-CPAC Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for S-CPAC is requested. | YES | ignore |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | reject |
| F1-terminating IAB-donor Indicator | O |  | ENUMERATED (true, ...) | This IE applies only if the UE is an IAB-MT. | YES | reject |
| Selected NID | O |  | NID 9.2.2.65 | This IE, together with the Selected PLMN IE, indicates the SNPN proposed for the SCG to the S-NG-RAN node. | YES | ignore |
| QMC Coordination Request | O |  | 9.2.3.197 | This IE contains information for managing configuration and reporting of one or more QoE and/or RAN visible QoE measurements at the S-NG-RAN node subject to addition. | YES | ignore |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| Source M-NG-RAN node ID | O |  | Global NG-RAN Node ID 9.2.2.3 | The NG-RAN Node ID of the source M-NG-RAN node in e.g. NR-DC to NR-DC handover. | YES | ignore |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | Ignore |
| Data Collection ID | O |  | 9.2.3.184 |  | YES | Ignore |
| M-NG-RAN Node PSCell Assumption | O |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates the M-NG-RAN node assumed PSCell. | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

-------------------------------------- NEXT CHANGE ----------------------------------

#### 9.1.3.26	DATA COLLECTION REQUEST

This message is sent by NG-RAN node1 to NG-RAN node2 to initiate the requested information reporting according to the parameters given in the message.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory, Tenth Bit = F1 Propagation Delay. Other bits are ignored by the NG-RAN node2. | YES | reject |
| Cell To Report List for Data Collection |  | 0..1 |  | Cell ID list to which the request applies. | YES | ignore |
| >Cell To Report Item for Data Collection |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| Requested Prediction Time | O |  | INTEGER (1..60, ...) | For one time reporting, it indicates the point in time, measured from reception of the DATA COLLECTION REQUEST message, for which predictions are provided. In periodic reporting, for each subsequent DATA COLLECTION UPDATE message, the point in time is shifted by the reporting periodicity. (unit: second) | YES | ignore |
| UE Trajectory Collection Configuration | O |  | 9.2.3.185 |  | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.2.3.186 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |

#### 9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory, Seventh Bit = F1 Propagation Delay. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxFailedCellMeasObjects | Maximum number of measurement objects that can fail per cell. Value is 124. |
| maxFailedMeasPerNode | Maximum number of measurement objects that can fail per node. Value is 124. |

-------------------------------------- NEXT CHANGE ----------------------------------

#### 9.1.3.29	DATA COLLECTION UPDATE

This message is sent by NG-RAN node2 to NG-RAN node1 to report the requested information.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Cell Measurement Result for Data Collection List |  | 0..1 |  |  | YES | ignore |
| >Cell Info Result for Data Collection Item |  | 1 .. < maxnoofCellsinNG-RANnode > |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Predicted Radio Resource Status | O |  | Radio Resource Status 9.2.2.50 | The IE only includes the SSB Area Radio Resource Status List IE, excluding the DL scheduling PDCCH CCE usage IE and UL scheduling PDCCH CCE usage IE. | – |  |
| >>Predicted Number of Active UEs | O |  | Number of Active UEs 9.2.2.62 |  | – |  |
| >>Predicted RRC Connections | O |  | RRC Connections 9.2.2.56 |  | – |  |
| UE Associated Info Result List |  | 0..1 |  |  | YES | ignore |
| >UE Associated Info Result Item |  | 1 .. < maxnoofUEReports > |  |  | – |  |
| >>UE Assistant Identifier | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | NG-RAN node UE XnAP ID allocated by NG-RAN node1. | – |  |
| >>UE Performance | O |  | 9.2.3.179 |  | – |  |
| >>Measured UE Trajectory | O |  | 9.2.3.182 | It contains information about cells that a UE has connected to. | – |  |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |
| >F1 Propagation Delay | O |  | FFS | The node level measured F1 Delay. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by a NG-RAN node. Value is 16384. |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |

-------------------------------------- NEXT CHANGE ----------------------------------

#### 9.2.1.5	PDU Session Resource Setup Info – SN terminated

This IE contains information for the addition of S-NG-RAN node resources related to a PDU session for DRBs configured with an SN terminated bearer option.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL NG-U UP TNL Information at UPF | M |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs | – |  |
| PDU Session Type | M |  | 9.2.3.19 |  | – |  |
| Network Instance | O |  | 9.2.3.85 | This IE shall be ignored if the Common Network Instance IE is present. | – |  |
| QoS Flows To Be Setup List |  | 1 |  |  | – |  |
| >QoS Flow To Be Setup Item |  | 1 .. <maxnoofQoSFlows> |  |  | – |  |
| >>QoS Flow Identifier | M |  | 9.2.3.10 |  | – |  |
| >>QoS Flow Level QoS Parameters | M |  | 9.2.3.5 | For GBR QoS flows, this IE contains GBR QoS flow information as received at NG-C | – |  |
| >>Offered GBR QoS Flow Information | O |  | GBR QoS Flow Information 9.2.3.6 | This IE contains M-Node offered GBR QoS Flow Information. | – |  |
| >>TSC Traffic Characteristics | O |  | 9.2.3.114 | Traffic pattern information associated with the QFI. Details in TS 23.501 [7]. | YES | ignore |
| >>Redundant QoS Flow Indicator | O |  | 9.2.3.118 |  | YES | ignore |
| >>M-NG-RAN Node QoS Flow Cell Group Assumption | O |  | ENUMERATED (scg, …) | Indicates the M-NG-RAN node assumed cell group serving the QoS Flow. | YES | ignore |
| Data Forwarding and Offloading Info from source NG-RAN node | O |  | 9.2.1.17 |  | – |  |
| Security Indication | O |  | 9.2.3.52 |  | – |  |
| Security Result | O |  | 9.2.3.67 | Indicates security activation status in MN. | YES | reject |
| Common Network Instance | O |  | 9.2.3.92 |  | YES | ignore |
| Default DRB Allowed | O |  | 9.2.3.93 |  | YES | ignore |
| Split Session Indicator | O |  | 9.2.3.94 |  | YES | reject |
| Non-GBR Resources Offered | O |  | 9.2.3.98 |  | YES | ignore |
| Redundant UL NG-U UP TNL Information at UPF | O |  | UP Transport Layer Information 9.2.3.30 | UPF endpoint of the NG-U transport bearer. For delivery of UL PDUs for the redundant transmission. | YES | ignore |
| Redundant Common Network Instance | O |  | Common Network Instance 9.2.3.92 |  | YES | ignore |
| Redundant PDU Session Information | O |  | 9.2.3.112 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows. Value is 64 |

-------------------------------------- NEXT CHANGE ----------------------------------

#### 9.2.3.179	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average UE Throughput DL | O |  | Bit Rate 9.2.3.4 |  |
| Average UE Throughput UL | O |  | Bit Rate 9.2.3.4 |  |
| Average Packet Delay | O |  | 9.2.3.187 |  |
| Average Packet Loss DL | O |  | Packet Loss Rate 9.2.3.11 |  |
| PSCell ID | C-ifSN |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates the PSCell where the UE performance was collected. |

| Condition | Explanation |
| --- | --- |
| ifSN | This IE shall be present if the UE performance was collected by the SN in NR-DC. |

-------------------------------------- ASN.1 CHANGES START HERE ----------------------------------

### 9.3.4	PDU Definitions

(skip unchanged)

id-MIAB-MT-BAP-Address,

id-S-CPAC-Request,

id-sk-Counter,

id-Source-M-NG-RANnodeID,

id-SourceSN-to-TargetSN-QMCInfo,

id-RegistrationRequestForDataCollection,

id-ReportCharacteristicsForDataCollection,

id-ReportingPeriodicityForDataCollection,

id-NodeAssociatedInfoResult,

id-SLPositioning-Ranging-Services-Info,

id-PDUSessionsListToBeReleased-UPError,

id-UserPlaneFailureIndication,

id-SRSPositioningConfigOrActivationRequest,

id-NRPPaPositioningInformation,

id-M-NG-RANnodePSCellAssumption,

maxnoofCellsinNG-RANnode,

maxnoofDRBs,

maxnoofPDUSessions,

maxnoofQoSFlows,

maxnoofServedCellsIAB,

maxnoofTrafficIndexEntries,

maxnoofTLAsIAB,

maxnoofBAPControlPDURLCCHs,

maxnoofServingCells,

maxnoofSSBAreas

FROM XnAP-Constants;

-------------------------------------- NEXT CHANGE ----------------------------------

-- **************************************************************

--

#### -- S-NODE ADDITION REQUEST

--

-- **************************************************************

SNodeAdditionRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container	{{ SNodeAdditionRequest-IEs}},

...

}

SNodeAdditionRequest-IEs XNAP-PROTOCOL-IES ::= {

{ ID id-M-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE mandatory}|

{ ID id-UESecurityCapabilities				CRITICALITY reject		TYPE UESecurityCapabilities						PRESENCE mandatory}|

{ ID id-s-ng-RANnode-SecurityKey			CRITICALITY reject		TYPE S-NG-RANnode-SecurityKey					PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUE-AMBR					CRITICALITY reject		TYPE UEAggregateMaximumBitRate					PRESENCE mandatory}|

{ ID id-selectedPLMN						CRITICALITY ignore		TYPE PLMN-Identity									PRESENCE optional }|

{ ID id-MobilityRestrictionList				CRITICALITY ignore		TYPE MobilityRestrictionList						PRESENCE optional }|

{ ID id-indexToRatFrequSelectionPriority	CRITICALITY reject		TYPE RFSP-Index										PRESENCE optional }|

{ ID id-PDUSessionToBeAddedAddReq			CRITICALITY reject		TYPE PDUSessionToBeAddedAddReq					PRESENCE mandatory}|

{ ID id-MN-to-SN-Container					CRITICALITY reject		TYPE OCTET STRING									PRESENCE mandatory}|

{ ID id-S-NG-RANnodeUEXnAPID				CRITICALITY reject		TYPE NG-RANnodeUEXnAPID								PRESENCE optional }|

{ ID id-ExpectedUEBehaviour					CRITICALITY ignore		TYPE ExpectedUEBehaviour							PRESENCE optional }|

{ ID id-requestedSplitSRB					CRITICALITY reject		TYPE SplitSRBsTypes									PRESENCE optional }|

{ ID id-PCellID								CRITICALITY reject		TYPE GlobalNG-RANCell-ID							PRESENCE optional }|

{ ID id-DesiredActNotificationLevel			CRITICALITY ignore		TYPE DesiredActNotificationLevel				PRESENCE optional }|

{ ID id-AvailableDRBIDs						CRITICALITY reject		TYPE DRB-List										PRESENCE conditional}

-- This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. --|

{ ID id-S-NG-RANnodeMaxIPDataRate-UL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-S-NG-RANnodeMaxIPDataRate-DL		CRITICALITY reject		TYPE BitRate										PRESENCE optional }|

{ ID id-LocationInformationSNReporting		CRITICALITY ignore		TYPE LocationInformationSNReporting			PRESENCE optional }|

{ ID id-MR-DC-ResourceCoordinationInfo		CRITICALITY ignore		TYPE MR-DC-ResourceCoordinationInfo			PRESENCE optional }|

{ ID id-MaskedIMEISV						CRITICALITY ignore		TYPE MaskedIMEISV									PRESENCE optional }|

{ ID id-NE-DC-TDM-Pattern					CRITICALITY ignore		TYPE NE-DC-TDM-Pattern								PRESENCE optional }|

{ ID id-S-NG-RANnode-Addition-Trigger-Ind	CRITICALITY reject		TYPE S-NG-RANnode-Addition-Trigger-Ind		PRESENCE optional }|

{ ID id-TraceActivation						CRITICALITY ignore		TYPE TraceActivation								PRESENCE optional }|

{ ID id-RequestedFastMCGRecoveryViaSRB3		CRITICALITY ignore		TYPE RequestedFastMCGRecoveryViaSRB3			PRESENCE optional }|

{ ID id-UERadioCapabilityID					CRITICALITY reject		TYPE UERadioCapabilityID							PRESENCE optional }|

{ ID id-SourceNG-RAN-node-ID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-ManagementBasedMDTPLMNList			CRITICALITY ignore		TYPE MDTPLMNList									PRESENCE optional }|

{ ID id-UEHistoryInformation				CRITICALITY ignore		TYPE UEHistoryInformation							PRESENCE optional }|

{ ID id-UEHistoryInformationFromTheUE		CRITICALITY ignore		TYPE UEHistoryInformationFromTheUE				PRESENCE optional }|

{ ID id-PSCellChangeHistory					CRITICALITY ignore		TYPE PSCellChangeHistory							PRESENCE optional }|

{ ID id-IABNodeIndication					CRITICALITY reject		TYPE IABNodeIndication								PRESENCE optional }|

{ ID id-NoPDUSessionIndication				CRITICALITY ignore		TYPE NoPDUSessionIndication						PRESENCE optional }|

{ ID id-CHOinformation-AddReq				CRITICALITY reject		TYPE CHOinformation-AddReq						PRESENCE optional }|

{ ID id-SCGActivationRequest				CRITICALITY ignore		TYPE SCGActivationRequest							PRESENCE optional }|

{ ID id-CPAInformationRequest				CRITICALITY reject		TYPE CPAInformationRequest						PRESENCE optional }|

{ ID id-S-NG-RANnodeUE-Slice-MBR			CRITICALITY reject		TYPE UESliceMaximumBitRateList					PRESENCE optional }|

{ ID id-F1-terminatingIAB-donorIndicator	CRITICALITY reject		TYPE F1-terminatingIAB-donorIndicator			PRESENCE optional }|

{ ID id-SelectedNID							CRITICALITY	ignore		TYPE NID											PRESENCE optional }|

{ ID id-QMCCoordinationRequest				CRITICALITY ignore		TYPE QMCCoordinationRequest						PRESENCE optional }|

{ ID id-SourceSN-to-TargetSN-QMCInfo		CRITICALITY ignore		TYPE QMCConfigInfo									PRESENCE optional }|

{ ID id-IABAuthorizationStatus				CRITICALITY ignore		TYPE IABAuthorizationStatus						PRESENCE optional }|

{ ID id-Source-M-NG-RANnodeID				CRITICALITY ignore		TYPE GlobalNG-RANNode-ID							PRESENCE optional }|

{ ID id-DataCollectionID					CRITICALITY ignore		TYPE DataCollectionID								PRESENCE optional }|

{ ID id-M-NG-RANnodePSCellAssumption		CRITICALITY ignore		TYPE GlobalNG-RANCell-ID							PRESENCE optional },

...

}

PDUSessionToBeAddedAddReq ::= SEQUENCE (SIZE(1..maxnoofPDUSessions)) OF PDUSessionToBeAddedAddReq-Item

PDUSessionToBeAddedAddReq-Item ::= SEQUENCE {

pduSessionId			PDUSession-ID,

s-NSSAI					S-NSSAI,

sN-PDUSessionAMBR		PDUSessionAggregateMaximumBitRate			OPTIONAL,

sn-terminated			PDUSessionResourceSetupInfo-SNterminated	OPTIONAL,

mn-terminated			PDUSessionResourceSetupInfo-MNterminated	OPTIONAL,

-- NOTE: If neither the PDU Session Resource Setup Info – SN terminated IE

-- nor the PDU Session Resource Setup Info – MN terminated IE is present,

-- abnormal conditions as specified in clause 8.3.1.4 apply.

iE-Extension			ProtocolExtensionContainer { {PDUSessionToBeAddedAddReq-Item-ExtIEs} }	OPTIONAL,

...

}

PDUSessionToBeAddedAddReq-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

RequestedFastMCGRecoveryViaSRB3 ::= ENUMERATED {true, ...}

-------------------------------------- NEXT CHANGE ----------------------------------

### 9.3.5	Information Element definitions

(skip unchanged)

id-MobileIABCell,

id-XR-Bcast-Information,

id-MaximumDataBurstVolume,

id-CPAC-Preparation-Type,

id-MN-only-MDT-collection,

id-BarringExemptionforEmerCallInfo,

id-Transmission-Bandwidth-asymmetric,

id-NRPPaPositioningInformation,

id-PSCellID,

id-F1PropagationDelay,

id-M-NG-RANnodeQoSFlowCellGroupAssumption,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

-------------------------------------- NEXT CHANGE ----------------------------------

#### -- F

(skip unchanged)

F1PropagationDelay ::= FFS

-------------------------------------- NEXT CHANGE ----------------------------------

#### -- M

(skip unchanged)

M-NG-RANnodeQoSFlowCellGroupAssumption ::= ENUMERATED {scg, ...}

-------------------------------------- NEXT CHANGE ----------------------------------

#### -- N

(skip unchanged)

NodeAssociatedInfoResult ::= SEQUENCE {

energyCost				EnergyCost			OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { NodeAssociatedInfoResult-ExtIEs} } OPTIONAL,

...

}

NodeAssociatedInfoResult-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-F1PropagationDelay		CRITICALITY ignore	EXTENSION F1PropagationDelay	PRESENCE optional},

...

}

-------------------------------------- NEXT CHANGE ----------------------------------

#### -- P

(skip unchanged)

-- **************************************************************

--

###### -- PDU Session Resource Setup Info - SN terminated

--

-- **************************************************************

PDUSessionResourceSetupInfo-SNterminated ::= SEQUENCE {

uL-NG-U-TNLatUPF				UPTransportLayerInformation,

pduSessionType					PDUSessionType,

pduSessionNetworkInstance		PDUSessionNetworkInstance																	OPTIONAL,

qosFlowsToBeSetup-List			QoSFlowsToBeSetup-List-Setup-SNterminated,

dataforwardinginfofromSource	DataforwardingandOffloadingInfofromSource													OPTIONAL,

securityIndication				SecurityIndication																			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {PDUSessionResourceSetupInfo-SNterminated-ExtIEs} }	OPTIONAL,

...

}

PDUSessionResourceSetupInfo-SNterminated-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SecurityResult						CRITICALITY reject	EXTENSION SecurityResult							PRESENCE optional}|

{ ID id-PDUSessionCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-DefaultDRB-Allowed					CRITICALITY ignore	EXTENSION DefaultDRB-Allowed						PRESENCE optional}|

{ ID id-SplitSessionIndicator				CRITICALITY reject	EXTENSION SplitSessionIndicator					PRESENCE optional}|

{ ID id-NonGBRResources-Offered				CRITICALITY ignore	EXTENSION NonGBRResources-Offered				PRESENCE optional}|

{ ID id-Redundant-UL-NG-U-TNLatUPF			CRITICALITY	ignore	EXTENSION UPTransportLayerInformation			PRESENCE optional}|

{ ID id-RedundantCommonNetworkInstance		CRITICALITY ignore	EXTENSION PDUSessionCommonNetworkInstance		PRESENCE optional}|

{ ID id-RedundantPDUSessionInformation		CRITICALITY ignore	EXTENSION RedundantPDUSessionInformation		PRESENCE optional},

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated ::= SEQUENCE (SIZE(1..maxnoofQoSFlows)) OF QoSFlowsToBeSetup-List-Setup-SNterminated-Item

QoSFlowsToBeSetup-List-Setup-SNterminated-Item ::= SEQUENCE {

qfi								QoSFlowIdentifier,

qosFlowLevelQoSParameters		QoSFlowLevelQoSParameters,

offeredGBRQoSFlowInfo			GBRQoSFlowInfo																						OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs} }	OPTIONAL,

...

}

QoSFlowsToBeSetup-List-Setup-SNterminated-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-TSCTrafficCharacteristics		CRITICALITY ignore	EXTENSION TSCTrafficCharacteristics 	PRESENCE optional}|

{ ID id-RedundantQoSFlowIndicator		CRITICALITY ignore	EXTENSION RedundantQoSFlowIndicator	PRESENCE optional}|

{ ID id-M-NG-RANnodeQoSFlowCellGroupAssumption		CRITICALITY ignore	EXTENSION M-NG-RANnodeQoSFlowCellGroupAssumption		PRESENCE optional},

...

}

-------------------------------------- NEXT CHANGE ----------------------------------

#### -- U

(skip unchanged)

UEPerformance ::= SEQUENCE {

dL-UE-AverageThroughput						BitRate					OPTIONAL,

uL-UE-AverageThroughput						BitRate					OPTIONAL,

uE-AveragePacketDelay						AveragePacketDelay		OPTIONAL,

uE-AveragePacketLossDL						PacketLossRate			OPTIONAL,

iE-Extensions								ProtocolExtensionContainer { { UEPerformance-ExtIEs} } OPTIONAL,

...

}

UEPerformance-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-PSCellID		CRITICALITY ignore	EXTENSION GlobalNG-RANCell-ID 	PRESENCE conditional},

-- This IE shall be present if the UE performance was collected by the SN in NR-DC.

...

}

-------------------------------------- NEXT CHANGE ----------------------------------

### 9.3.7	Constant definitions

id-UserPlaneFailureIndication																		ProtocolIE-ID ::= 469

id-MN-only-MDT-collection																			ProtocolIE-ID ::= 470

id-BarringExemptionforEmerCallInfo																	ProtocolIE-ID ::= 471

id-Transmission-Bandwidth-asymmetric																ProtocolIE-ID ::= 472

id-SRSPositioningConfigOrActivationRequest															ProtocolIE-ID ::= 473

id-NRPPaPositioningInformation																		ProtocolIE-ID ::= 474

id-M-NG-RANnodePSCellAssumption																		ProtocolIE-ID ::= xx1

id-PSCellID																							ProtocolIE-ID ::= xx2

id-F1PropagationDelay																				ProtocolIE-ID ::= xx3

id-M-NG-RANnodeQoSFlowCellGroupAssumption															ProtocolIE-ID ::= xx4

END

-- ASN1STOP

-------------------------------------- END OF CHANGES ----------------------------------