3GPP TSG-RAN WG3 Meeting #126	R3-247541

# Orlando, USA, 18th - 22th Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1961 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NTN time-based handover over S1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CMCC, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-18, the S1-based handover with time-based trigger condition was introduced for IoT NTN, and only the Early data forwarding mechanism was introduced in the ENB and MME EARLY STATUS TRANSFER messages. While, the late data forwarding mechanism has not been supported. And the CHO over X2 could be taken as the reference. For the CHO over X2, according to the description in step 11a and 11b, after receiving the HANDOVER SUCCESS message from the target eNB, the source eNB shall send the SN STATUS TRANSFER to perform the late data forwarding. 11a/b. The target eNB sends the HANDOVER SUCCESS message to the source eNB to inform that the UE has successfully accessed the target cell. In return, the source eNB sends the SN STATUS TRANSFER message. NOTE 2: Late data forwarding may be initiated as soon as the source eNB receives the HANDOVER SUCCESS message. Considering the S1 time-based HO, before triggering the eNB STAUTS TRASNFER message and MME STAUTS TRASNFER message to perform the data forwarding, the source eNB should be informed by the target eNB that the S1-based handover has succeeded via the HANDOVER NOTIFY message and HANDOVER SUCCESS message. However, in current specification, the HANDOVER SUCCESS message only applies for DAPS handover. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) on S1-based handover with time-based trigger condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description of Handover SUCCESS message to support the S1-based handover with time-based trigger condition for NTN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The late data forwarding for S1-based handover with time-based trigger condition cannot work properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

8.4.8	Handover Success

8.4.8.1	General

The Handover Success procedure is used during a DAPS Handover and a S1 handover with time-based trigger condition, to inform the source eNB that the UE has successfully accessed the target eNB. The procedure uses UE-associated signalling.

8.4.8.2	Successful Operation

Figure 8.4.8.2-1: Handover Success

The MME initiates the procedure by sending the HANDOVER SUCCESS message to the source eNB.

8.4.8.3	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source eNB shall ignore the message.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>