3GPP TSG-RAN WG3 #126	R3-247559

# Orlando, U.S.A., 18th-22nd November 2024

Agenda Item:	17.3

Source:	Ericsson, Deutsche Telekom, China Unicom

Title: 	Xn impact of On-demand SIB1 for UEs in idle/inactive mode

Document for:	Discussion and Approval

# 1	Background

A work item on “Enhancements of network energy savings for NR” was agreed in [1]. The Work Item Description (WID) was revised in [2]. According to the revised WID, the objectives of the work item are the following:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we discuss issues and solutions regarding Objective 2 of the work item (see highlighted text above), focusing on the impact on Xn.

# 2	Discussion

In the following, we discuss the main principles of the inter NG-RAN node signaling to enable on-demand SIB1 (OD-SIB1) transmission, considering non-split architecture.

## 2.1	General scenario

Figure 1 illustrates an example deployment scenario for OD-SIB1 transmission, where the “NES Cell” of gNB1 is meant to operate in OD-SIB1 transmission mode and the “Cell A” of gNB2 provides the UL Wake-Up Signal (WUS) configuration (that is needed to request SIB1 transmission in the NES Cell) to UEs. The scenario corresponds to Case 2 as defined by RAN1, assumed for normative work according to the WID.

Figure 1: An example deployment scenario for OD-SIB1 transmission (corresponding to Case 2).

We assume that, before the NES Cell operates in OD-SIB1 transmission mode, gNB1 provides the UL WUS configuration of its NES Cell to gNB2 in step 1. In step 2, gNB2 provides the UL WUS configuration of the NES Cell to UEs camping on (or connected to) Cell A. In step 3, when needed, a UE (in RRC Idle/Inactive mode) sends the UL WUS to the NES Cell and, in response, receives SIB1 from the NES Cell in step 4.

## 2.2	Previous agreements

At the RAN3#125 meeting, the following was agreed [3]:

Case 2 can be supported with the below details:

- UL WUS configuration is decided by NES Cell DU.

- UL WUS configuration transmission from NES Cell DU to NES Cell CU. NES gNB-CU sends UL WUS configuration to Cell A gNB-CU over Xn Interface.

- Upon the reception of the UL WUS configuration, Cell A can provide feedback to NES Cell on whether the WUS configuration Transmission for NES Cell is accepted in Cell A (e.g., confirm, reject). FFS if Cell A broadcasts it right away or not.

Moreover, at the RAN3#125-bis meeting, the following agreements were captured [4]:

The UL WUS configuration will be transferred over Xn via a new class1 defined procedure.

Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

Hence, RAN3 has agreed to introduce a new Class 1 procedure for signaling the UL WUS configuration from NES-Cell gNB to Cell-A gNB, and that the Cell-A gNB shall provide feedback to the NES-Cell gNB, e.g., accept/reject the UL WUS configuration provisioning. In addition, it was agreed that the Cell-A gNB can stop providing the UL WUS configuration, and signal to the NES-Cell gNB about this interruption.

## 2.3	Further discussion on the new procedure(s)

### 2.3.1	Design of the new Class 1 procedure as start/stop request

Despite recent agreement on the new class 1 Xn procedure, the details are still to be specified.

As agreed, the new Class 1 procedure over Xn will carry the UL WUS configuration. The UL WUS configuration is exchanged over Xn only when the NES-Cell gNB intends to put the NES Cell into OD-SIB1 operation and cells of another gNB must assist. Hence, the new Class 1 procedure is a procedure that is initiated by the NES-Cell gNB.

Observation 1: The new Class 1 procedure is initiated by the NES-Cell gNB to start the UL WUS configuration provision.

We further propose to design the procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The Cell-A gNB can accept or reject the (start) request, but the Cell- A gNB cannot reject the “stop” request, which is similar to, e.g., the PDU session resource release procedure, where the “release” will never fail. One way to distinguish accept and reject is to introduce different response messages over Xn, e.g., a “response” message and a “failure” message, as assumed in the TP below. Another way is to define a dedicated IE in the response message. We favor the first option.

Proposal 1: RAN3 to agree the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Since the Cell-A gNB can have many cells and cannot judge which of its cell(s) must serve as Cell A, as discussed in [5], the NES-Cell gNB shall further indicate in its (start) request to the Cell-A gNB which cell(s) of the Cell-A gNB should assist in providing the UL WUS configuration.

Proposal 2: RAN3 to agree that in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should serve as Cell A (assisting in providing the UL WUS configuration).

### 2.3.2	Design of the new Class 2 procedure to signal the feedback regarding the UL WUS configuration provisioning

It is understood that Cell A feedback (before and during OD-SIB1 operation) is crucial for the NES-Cell gNB to ensure that the NES Cell is still accessible for idle/inactive UEs when it goes into OD-SIB1 transmission. But tt is up to NES-Cell gNB to determine when it goes to OD-SIB1.

To simplify discussions, the former type of feedback, which is sent before OD-SIB1 transmission is activated, could be referred to as “early feedback”, while the latter type of feedback, which is sent while OD-SIB1 transmission is active, could be referred to as “late feedback”.

Furthermore, it is yet to be agreed how (early and late) feedback regarding the UL WUS configuration provisioning is to be sent from the Cell-A gNB to the NES-Cell gNB. These open issues are addressed in the following, and the signaling steps are again summarized thereafter.

As reflected in the agreements, the NES-Cell gNB must be aware of the UL WUS configuration provisioning status at the Cell-A gNB and must therefore be informed about changes in the status. Especially status changes during OD-SIB1 operation at the NES Cell (late feedback, e.g., provisioning interrupted), must be provided by the Cell-A gNB to the NES-Cell gNB, as agreed in RAN3.

As specified in TS 38.331 (see IE DownlinkConfigCommonSIB), the SI modification period, expressed in number of radio frames, follows as: m = modificationPeriodCoeff * defaultPagingCycle. The modificationPeriodCoeff is between 2 and 16, and the defaultPagingCycle is between 32 and 256 radio frames. This means that the SI modification period m can be up to 4096 radio frames (≈ 41 seconds), and it should not be optimized for the sake of OD-SIB1 transmission alone. For the defaultPagingCycle, for example, large values are common, since they improve the UE energy efficiency.

Observation 2: The modification period for system information can be up to 41 seconds.

The new Class 1 procedure for UL WUS configuration provision should not be kept open for long, certainly not as long as 41 seconds or more (or until the SI update has reached all UEs in Cell A and the UL WUS configuration provisioning in Cell A is ensured). To avoid unfavorable protocol design, the response to the request should be sent promptly from Cell-A gNB to NES-Cell gNB, indicating that Cell-A gNB will perform the UL WUS configuration provision.

Observation 3: The response to a request for an UL WUS configuration provisioning should be signaled promptly by Cell-A gNB to the NES Cell gNB in the class 1 procedure.

The NES cell should not go to OD-SIB1 mode after the positive response in the class 1 UL WUS configuration provision procedure, without knowing at least  UL WUS configuration has been successfully broadcasted by one Cell-A gNB. to ensure the NES cell cancontinuously serve the legacy UE.

Observation 4: The NES cell should not go to the OD-SIB1 mode without knowing that its UL WUS configuration has been successfully broadcasted at least by one Cell-A gNB.

The agreed new Class 2 procedure should be used to convey feedback regarding the UL WUS configuration provisioning status in Cell A to inform the NES Cell gNB about starting of the successful provisioning.

Proposal 3: RAN3 to agree the new Class 2 procedure is used to convey feedback regarding the UL WUS configuration provisioning status as “started”/“provided” from Cell-A gNB to NES-Cell gNB. Only after the NES-Cell receive the “started”/”provided” indication, it can go to the OD-SIB1 mode.

### 2.3.3	Outline of the signaling flow for the two new procedures

We now summarize of the signaling steps and the interaction of the two procedures described above.

Figure 2 shows how the procedures can be used by a NES Cell gNB (gNB1) for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission. In a step 1, gNB1 generates an UL WUS configuration. In a step 2, gNB1 sends to gNB2 an UL WUS configuration provision request message comprising the UL WUS configuration and indication of cell(s) of gNB2 in which gNB1 requests the UL WUS configuration to be provided. In response, gNB1 receives from gNB2 an UL WUS configuration provision response message with an accept/reject indication for the request. In case of accept, gNB2 starts providing the UL WUS configuration in requested cell(s) in a step 3, however subject to the SI modification period, as discussed before. In a step 4, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is now providing the UL WUS configuration in requested cell(s). In a step 5, gNB1 starts operating the NES Cell in OD-SIB1 transmission mode.

Figure 2: An example of how new procedures can be used for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission.

We believe, it is sufficient and most appropriate, that gNB2 responds with “accept” if it can provide the UL WUS configuration in all cells requested (by gNB1) to be Cell A, and “reject” otherwise.

If the UL WUS configuration provisioning at gNB2 is temporarily interrupted at some point, in a step 6, gNB2 sends to gNB1 an UL WUS configuration provision status message indicating that it is not providing the UL WUS configuration in the requested cell(s). After receiving such a message as in step 6 (Cell A not providing the UL WUS configuration anymore), in a step 7, gNB1 may switch the NES Cell back to periodic SIB1 transmission mode, e.g., if the loss of Cell A(s) at gNB2 means lack of Cell-A coverage for the NES Cell (meaning no Cell-A redundancy).

Figure 3 shows how the proposed procedures can be used to deactivate OD-SIB1 transmission and revert the NES Cell to periodic SIB1 transmission (until further notice). If gNB1 decides to stop using OD-SIB1 transmission in the NES Cell for a longer period, it can inform gNB2 that the provisioning of the UL WUS configuration is no longer needed. Thus, it can send to gNB2 an UL WUS configuration provision request message with a corresponding indication, to relieve gNB2 from the additional provisioning of the UL WUS configuration to UEs at times when not needed. Unnecessary provisioning of an UL WUS configuration means both an unnecessary occupying of radio resources as well as a continuously increased energy by Cell A.

Figure 3: An example of how new procedures can be used to deactivate OD-SIB1 transmission and to revert back to periodic SIB1 transmission (until further notice).

Proposal 4: RAN3 to agree to capture in a Stage 2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2, and to capture the signaling flow for deactivating OD-SIB1 transmission and reverting back to periodic SIB1 transmission as depicted in Figure 3.

A TP reflecting the Proposal 1, 2 and 3 over XnAP is provided in Chapter 4.

Proposal 5: RAN3 to agree on the TP for BL CR for TS 38.423 provided in Chapter 4.

# 3	Proposals

In this contribution, we discussed issues and high-level solutions for Objective 2 of the work item mentioned above and made the following observations and proposals:

Observation 1: The new Class 1 procedure is initiated by the NES-Cell gNB to start the UL WUS configuration provision.

Proposal 1: RAN3 to agree the design of the new Class 1 procedure as a start/stop request, sent by a NES-Cell gNB to request a Cell-A gNB to start/stop providing the UL WUS configuration. The “start” request can be accepted or rejected by the Cell-A gNB. The “stop” request is always successful.

Proposal 2: RAN3 to agree that in the new Class 1 procedure, the NES-Cell gNB indicates to the Cell-A gNB which cell(s) of the Cell-A gNB should serve as Cell A (assisting in providing the UL WUS configuration).

Observation 2: The modification period for system information can be up to 41 seconds.

Observation 3: The response to a request for an UL WUS configuration provisioning should be signaled promptly by Cell-A gNB to the NES Cell gNB.

Observation 4: The NES cell should not go to the OD-SIB1 mode without knowing that its UL WUS configuration has been successfully broadcasted at least by one Cell-A gNB.

Proposal 3: RAN3 to agree the new Class 2 procedure is used to convey feedback regarding the UL WUS configuration provisioning status as “started”/“provided” from Cell-A gNB to NES-Cell gNB. After the NES-Cell receive this “started”/”provided” indication, it can go to the OD-SIB1 mode.

Proposal 4: RAN3 to agree to capture in a Stage 2 specification the signaling flow for activating OD-SIB1 transmission and temporary fallback to periodic SIB1 transmission as depicted in Figure 2, and to capture the signaling flow for deactivating OD-SIB1 transmission and reverting back to periodic SIB1 transmission as depicted in Figure 3.

Proposal 5: RAN3 to agree on the TP for BL CR for TS 38.423 provided in Chapter 4.

# 4	Text Proposal for BL CR for TS 38.423

*************** Start of TP ***************

### 8.x.x	UL WUS Configuration Provision

#### 8.x.x.1	General

The purpose of the UL WUS Configuration Provision procedure is to enable an NG-RAN node to request a neighbouring NG-RAN node to provide an UL WUS configuration in one or more cells.

The procedure uses non UE-associated signalling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: UL WUS Configuration Provision, successful operation

The NG-RAN node1 initiates the procedure by sending the UL WUS CONFIGURATION PROVISION REQUEST message to the peer NG-RAN node2 to start a provisioning, stop a provisioning, of an UL WUS configuration in one or more cells of the NG-RAN node2. Upon receipt, the NG-RAN node2:

-	shall initiate the requested provisioning according to the parameters given in the request in case the Request Type IE set to "start"; or

-	shall terminate the indicated (previously admitted) provisioning in case the Request Type IE is set to "stop".

The NG-RAN node2 shall, if supported, provide the UL WUS configuration included in the UL WUS CONFIGURATION PROVISION REQUEST message in the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message and shall respond with the UL WUS CONFIGURATION PROVISION RESPONSE message.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: UL WUS Configuration Provision, unsuccessful operation

If the NG-RAN node2 cannot provide the UL WUS configuration included in the UL WUS CONFIGURATION PROVISION REQUEST message in the cells indicated in the UL WUS CONFIGURATION PROVISION REQUEST message, it shall respond with the UL WUS CONFIGURATION PROVISION FAILURE message with an appropriate cause value.

#### x.x.x.4	Abnormal Conditions

Void.

### 8.x.y	UL WUS Configuration Provision Change

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node to inform a neighbouring NG-RAN node about status changes concerning an admitted UL WUS configuration provision.

It is also used to indicate that the UL WUS configuration provisioning is ongoing or interrupted.

The procedure uses non UE-associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: UL WUS Configuration Provision Change, successful operation

The NG-RAN node2 shall indicate the status of the admitted UL WUS configuration provision in the UL WUS CONFIGURATION PROVISION STATUS message.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

#### 9.x.x.x	UL WUS CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request provisioning of an UL WUS configuration in one or more cells.

Direction: NG-RAN node1 ® NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Request Type | M |  | ENUMERATED(start, stop, …) | Type of request concerning the provisioning of the UL WUS Configuration. | YES | reject |
| UL WUS Configuration | M |  | OCTET STRING | Includes the xyz message as defined in subclause a.b.c of TS 38.331. | YES | reject |
| Assisting Cells List |  | 1 |  | List of cells of the NG-RAN node2 requested to provide the UL WUS configuration. | YES | reject |
| >Assisting Cells Item |  | 1 .. < maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>NR CGI | M |  | 9.2.2.7 | Indicates the NR_CGI at the NG-RAN node2 | – |  |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellsinNG-RANnode | Maximum no. cells that can be served by an NG-RAN node. Value is 16384. |

#### 9.x.x.y	UL WUS CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration will be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.x.x.z	UL WUS CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the UL WUS configuration cannot be provided in the requested cells.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | ignore |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | ignore |

#### 9.x.x.t	UL WUS CONFIGURATION PROVISION STATUS

This message is sent by NG-RAN node2 to NG-RAN node1 to report a status change concerning an admitted UL WUS configuration provision.

Direction: NG-RAN node2 ® NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| Provision ID | M |  | INTEGER (0..255) | Allocated by the NG-RAN node1 | YES | reject |
| Provision Status |  |  | ENUMERATED (providing, not providing, …) | Indicates the provision status of the UL WUS configuration in the assisting cells. |  |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

*************** End of TP ***************

# 5	References

[1]	RP-234065	New WID: Enhancements of network energy savings for NR

[2]	RP-242354	Revised WID: Enhancements of network energy savings for NR

[3] 	R3-244708	Summary of offline disc at RAN3#125, Ericsson (Moderator)

[4]	R3-245737 	Summary of offline disc at RAN3#125-bis, Ericsson (Moderator)

[5]	R3-245498	Xn impact of On-demand SIB1 for UEs in idle/inactive mode, Ericsson