3GPP TSG-RAN WG3 Meeting #126	R3-247564

Orlando, USA, 18 – 22 November, 2024

Agenda Item:	11.3

Source:	Nokia, Jio

Title:	Discussion on signalling for AI/ML-based CCO

Document for:	Discussion

# 1	Introduction

We here provide discussion on possible solutions for F1 and Xn signalling following agreements, working assumption and open point capture at last RAN3 meeting.

The following aspects are discussed:

- CCO scenario with multiple DUs involved

- Legacy and AI/ML based CCO co-existence

- Information to validate the prediction

# 2	Discussion

## 2.1	CCO scenario with multiple DUs involved

Discussions at RAN3#125bis led to the following agreements on timing information.

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

From these agreements it can be seen that time for future coverage state and time for predicted CCO issue are different.

Also, the following working assumption on F1AP signalling procedures was taken with the intention to simplify coexistence of legacy CCO and Rel-19 AI/ML-based CCO:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

How to enable differentiation between time for future coverage state and time for predicted CCO issue in F1 signalling is a new aspect which requires RAN3 discussion.

In the legacy signalling framework, the gNB-DUs receiving CCO issue information from the gNB-CU will determine whether it will have to solve the issue, or whether it will rather need to apply a matching CCO state is determined based on the content of the Affected Cells and Beams IE. Copying from TS 38.473 in the bullets below:

- Case where the receiving gNB-DU resolves the CCO issue: If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message, and the NR CGI IE contained in the Affected Cells and Beams IE is served by the gNB-DU, the gNB-DU may use it to determine a new cell and/or beam configuration.

- Case where the receiving gNB-DU adopts a matching CCO state: If the CCO Assistance Information IE is contained in the GNB-CU CONFIGURATION UPDATE message and the NR CGI IE contained in the Affected Cells and Beams IE is not served by the gNB-DU, the gNB-DU may use it to adjust coverage of its cells.

It can be observed from the above that there is no transfer of information concerning CCO state change between gNB-DUs served by the same gNB-CU, so a gNB-DU2 will need to be able to derive, via some configuration, the CCO state adopted by a gNB-DU1 serving a cell that is affected by a CCO issue. This approach comes with the advantage that gNB-CU configuration update messages related to a given CCO issue may be sent out in parallel from the gNB-CU. On the other side, this approach requires a tight OAM coordination between the different DUs already in Rel-17. And when it comes to Rel-19 aspects, we need to take into account the additional new information of time for future coverage state.

In order to provide consistent signalling solutions for F1 and Xn, we therefore propose to look into the following overall scenario:

Figure 1:illustration of scenario where CU1 is serving three DUs, namely DU1, DU2 and DU3. Similarly, CU2 is serving DU’.

Figure 1 shows gNB-CU1 (CU1) is serving three gNB-DUs, namely gNB-DU1 (DU1), DU2 and DU3, while CU2 serves DU’. Furthermore, the Figure 2 shows illustration for the case wherein a CCO issue is predicted by CU1 that involves all cells of its serving DUs (assume each DU serves a cell). Along with predicted CCO issue, CU1 predicts the time for predicted CCO issue denoted as t0. As shown in Figure 2 the predicted CCO issue and t0 is sent to each of serving DU in parallel. Each DU determines a new cell and/or beam configuration to solve the predicted CCO issue, and it also determines the time for future coverage state. In shown example the DU1, DU2 and DU3 independently determines their own future coverage state denoted as c1, c2 and c3 respectively; and time for future coverage state denoted as t1, t2, t3 respectively. Each of these DU provides the future coverage state and time for future coverage state information to CU1 in GNB-DU CONFIGURATION UPDATE. At this point we see that CU1 has future CCO state for each of its DU denoted as C = (c1,c2,c3) and three different time for future coverage state (t1, t2, t3). The following issues arise

- Dilemma on which time for future coverage state should be shared with neighbour nodes.

- Having different time for future coverage state means that future coverage states are applied by these DUs at different times which gives rise to incompatible coverage configurations at different DUs and further at the neighbour nodes. Consequently, this will affect UE and overall network performance adversely.

It is evident that the aforementioned issues have arisen because CU1 simultaneously sent the predicted CCO issue and t0 to the concerned DUs.

Observation 1: In multi-DU scenario, issue of multiple time for future coverage state arises when predicted CCO issue and time for predicted CCO issue is shared with more than one concerned DU in parallel. Furthermore, this different times for future coverage state leads to incompatible coverage configurations adopted at different DUs at different times which negatively impacts UE and overall network performance.

Figure 2: illustration of working of CCO framework in multi-DU scenario wherein CCO issue is sent to multiple DUs leading to multipletime for future coverage state.

In order to avoid the issues mentioned before, the AI/ML based CCO in multi-DU scenario should work as illustrated in Figure 3, wherein the predicted CCO issue and time for predicted CCO issue is sent to only one concerned DU.

As shown in Figure 3, the CCO issue is predicted by CU1 that involves all cells of its serving DUs (assume each DU serves a cell). Along with predicted CCO issue, CU1 predicts the time for predicted CCO issue denoted as t0. This time CU1 sends the predicted CCO issue and time for predicted CCO issue t0 to DU1 only (chosen randomly or by implementation). DU1 independently determines its own future coverage state denoted as c1 along with time for future coverage state denoted as t1. DU1 provides the c1 and t1 information to CU1 in GNB-DU CONFIGURATION UPDATE. At this point, CU1 shall send the CCO Assistance Information and the t1 to the remaining DUs involved (in parallel) in the predicted CCO issue i.e. DU2 and DU3. Herein the legacy CCO Assistance Information containing coverage modification cause and affected cells and beams is proposed to be resused. It is worth noting that if only modification cause is provided to DU2 and DU3 then these nodes cannot determine a new cell and/or beam configuration or adjust coverage of its cells to matching coverage configuration. Therefore, Affected Cells and Beams should be provided.

As shown in Figure 3, the three stages denoted as 1, 2, and 3 should be sequential whereas procedures within each stage can run in parallel e.g. CU1 sending CCO Assistance Information and t1 to DU2 and DU3 in parallel.

By sharing predicted CCO issue and time for predicted CCO issue to only one concerned DU there will not be more than one time for future coverage state. This also means the issue of incompatible coverage configurations at different DUs and further at the neighbour nodes arised due to multiple time for future coverage state is circumvented. Furthermore, the stage 2 procedure parallism can be seen: as the CU1 can share the CCO assistance information and time for future coverage state with DU2 and DU3 in parallel, after receiving future CCO state and time for future coverage state from DU1. Finally, CU1 after receiving future coverage state from all the concerned DUs can share it along with time for future coverage state t1 with CU2. In case there were multiple CUs involved (not shown in Figure), CU1 can share this information again in parallel fashion.

Observation 2: In multi-DU scenario, by sharing predicted CCO issue and time for predicted CCO issue to only one concerned DU

- the issue of multiple time for future coverage state is resolved. This also means the issue of incompatible coverage configurations at different DUs and further at the neighbour nodes arised due to multiple time for future coverage state is circumvented.

- CU1 after receiving future CCO state and time for future coverage state from one of the concerned DU, can share the CCO assistance information and time for future coverage state with remaining concerned DUs in parallel.

- CU1 can inform the neighbour nodes about future coverage state of all concerned served DUs and time for future CCO state in parallel.

Figure 3: (a) illustration of working of CCO framework in multi-DU scenario wherein CCO issue is sent to only single DU.

Proposal 1:  When the gNB-CU predicts a CCO issue, it first informs the gNB-DU serving the cell affected by the issue, which determines the future CCO state and the time for future coverage state. The gNB-CU then informs other DUs that need to adjust their coverage.

As seen from the Figure 3, the predicted CCO issue and the time for predicted CCO issue t0 is sent by CU1 to DU1. Now there could be two options to send this information, 1. Enhancing legacy CCO Assistance Information IE or 2. Using a new IE. In selection one of the options, one should consider that while sending the predicted CCO issue to DU1, CU1 includes the start time of the predicted CCO issue t0. Whereas, to inform DU2 and DU3, CU1 sends CCO assistance information and time for future state t1 (initially received from DU1). A possible way forward could be to use a new IE to send CCO issue prediction information from CU1 to DU1, which includes the time of the predicted CCO issue. In stage 2, in the direction from CU1 to DU2/DU3, the CCO Assistance Information IE could be reused, enhanced with time for future coverage state.

The gNB-DU1 replies to CU1 using the Coverage Modification Notification IE. This IE includes the Cell Coverage State IE with mandatory presence. It is possible to enhance this IE to include the time for future coverage state t1, and if this time information IE is present the legacy Cell Coverage State could be interpreted as Future Cell Coverage State. However the range of the legacy IE includes value ‘0’, indicating inactive cell, which is not intended as a valid new CCO state after the agreed CCO issues ‘coverage’ and ‘cell edge capacity’. Also, the legacy IE includes the Coverage Modification Cause IE (optionally present), which in addition to the two CCO causes also include the network energy saving cause. A Future Coverage Modification Cause IE is therefore needed. RAN3 should therefore consider to create a new Future Coverage Modification Notification IE.

This leads to the following proposals:

Proposal 2: RAN3 to discuss creation of a new IE to convey the predicted CCO issue and time for predicted CCO issue from the gNB-CU to gNB-DU that is serving cells contained in the Affected Cells and Beams IE.

Proposal 3: RAN3 to discuss reuse of the CCO Assistance Information IE, enhanced with predicted CCO issue and time for future coverage state, in the direction from the gNB-CU to gNB-DUs not serving cells contained in the Affected Cells and Beams IE.

Proposal 4: RAN3 to discuss creation of a new Future Coverage Modification Notification IE to convey Future Coverage Modification Cause and time for future coverage state in the direction from gNB-DU to gNB-CU.

Nature of time for predicted CCO issue and time for future coverage state

We consider it essential to comprehend and determine the manner in which the timing information elements, specifically the time for the predicted CCO issue t0 and the time for the future coverage state t1, should be represented, whether in relative or absolute terms.

As seen from Figure 3, the time for predicted CCO issue t0 is sent to single DU serving the cell affected by the issue. Therefore, this timing information can be expressed in either absolute or relative terms.

Proposal 5: RAN3 to discuss whether the time for predicted CCO issue is encoded as absolute or relative time.

As depicted in Figure 3, the time for future coverage state t1 is determined by DU1 and sent to CU1. CU1 processes the message received and then CU1 shares t1 with DU2 and DU3. DU2 and DU3 may require some processing time to determine the matching coverage configuration. Once this is completed, each DU provides their determined future coverage configuration to CU1. Again, CU1 processes the message(s) received from DUs. Now, in case the t1 is a relative value, then at the end of each message the entities CU or DU may need to recalculate/update the timing information to account for aforementioned processing time(s) at these entities. This recalculation/updation of timing information essentially transforms a relative timing information to an absolute timing information (which defeats the purpose of using relative time).

Observation 3: If time for future coverage state is expressed in relative terms, then the time taken by CU or DU to process the message or complete a task (e.g. determine the matching coverage configuration which is to be included in reply message) effectively converts the timing information into an absolute form.

Proposal 6: Time for future coverage state is encoded as absolute time.

## 2.3	Legacy and AI/ML based CCO co-existence

One of discussion in last RAN3#125bis meeting was on the co-existence of legacy and AI/ML based CCO. Here in this subsection, we analyse scenorios that exhibit such co-existence.

A) Sending legacy (immediate) CCO issue and predicted CCO issue for the same cell in the same message

Sending both a legacy (immediate) CCO issue and a predicted CCO issue for the same cell in the same message means gNB-CU has determined a legacy CCO issue that occurs immediately and also gNB-CU has predicted a CCO issue that will happen some time in future. However, it is important to note that the gNB-CU will be unaware of the new CCO state adopted by the gNB-DU as a result of the immediate CCO issue resolution, and therefore it is impossible for gNB-CU to predict a future CCO issue accurately i.e. the predicted future CCO issue is likely to be a wrong prediction. Therefore, legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be send in the same message.

Proposal 7: Legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be send in the same message.

B) Sending legacy (immediate) CCO issue and predicted CCO issue for the same cell in the two different messages

Depending on the order in which the CCO issue (legacy or AI/ML based) is sent there can be following two subcases.

1. gNB-CU sends first a legacy (immediate CCO issue) and then a predicted CCO issue

It means that gNB-CU first determines a legacy CCO issue which it sends and then gNB-CU predicts a CCO issue that will happen some time in future. Now, it should be understood that the gNB-CU while predicting the CCO issue will take into account new CCO state adopted by the gNB-DU as a consequence of the immediate CCO issue that was determined earlier. Therefore, it can be said that the prediction is based on latest information/situation and thus valid.

Proposal 8: It is possible to send a legacy (immediate) CCO issue first in one message and then the predicted CCO issue for the same cell later in a different message.

2. gNB-CU sends first predicted CCO issue and then legacy (immediate CCO issue)

Opposite to case 1, now gNB-CU first predicts and sends a CCO issue that will happen some time in future (say t). Then gNB-CU determines and sends a legacy CCO issue before expiry of t. Now, in order to solve the legacy (immediate) CCO issue, new cell and/or beam configuration (additionally, matching of coverage configuration in own/neighbour cells) will be applied at some of the gNB-DU served by gNB-CU. It means that the situation at which the CCO prediction was made will no longer be valid, and thus, the prediction itself will be invalid. In other words, legacy CCO issue resolution creates a new situation, rendering the initial CCO prediction obsolete.

Proposal 9: In case a legacy gNB-CU sends first predicted CCO issue with time for CCO prediction t, and then legacy (immediate) CCO issue for the same cell before expiry of prediction time t; the first predicted CCO issue should be discarded (deemed as invalid).

## 2.4	Information to validate the prediction

The following open point was identified at last meeting to be continued:

Information to validate the prediction is needed.

Before discussing the exact feedback information to validate the prediction, it is necessary to understand if the information required for validation should be pursued for predicted CCO issue and/or for the future coverage state adopted as a consequence of the predicted CCO issue.

The goal of using AI/ML based CCO is to proactively prevent the CCO issue from occurring, and therefore if the correct actions are taken to solve this predicted issue, then the CCO issue will not materialize and therefore can’t be confirmed. It can additionally be observed that any CCO state change happening in the time interval between the prediction is made and until the time for predicted CCO issue will in principle change the assumptions behind the prediction which also reduces the possibilities to verify the prediction.

Observation 4: It is not possible to validate the prediction of the CCO issue because it may not materialize due to corrective action and/or because the underlying assumptions of the prediction will change if a CCO state change is applied.

Let us therefore discuss the process of validating the future coverage state. For the case of CCO issue and CCO state changes concerning cells controlled by a single node, information locally available in the gNB-CU may be used for such validation. However, standardisation is needed for the case of CCO issue and CCO state changes concerning cells controlled by more than one node. In the legacy (reactive) case, a CCO configuration profile selected by an NG-RAN node 1 is signaled to a neighboring NG-RAN node 2, so that this NG-RAN node 2 matches/ adapts its own CCO configuration accordingly. However, in the case where a CCO issue is predicted in-advance by an NG-RAN node 1, there is currently a gap on how this NG-RAN node 1 can effectively coordinate with a neighboring NG-RAN node 2 in order for them to jointly apply their CCO configuration states in a way such that: i) the CCO issue predicted by NG-RAN node 1 is resolved in time (i.e., ahead of its predicted occurrence) and ii) the resolution of this CCO issue does not create another CCO issue to either of the neighboring NG-RAN nodes.

The following feedback information was captured in TR 38.743 to be considered to be collected from gNBs:

-	Measured radio resource status

-	Legacy UE performance feedback for those UEs handed over from the source gNB

-	SON Reports (e.g., RLF, CEF, RA)

The resolution of the CCO issue can be verified by UE performance feedback as indicated by the TR. The NG-RAN node 1 will also monitor CCO issue detection or prediction provided by NG-RAN node 2 and other neighbouring nodes, and in this way detect any potential issue created there.

Proposal 10: UE performance feedback and CCO issue detection in neighbour nodes to be used as information to validate the future coverage state.

# 3	Conclusion

We have made the following proposals:

Observation 1: In multi-DU scenario, issue of multiple time for future coverage state arises when predicted CCO issue and time for predicted CCO issue is shared with more than one concerned DU in parallel. Furthermore, this different times for future coverage state leads to incompatible coverage configurations adopted at different DUs at different times which negatively impacts UE and overall network performance.

Observation 2: In multi-DU scenario, by sharing predicted CCO issue and time for predicted CCO issue to only one concerned DU

- the issue of multiple time for future coverage state is resolved. This also means the issue of incompatible coverage configurations at different DUs and further at the neighbour nodes arised due to multiple time for future coverage state is circumvented.

- CU1 after receiving future CCO state and time for future coverage state from one of the concerned DU, can share the CCO assistance information and time for future coverage state with remaining concerned DUs in parallel.

- CU1 can inform the neighbour nodes about future coverage state of all concerned served DUs and time for future CCO state in parallel.

Proposal 1:  When the gNB-CU predicts a CCO issue, it first informs the gNB-DU serving the cell affected by the issue, which determines the future CCO state and the time for future coverage state. The gNB-CU then informs other DUs that need to adjust their coverage.

Proposal 2: RAN3 to discuss creation of a new IE to convey the predicted CCO issue and time for predicted CCO issue from the gNB-CU to gNB-DU that is serving cells contained in the Affected Cells and Beams IE.

Proposal 3: RAN3 to discuss reuse of the CCO Assistance Information IE, enhanced with predicted CCO issue and time for future coverage state, in the direction from the gNB-CU to gNB-DUs not serving cells contained in the Affected Cells and Beams IE.

Proposal 4: RAN3 to discuss creation of a new Future Coverage Modification Notification IE to convey Future Coverage Modification Cause and time for future coverage state in the direction from gNB-DU to gNB-CU.

Proposal 5: RAN3 to discuss whether the time for predicted CCO issue is encoded as absolute or relative time.

Observation 3: If time for future coverage state is expressed in relative terms, then the time taken by CU or DU to process the message or complete a task (e.g. determine the matching coverage configuration which is to be included in reply message) effectively converts the timing information into an absolute form.

Proposal 6: Time for future coverage state is encoded as absolute time.

Proposal 7: Legacy (immediate) CCO issue and predicted CCO issue for the same cell should not be send in the same message.

Proposal 8: It is possible to send a legacy (immediate) CCO issue first in one message and then the predicted CCO issue for the same cell later in a different message.

Proposal 9: In case a legacy gNB-CU sends first predicted CCO issue with time for CCO prediction t, and then legacy (immediate) CCO issue for the same cell before expiry of prediction time t; the first predicted CCO issue should be discarded (deemed as invalid).

Observation 4: It is not possible to validate the prediction of the CCO issue because it may not materialize due to corrective action and/or because the underlying assumptions of the prediction will change if a CCO state change is applied.

Proposal 10: UE performance feedback and CCO issue detection in neighbour nodes to be used as information to validate the future coverage state.