3GPP TSG RAN WG3 Meeting #126	R3-247577

Orlando, U.S.A., 18 - 22 November, 2024

Agenda Item:	20.1

Source:	Ericsson

Title:	Analysis and discussion on the Reply LS on AIML data collection

Document for:	Discussion and Decision

# Introduction

In the LS in [1], RAN plenary listed the following solutions detailed by RAN2 and concerning training data collection for UE sided models:

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

The LS in [1] asks SA and SA2 the following:

“TSG RAN kindly asks TSG SA to initiate the work and at least for SA WG2 to provide inputs on solutions 1b, 2, and 3 by RAN#106.”

In turn SA2 has sent an LS in [2], where SA2 poses the following question and actions to RAN3:

SA2 would like to ask about the following terms: - Controllability of MNO on data transfer: In RP-242389 / S2-2409600, it is stated that full controllability is required when “the MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer” SA2 kindly request RAN2 to provide further clarification on the following areas: Q1: Are there any aspects of the UE-data collection controllability, that required NG-RAN involvement? If so, what is the involvement of NG-RAN in UE-data collection controllability, e.g., what aspects of MNO controllability would require NG-RAN involvement and what would such involvement be? As an example of the kind of feedback that is requested, some companies in SA2 understand that initiating (e.g., triggering), terminating collection of UE-side data and controlling data transfer may require NG-RAN involvement, and it is currently not clear what this involvement may be. […] 2. Actions: To RAN3 ACTION: SA2 kindly ask RAN3 to consider question #1 above and provide as early as possible any feedback.

In this paper an answer to the question in the LS in [2] is provided by means of analysing the solutions described in the LS in [1].

# Discussion

We will answer the question in [2] on a per solution basis.

## 2.1	Answer for Option 1A

Option 1A is based on an over the top communication between the UE and an OTT server. Such communication may use a 3GPP or a non-3GPP network. When it uses a 3GPP network, the solution is based on transferring of training data from the UE to the OTT server by means of UP tunnels.

OTT communication between the UE and an OTT server does not imply any involvement by the NG-RAN because the NG-RAN is not aware of whether a UP channel is transporting data for UE sided model training.

The answer to the question in [2] is therefore rather straightforward and can be formulated as follows:

Answer to Q1 in case of Option 1A:

The NG-RAN has no involvement in and it is not impacted by data collection via Option 1A.

## Answer for Option 1B

Option 1B is based on the establishment of a PDU Session for data transfer from the UE to a Server for data collection for UE-side model training.

In this option a PDU Session is dedicated to the purpose of reporting data collection for UE-side model training from the UE to the server. It is plausible to assume that such PDU Session will be marked with identifiers, e.g. 5QI, S-NSSAI, that would let the NG-RAN understand that the PDU Session is established for training data collection for UE-side models.

The NG-RAN is involved in setting up such communication channel in various ways that are covered by legacy functionality and that we list below:

- The RAN is responsible for admitting the PDU Session. This involves checking the QoS Level QoS Parameters of the QoS Flows included in the PDU Session and the associated ARP values and decide whether resources can be allocated to the PDU Session or not. If the admission control is successful, resources can be allocated to the PDU Session and the UP channels associated to the PDU Session can be established. If the admission control is not successful, resources are not allocated to the PDU Session and the UP channels associated to the PDU Session are not established

- The RAN is responsible for maintaining the UP channels associated to the PDU Session, so long as resources are available to serve them and so long as there is no need to pre-empt such channels (e.g. in case of high load and the need to serve higher priority traffic)

- The RAN is responsible to apply appropriate RRM policies to the established UP channels, derived from the policy the operator wants to apply to such UP channels, for example, scheduling policies.

- The RAN is responsible for checking the available subscriber´s information and decide whether the UP channels can be established in specific coverage areas. An example could be checking the Mobility Restriction List information to determine if the UP channels can be supported during mobility to a different coverage area.

In light of the above, the following answer can be derived:

Answer to Q1 in case of Option 1B:

The NG-RAN is involved in the configuration and management of the UP channels established for option 1B, in accordance with the policies assigned by the operator for such UP channels. Option 1B has no standardisation impact on the NG-RAN.

## Answer for Option 2

Option 2 consists of establishing a control plane tunnel where data for UE-side model training are transferred over NAS.

The NG-RAN is involved in the configuration of such CP tunnels and in the signalling of interface messages carrying NAS payload. The tasks performed by the NG-RAN are listed as follows:

- The NG-RAN is involved in the selection of an appropriate AMF for the NAS communication to be established.

- The NG-RAN is responsible to transfer NAS messages over the NG interface between AMF and UE.

- The NG-RAN is responsible to notify the AMF of any undelivered DL NAS PDU

- Given the potentially high data rate and payload that training data for UE -side models may generate, the NG-RAN may be unable to deliver all the UL NAS PDUs generated by the UE. Hence, mechanisms to enable the NG-RAN to notify the AMF of undelivered UL NAS PDUs may be needed. In such case, the NG-RAN would be responsible for such notification.

In light of the above, the following answer can be derived:

Answer to Q1 in case of Option 2:

The NG-RAN is involved in the configuration of the CP channels established for option 2, in the signalling of NAS PDU messages in UL and DL and in the notification of undelivered NAS PDUS. Option 2 may have standardisation impact on the NG-RAN, if mechanisms for UL NAS delivery failure would be needed, in cases of NAS traffic overload.

## Answer for Option 3

Option 3 is based on establishing a control plane tunnel from the NG-RAN to the OAM, where data for UE-side model training are transferred over the RAN to OAM interface.

It should be pointed out that the closest mechanism to Option 3 supported today by the standard is Signalling based MDT, where a measurement context is established per UE and where MDT measurements are reported on a per UE basis to the OAM. However, in Signalling based MDT, collected measurements are fully specified, hence one question would be if the same would apply to training data collected for UE side models, namely would such training data be fully specified? The question is relevant because there are no solution today on how to configure and collect measurements that are not specified.

Another observation is that signalling based MDT has been designed for tracing few UEs in the network, as it is expensive to maintain a signalling based context per UE. It is therefore plausible to ask whether Signalling based MDT would scale in case the number of UEs from which training data is collected is high.

The involvement of the NG-RAN in this option may therefore be captured as follows:

- The NG-RAN checks whether user consent is available for the UE and for the requested measurements.

  - It is unclear whether existing MDT user consent framework can be reused or if standard changes are required to the MDT user consent framework.

- The NG-RAN receives a measurement collection configuration from the OAM and it configures the UE with such measurements (assuming use consent is in place).

  - If new measurements are required with respect to current MDT measurements, the standard will have to be impacted.

  - If some of the required measurements are not specified, it is unclear how the RAN will configure such measurements.

- The NG RAN collects UE measurements and logs it into Trace Records.

  - If new measurements are required with respect to current MDT measurements, the standard will have to be impacted.

  - If some of the required measurements are not specified, it is unclear how the RAN will log such measurements.

In light of the above, the following answer can be derived:

Answer to Q1 in case of Option 3:

Assuming that Option 3 consists of reusing MDT, the NG-RAN is involved in checking of user consent for MDT measurements collection for the UE, receiving an MDT measurement configuration from the OAM, configuring measurements at the UE, collecting UE measurements and reporting them to the OAM.
It should be noted that Option 3 will incur in standardisation impacts if the data to be reported are not covered by existing MDT measurements. If the data to be collected is not specified it is unclear how the corresponding measurements can be configured, collected and reported.

# Conclusion

In this contribution the LS in [2] was analysed in light of the information in [1].

Answers in reply to the question asked in [2] have been derived.

It is proposed to agree to the LS in the Annex, which reflects the answers derived in the paper.

# References

- RP-242389, LS on AIML data collection, RAN

- R3-247017/ S2-2411191, Reply LS on AIML data collection, SA2.

3GPP TSG RAN WG3 Meeting #126	R3-24xxxx

Orlando, U.S.A., 18 - 22 November, 2024

Title:	Reply to Reply LS on AIML data collection

Response to:	LS R3-247017 (S2-2411191) Reply LS on AIML data collection

Release:	Rel-19

# Work Item:	     NR_AIML_Air

Source:	RAN3

To:	SA2, RAN2

Cc:	TSG SA, TSG RAN, SA3, SA5

Contact Person:	Angelo Centonza

E-mail Address:	angelo.centonza@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN3 would like to thank SA2 for the LS in R3-247017 (S2-2411191) on AIML data collection.

RAN3 has discussed it and derived the following answers for the question asked by SA2:

Answer to Q1 in case of Option 1A:

The NG-RAN has no involvement in and it is not impacted by data collection via Option 1A.

Answer to Q1 in case of Option 1B:

The NG-RAN is involved in the configuration and management of the UP channels established for option 1B, in accordance with the policies assigned by the operator for such UP channels. Option 1B has no standardisation impact on the NG-RAN.

Answer to Q1 in case of Option 2:

The NG-RAN is involved in the configuration of the CP channels established for option 2, in the signalling of NAS PDU messages in UL and DL and in the notification of undelivered NAS PDUS. Option 2 may have standardisation impact on the NG-RAN, if mechanisms for UL NAS delivery failure would be needed, in cases of NAS traffic overload.

Answer to Q1 in case of Option 3:

Assuming that Option 3 consists of reusing MDT, the NG-RAN is involved in checking of user consent for MDT measurements collection for the UE, receiving an MDT measurement configuration from the OAM, configuring measurements at the UE, collecting UE measurements and reporting them to the OAM.

It should be noted that Option 3 will incur in standardisation impacts if the data to be reported are not covered by existing MDT measurements. If the data to be collected is not specified it is unclear how the corresponding measurements can be configured, collected and reported.

2. Actions:

To SA2

ACTION:  RAN3 kindly ask SA2 to take the above answers into account.

3. Date of Next SA2 Meeting:

TSG-RAN WG3 Meeting RAN3#127			17 - 21 February 2025		Athens, GR

TSG-RAN WG3 Meeting RAN3#127bis			7 - 11 April 2025		China, CN