3GPP TSG-RAN WG3 Meeting #126	R3-247579

Orlando, US, 18 - 22 Nov, 2024

Agenda Item:	18.2

Source:	Ericsson

Title:	Discussion of LPWUS impacts on NG-RAN architecture

Document for:	Discussions

# 1	Introduction

In Last RAN3 meeting, the following LP-WUS agreements were made:

- The LP-WUS assistance information IE, which includes the CN assigned subgroup ID for LP-WUS, is introduced in the NG paging message. FFS the number of the subgroups. - The LP-WUS assistance information IE is introduced in the NGAP core Networking assistance information IE to assist anchor gNB to initiate RAN paging. FFS the number of the subgroups. - The LP-WUS assistance information IE, which indicates the CN assigned subgroup ID for LP-WUS, is introduced in F1 paging message for CN paging and RAN paging. FFS the number of the subgroups. - Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping. - The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

RAN3 has also received this meeting an LS from RAN2 on the status of LP-WUS based subgrouping signalling framework in [1]. We discuss in this paper the aspects to be considered in RAN3 for LP-WUS progress.

# 2	Discussion on RAN3 impacts

RAN2 has indicated the following agreements in their LS:

1) The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups. 2) RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8. 3) RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE. 4) For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. 5) For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

Most of these agreements have already been considered by RAN3 in previous RAN3#125bis meeting. In fact, RAN3 has already captured in previous meeting the agreements related to re-utilization of PEI framework for LP-WUS, namely the introduction of LP-WUS Assistance Information IE with CN subgroup IDs (FFS on value range, to be defined by CT1) in the PAGING message and CNAIfRI IE in NGAP.

It is worth nothing that the value range of the new IE should be extensible: INTEGER(0..7,…).

Regarding XnAP, companies have mentioned that the introduction of LP-WUS Assistance Information IE to RAN PAGING message is conditional to decision in RAN2 whether the LP-WUS is restricted to last serving cell or not. We expect that like PEI, a UE will be configured to monitor LP-WUS, either only in the last used cell or any other cells (after cell reselection). The network indicates in system information the whether UE monitors LP-WUS in last used cell.

Observation 1: It is very likely that LP-WUS will follow the same configuration as PEI, where a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection). This FFS should be captured as pending RAN2 progress.

Therefore, we propose that when sending the XnAP RAN PAGING message to other NG-RAN nodes, the old NG-RAN node includes the LP-WUS Assistance Information IE in the message. An EN can be added in the BL CR to capture this is pending RAN2’s progress. A proposal for XnAP BL CR is described in [2].

Proposal 1: When sending the XnAP RAN PAGING message to other NG-RAN nodes, the old NG-RAN node includes the LP-WUS Assistance Information IE. An EN can be added in XnAP BL CR to capture this is pending RAN2’s progress.

Regarding the F1AP FFS:

- Introducing LP-WUS subgrouping capability related information into F1 paging message for UE_ID based subgrouping. FFS on per UE or per cell. FFS for CN based subgrouping.

If we make a parallel with PEI subgrouping capability, this new indication should be per cell capability. In fact, in Rel-17 PEI, RAN2 had specified that the UE supports receiving paging early indication in DCI format 2_7. And it can be observed that this PEI-subgroupingSupport capability is given for a list of frequency bands.

UE-ID based PEI capability

The UE can indicate to support UE-ID based PEI per band in the radio paging capabilities:

UE-RadioPagingInfo-r17 ::=            SEQUENCE {

-- R1 29-1: Paging enhancement

pei-SubgroupingSupportBandList-r17 SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

...

}

maxBands                 INTEGER ::= 1024    -- Maximum number of supported bands in UE capability.

The IE FreqBandIndicatorNR is used to convey an NR frequency band number as defined in TS 38.101-1 [15], TS 38.101-2 [39] and TS 38.101-5 [75].

FreqBandIndicatorNR ::=             INTEGER (1..1024)

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| pei-SubgroupingSupportBandList-r17 Indicates whether the UE supports receiving paging early indication in DCI format 2_7 as specified in TS 38.304 [21] for a list of frequency band. The UE shall support UEID based subgrouping for a frequency band if it indicates supporting of paging early indication reception for the frequency band. The set of OFDM symbols within a slot where UE can monitor the PEI PDCCH in Type 2A CSS is the same as the requirement for paging PDCCH in Type 2 CSS for IDLE and INACTIVE mode UEs. | UE | No | No | No |

If the UE supports PEI, then the UE shall support UEID-based PEI. Optionally the UE may also support CN-assigned PEI. The gNB may indicate to support UE-based and/or CN-assigned based PEI (as specified in TS 38.300).

The UE capability for CN-assigned PEI is indicated per UE via NAS signalling (PEIPS) in the REGISTRATION REQUEST message, see 24.501:

5.3.25	Paging Early Indication with Paging Subgrouping Assistance

A UE may indicate its capability to support NR paging subgrouping during registration procedure when the UE:

-	initiates a registration procedure with 5GS registration type IE not set to "emergency registration"; and

-	does not have an active emergency PDU session.

However the UE only supports PEI using CN-assigned subgrouping in the bands indicated in pei-SubgroupingSupportBandList-r17 capability. Similar approach should be assumed followed for LP-WUS.

Observation 2: In the UEID-based subgrouping capability the UE can indicate in which of the supported bands the UE supports LP-WUS by including a band list.

Observation 3: If the UE indicates to support CN-assigned subgrouping then the UE supports LP-WUS using CN-assigned subgrouping in the bands indicated with UEID-based subgrouping capability.

Below how the PEI Subgrouping capability was introduced in Rel-17:

TS 38.473 extract:

### 8.7.1 Paging […] The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24]. […] ### 9.2.6 Paging messages #### 9.2.6.1 PAGING This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs. Direction: gNB-CU gNB-DU IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; AssignedCriticality. Message Type; M; 9.3.1.1; YES; ignore. UE Identity Index value; M; 9.3.1.39; YES; reject. CHOICE Paging Identity; M; YES; reject. >RAN UE Paging identity. >>RAN UE Paging identity; M; 9.3.1.43; -. >CN UE paging identity. >>CN UE paging identity; M; 9.3.1.44; -. Paging DRX; O; 9.3.1.40; It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX; YES; ignore. Paging Priority; O; 9.3.1.41; YES; ignore. Paging Cell List; 1; YES; ignore. >Paging Cell Item IEs; 1 .. <maxnoofPagingCells>; EACH; ignore. >>NR CGI; M; 9.3.1.12; -. >>Last Used Cell Indication; O; ENUMERATED(true, …); YES; ignore. >>PEI Subgrouping Support Indication; O; ENUMERATED(true, …); YES; ignore.

Hence the new LP-WUS Subgrouping Capability should also be introduced in cell level (in a frequency band level). As explained above, the PEI Subgrouping Support Indication IE is associated to both subgrouping schemes ( CN-subgrouping or UE-ID based subgrouping). Hence, the new LP-WUS Subgrouping Support Indication should also follow similar scheme and encoding as the PEI subgrouping indication.

Observation 4: In Rel-17 PEI, a generic PEI Subgrouping Support Indication IE was added into F1 paging message per cell level for both UE_ID based subgrouping and CN based subgrouping.

Proposal 2: LP-WUS Subgrouping support indication in F1AP follows same scheme as PEI: RAN3 to introduce a LP-WUS Subgrouping Support indication IE in the Paging Cell Item IEs in the F1 PAGING message. If received, the gNB-DU shall, if supported, consider the UE is capable to receive LP-WUS in that cell.

Regarding the second FFS:

- The gNB-DU is responsible to generate and broadcast the LP-WUS related configuration, including LP-SS configuration, LP-WUS configuration. FFS on entry/exit condition for LP-WUS monitoring.

Those entry/exit conditions are executed at the UE side, i.e. not NW side. When in RRC_Idle state, based configuration in SIB and the UE capability, the UE can indicate to NW it can use LP-WUS, afterwards the NW always sends LP-WUS.

Proposal 3: endorse BL CR [2] for XnAP

# 3	Conclusion

Observation 1: It is very likely that LP-WUS will follow the same configuration as PEI, where a UE will be configured to monitor LP-WUS, either only in the last used cell, if indication is sent in the system information, or any other cell (after cell reselection). This FFS should be captured as pending RAN2 progress.

Proposal 1: When sending the XnAP RAN PAGING message to other NG-RAN nodes, the old NG-RAN node includes the LP-WUS Assistance Information IE. An EN can be added in XnAP BL CR to capture this is pending RAN2’s progress.

Observation 2: In the UEID-based subgrouping capability the UE can indicate in which of the supported bands the UE supports LP-WUS by including a band list.

Observation 3: If the UE indicates to support CN-assigned subgrouping then the UE supports LP-WUS using CN-assigned subgrouping in the bands indicated with UEID-based subgrouping capability.

Observation 4: In Rel-17 PEI, a generic PEI Subgrouping Support Indication IE was added into F1 paging message per cell level for both UE_ID based subgrouping and CN based subgrouping.

Proposal 2: LP-WUS Subgrouping support indication in F1AP follows same scheme as PEI: RAN3 to introduce a LP-WUS Subgrouping Support indication IE in the Paging Cell Item IEs in the F1 PAGING message. If received, the gNB-DU shall, if supported, consider the UE is capable to receive LP-WUS in that cell.

Proposal 3: endorse BL CR [2] for XnAP

# 4	References

[1]: R3-247007, LS on LP-WUS subgrouping, RAN2(vivo)

[2]; R3-247569: BL CR to XnAP: introduction of LP-WUS function, Ericsson