3GPP TSG-RAN WG3 Meeting #126	R3-247589

Orlando, Florida, U.S., 18th – 22nd November 2024

Agenda Item:	8.1

Source:	Ericsson

Title:	On the RAN2 LS on "UL RRC message segmentation for UECapabilityInformation"

Document for:	Discussion and Agreement

# 1	Introduction

RAN3 received an LS from RAN2 in [1] concerning UL RRC message segmentation for UECapabilityInformation.

RAN2 informs RAN3 that it wishes the gNB to be able to indicate the maximum number of UL segments the UE is allowed to use.

RAN2 points out, that it became aware of the fact that gNB limitations may stem from network implementations that do not always support the maximum number of RRC segments. Such limitations may reside in RAN or CN.

And the final request from RAN2 is as follows:

RAN2 therefore would like to ask RAN3 to define a signalling solution where CN informs RAN about the maximum UE capability size that CN can accommodate.

# 2	Discussion

While it is theoretically possible that there are limitations within RAN or within the CN, we consider such deployment/implementation aspects not being part of 3GPP standardisation work.

A different aspect may lie in the way how NG related control plane information is transported by means of the signalling TNL (SCTP).

The respective RFC explicates, that an endpoint may support fragmentation, but it must support reassembly. Further it explicates that an endpoint supporting fragmentation must fragment a user message if the size exceeds the current MTU.

Observation 1:	We expect the actual limitations in terms handling large UE Capability Information related messages do not lie in actual storing them, but rather in transmitting them via RAN interfaces.

While we consider that existing deployments do have an actual limitation in terms of message size being able to send over RAN interfaces, we also consider that such limitations are configurable and not a matter of dynamic changes, but rather static system parameters, which would rather call for an OAM solution.

We therefore propose to response to RAN2 in that sense.

Observation 2:	Existing deployments do have an actual limitation in terms of message size being able to send over RAN interfaces. It is expected that such limitations are configurable and rather static system parameters.

Proposal 1:	We propose to reply to RAN2 that RAN3 considered their case but did not agree to include the indication in NGAP.

# 3	Conclusion and Proposals

It is proposed to reply to RAN2 in the following way:

1. Overall Description:

RAN3 thanks RAN2 for their LS in R2-2409313/R3-247011 on UL RRC message segmentation for UECapabilityInformation.

RAN3 has considered the request and concluded that such information is regarded as a static system parameter that does not need application layer signalling on NG-C, however RAN2 can assume that this information is available to a gNB.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks RAN2 to take the above information into account.

# 4	References

[1]	R3-247011/R2-2409313 "LS on UL RRC message segmentation for UECapabilityInformation", LS in from RAN2 to RAN3#126