3GPP TSG-RAN WG3 Meeting #126	R3-247591

Orlando, Florida, USA, November 18th – 22nd, 2024

Agenda item:	13.2

Source: 	LG Electronics Inc.

Title: 	Keep continuing discussions on Inter-CU LTM signaling design

Document for:	Discussion and Decision

# Introduction

The discussions for Rel-19 Inter-CU LTM have consumed 4 meetings so far (from April 2024). The related RAN3 and RAN2 agreements are provided in Annex A and B, respectively.

In the last meeting, RAN3 made a little progress, some of which were reflected onto the basic stage-2 signalling flow [1], which is reproduced below.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

##### 9.2.3.5.X C-Plane Handling of inter-gNB L1/L2 Triggered Mobility

This procedure is used for the case when the UE moves from one gNB to another gNB during NR operation for LTM. Figure 9.2.3.5.X-1 shows the inter-gNB LTM procedure for intra-NR.

Figure 9.2.3.5.X-1: Signalling procedure for inter-gNB LTM

The procedure for inter-gNB LTM is as follows:

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB containing measurements of neighbouring cells.

2.	The source gNB decides to configure inter-gNB LTM.

3.	The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM.

Editor’s Note:  Working Assumption: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell

4.	Admission Control may be performed by the candidate gNB(s).

5.	The candidate gNB(s) prepares the LTM configuration(s) and sends inter-gNB response (HO REQUEST ACKNOWLEDGE) to the source gNB including the generated RRC configurations for the accepted candidate cell.

6.	The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s).

7.	The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB.

Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

8.	The source gNB sends an RRCReconfiguration message to the UE.

9.	The UE stores the LTM candidate configurations and sends an RRCReconfigurationComplete message to the source gNB.

9a	If early data forwarding is applied, the source gNB sends the EARLY STATUS TRANSFER message.

10/11. Early synchronization to some LTM candidate cell(s) belonging to the candidate gNB(s) may be performed. The candidate gNB(s) sends the TA INFORMATION TRANSFER message to the source gNB if early TA acquisition is performed to the candidate gNB(s).

Editor’s Note: Details are FFS on step 10/11.

12.	 The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the source gNB. L1 measurement should be performed as long as RRC reconfiguration (step 8) is applicable.

13.	The source gNB determines to initiate inter-gNB LTM.

14.	The source gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

15.	The source gNB sends the CELL SWITCH NOTIFICATION message to the target gNB to indicate the initiation of Cell Switch command to the UE.

16.	The target gNB detects the UE access. The UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

17/18. The target gNB sends the HANDOVER SUCCESS message to the source gNB to inform that the UE has successfully accessed the target cell. In return, the source gNB sends the SN STATUS TRANSFER message following the principles described in step 7 of Intra-AMF/UPF Handover in clause 9.2.3.2.1.

NOTE :	Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

19.	The UE sends an RRCReconfigurationComplete message to the target gNB.

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

Editor’s Note: Working Assumption: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

In this contribution, we continue providing our views for Rel-19 Inter-CU LTM signalling design.

# Discussion

## Inter-CU LTM preparation details

### HO REQ: per cell vs multiple

WA is on per cell basis:

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

3. The source gNB requests inter-gNB LTM for one or more candidate cells belonging to one or more candidate gNB(s). The source gNB initiates a HANDOVER REQUEST message per candidate cell containing one candidate cell ID and the CSI resource configuration for subsequent LTM. Editor’s Note: Working Assumption: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell

Regarding the step 3 FFS in the first round-trip HO procedure, per candidate cell is appropriate:

- As already agreed in RAN2, the maximum number of LTM candidate cells is 8, regardless of intra-CU or inter-CU. This number 8 is the same to that of CHO (maxnoofCellsinCHO = 8), that of (S-)CPAC (maxnoofPSCellCandidates = 8), and that of the combination of both (i.e. CHO+CPAC). From Rel-16, the parallel CHO preparations per cell basis have been the basis and working well.

- Enhancing the existing HO procedure messages to cater for multiple candidate cells by a single procedure (and for Inter-CU LTM only) would require significant makeshift in stage-3.

- The potential complication of error handling due to (possibly) having multiple UE-associations for the same UE over Xn between two CUs (raised by [2]) can be streamlined, if only one single UE-association is established and maintained during HO preparation phase for Inter-CU LTM.

The third bullet requires more explanations. In previous releases, initiating multiple UE associations was allowed by the source by including different Source NG-RAN node UE XnAP ID references into HO REQ messages for different candidate cells [3]:

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. The procedure uses UE-associated signalling.

The target treats those different UE associations as if they are for separate UEs. The management of their subsequent signaling (multiple associations for the same UE) was left up to the source's implementation.

This was acceptable because conditional handover was designed for one-time cell switch, and those candidate cells configurations are released in the UE and NW after the successful execution. However, more complex interactions over XnAP are expected for Inter-CU LTM, not only during preparation phase, but also during execution phase that would not end one-time. For the ease of XnAP signalling management and simplifying error handling, enforcing a single UE association for Inter-CU LTM would be beneficial.

Moreover, for the ease of subsequent LTM, RAN3-125bis has agreed the following:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

Though the solution on how to achieve UE-association in-between C-CUs is still nascent, implementing this principle can greatly streamline the subsequent LTM if the first round-trip HO preparation results in “only one” UE association between S-CU and each C-CU. If we allow multiple UE-associations from the beginning, it would complicate the identification of the same UE when establishing UE-association in-between, due to each C-CU having multiple UE associations and multiple pair of UE XnAP IDs associated with S-CU.

Proposal 1: The first round-trip (the existing HO procedure) is triggered per each candidate cell.

Proposal 2: To ease XnAP signalling management and error handling between CUs for subsequent LTM, during HO preparation, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and a single UE-associated signaling connection is established/maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

### The second round-trip (“LTM Configuration Update”) should be marked mandatory or optional?

6. The source gNB sends an LTM CONFIGURATION UPDATE message to the candidate gNB(s) to update the LTM configurations of candidate cell(s). 7. The candidate gNB(s) sends the LTM CONFIGURATION UPDATE ACKNOWLEDGE message to the source gNB. Editor’s Note: FFS on whether step 6 and 7 are mandatory or optional.

When multiple entities are involved, a two-round-trip interaction between entities is deemed inevitable for subsequent LTM because admission control and finalizing LTM configurations are not expected to be performed simultaneously. The first round-trip involves initial context setup and retrieval of necessary configurations and resources, allowing the source to collect configurations and information from all the admitted candidate cells to generate a common lower-layer configuration for these cells. The second round-trip is used to exchange the collected information and finalize LTM configurations to the UE from each involved entity.

Having this in mind, RAN3 previously discussed a similar issue at Rel-18 Inter-DU LTM, where optional was decided for the second round-trip to a candidate DU (TS 38.401 Figure 8.2.1.5-1, Steps 7-8). Though it was considered necessary for subsequent LTM, this decision was made from the fact that LTM configurations for subsequent cell switch can be prepared and configured to the UE at a later stage (i.e. after the initial LTM configuration, with the expense of additional RRC reconfiguration).

Another consideration was to cater for some “ideal” scenario (i.e. unusual), where (A) all the candidate cells chosen by CU lie in C-DU(s) (not in S-DU); and (B) those were all successfully admitted by C-DU(s) during Steps 3-4, so that CU doesn’t have to update the (common) RS resource configuration of candidate cells to update the L1 MR configuration of each candidate cell from C-DU(s). In Rel-18, only SSB-based L1 measurement was supported, and the (cell-specific) SMTC information provided to CU via the legacy non-UE associated F1 signaling (via F1 SETUP REQEST or gNB-DU CONFIGURATION UPDATE, through 9.3.1.10 Served Cell Information > Measurement Timing Configuration IE) was enough for CU to collect and generate the common RS resource configurations for LTM. As a result, CU can provide the (common) resource pool of candidate cells from the beginning.

In Rel-19, “CSI-RS”-based L1 measurement is in the scope [4], whose resource configuration is UE-specific. This means that S-CU who initiates Inter-CU LTM first need to retrieve the UE-specific CSI-RS resource configuration allocated by each C-CU to construct the (common) RS resource pool from all the candidate cells. However, this does not mean the Steps 6-7 should be mandatory, as it is still true that that those steps with each C-CU can be omitted if S-CU chooses to address subsequent LTM at a later stage.

Moreover, if we focus on “SSB”-based (also in the scope by default), the same logic in Rel-18 applies to Rel-19 Inter-CU LTM. The (cell-specific) SMTC information is already exchanged via the legacy non-UE associated Xn signalling (via Xn SETUP REQUEST/RESPONSE or NG-RAN NODE CONFIGURATION UPDATE/ACKNOWLEDGE, through 9.2.2.11 Served Cell Information NR or 9.2.2.13 Neighbour Information NR). This is why RAN3-125bis agreed the following:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

Then, the “ideal” scenario is still possible though, where (A) all the candidate cells chosen by S-CU lie in (other) candidate CU(s); and (B) those were all successfully admitted by C-CU(s) during Steps 3-4, so that S-CU doesn’t have to update the (common) RS resource configuration of candidate cells to update the L1 MR configuration of each candidate cell from C-CU(s).

Proposal 3: The second round-trip (“LTM Configuration Update”) of Steps 6-7 should be marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM. Even for “CSI-RS”-based L1 measurement (whose resource configuration is UE-specific), the second round-trip can be skipped if S-CU chooses to address subsequent LTM at a later stage.

### The second round-trip (“LTM Configuration Update”): per cell vs multiple?

The current XnAP BL CR [5] captured the initial stage-3 structure as per-cell basis:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.1.1.x3		LTM CONFIGURATION UPDATE

This message is sent by the NG-RAN node1 to update LTM configuration data.

Direction: NG-RAN node1  NG-RAN node2.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1 | YES | reject |
| NG-RAN node 2 UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2 | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| Early Sync Information | O |  | FFS |  | YES | ignore |
| Early Sync Information for SUL | O |  | FFS |  | YES | ignore |
| LTM Information Request | O |  | 9.2.1.xx1 |  | YES | ignore |

#### 9.1.1.x4		LTM CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by the NG-RAN node2 to inform the NG-RAN node1 to acknowledge update of information associated to an LTM configuration.

Direction: NG-RAN node2  NG-RAN node1.

Editor’s note: Details on IEs need to be continued.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node 1 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 1. | YES | reject |
| NG-RAN node 2 UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the NG-RAN node 2. | YES | reject |
| Target Cell Global ID (FFS) | M |  | NR CGI 9.2.2.7 |  | YES | reject |
| LTM Information Response | O |  | 9.2.1.xx2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

Note that the purpose of the second round-trip is not about modifying radio bearer details that have already been admitted. Instead, it is more like a post-LTM-preparation step to finalize LTM configurations to the UE with each C-CU, based on the common information collected from all the admitted candidate cells. The essence of LTM design lies on common L1 resource configuration and cell-specific L1 report configuration. Any update or change to the common information collected at S-CU (e.g. any modification to a set of candidate cells) may require updating the LTM configuration generated by each C-CU wholistically. If there is more than one candidate cell admitted in a C-CU, the LTM Configuration Update toward that C-CU is better to cater for those multiple candidate cells by one-shot.

Another point is that this new procedure is now being specified from scratch. This is different to the first round-trip procedure, where our beloved HO signalling has been designed per-cell basis from the beginning of NR. This per-cell HO structure has been honored from Rel-15. Enhancing the HO messages to cater for multiple candidate cells (and for Inter-CU LTM only) would require significant makeshift in stage-3. But this new “LTM Configuration Update” procedure permits full freedom to specify as we want.

Proposal 4: Design the stage-3 messages of the second round-trip (“LTM Configuration Update”) procedure to cater for the update of multiple candidate cells configurations simultaneously (i.e. not per-cell basis as in the existing HO procedure).

### The second round-trip (“LTM Configuration Update”): who can initiate?

In [5], the procedure is captured to be initiated by any involved CU. But this needs to be fixed to be triggered from the source. According to the current 38.300 BL CR [1], it is triggered from S-CU during preparation phase. Considering the potential usage of this procedure for subsequent LTM, it is expected to be triggered from the CU who is currently serving the UE (i.e. “source”).

Proposal 5: Fix the second round-trip (“LTM Configuration Update”), to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during preparation phase, and also expected to be initiated from CU who is currently serving the UE (i.e. “source”) in subsequent LTM.

### LTM Cell ID in XnAP: NR CGI (9.2.2.7) vs Target Cell Global ID (9.2.3.5)

It is true that the scope of LTM is in NR, but RAN3 has been using 9.2.3.5 Target Cell Global ID, which includes NR CGI as well:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

#### 9.2.3.25	Target Cell Global ID

This IE contains either an NR CGI or an E-UTRA CGI.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Target Cell | M |  |  |  |
| >NR |  |  |  |  |
| >>NR CGI | M |  | 9.2.2.7 |  |
| >E-UTRA |  |  |  |  |
| >>E-UTRA CGI | M |  | 9.2.2.8 |  |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

For forward compatibility, we suggest to stick to the Target Cell Global ID.

Proposal 6: For LTM Cell ID in XnAP, use 9.2.3.5 Target Cell Global ID (instead of 9.2.2.7 NR CGI).

### Details of information exchange between S-CU and C-CU

Given the Rel-18 Intra-CU LTM designs serve as baseline, the information exchanged during the first round-trip (the existing HO procedure, triggered per candidate cell) and during the second round-trip (new UE associated class-1 “LTM Configuration Update”, per node) can mirror what we have specified in Rel-18 and also from S-CPAC.

The initial works captured in the current XnAP BL CR [5] cover only a few necessary information exchanges and not complete. We propose to fill the gap, taking into account the proposals above.

Proposal 7: During the first round-trip (the existing HO procedure), the below information should be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - (O) LTM initiation indication (already by LTM Information Request IE, but requires the underlying (M) “LTM Information Setup” as in F1AP to avoid all optionals?)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE, but details of the list need more discussion)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) SSB / CSI-RS resource configuration of candidate cells, if available (already by LTM Information Request IE > CSI Resource Configuration IE, but needs more discussion)

  - (O) Early Sync Information Request (agreed, already by Early Sync Information Request IE, needs more discussion and shall be fixed to refer to 9.2.1.xx3).

- (per-cell) HO REQ ACK message to include:

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) SSB / CSI-RS resource configuration allocated from the admitted cell (not implemented yet)

  - (O) Early Sync Information, which contains DL/UL early sync info allocated from the admitted candidate cell (agreed, already by Early Sync Information Response IE, needs more discussion)

  - (O) LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, but the semantic of the mandatory existing container in HO REQ RESP shall be updated to be ignored if included)

  - (O) Data Forwarding proposal to be used for subsequent LTM (not implemented yet)

Proposal 8: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information should be exchanged:

- (per-node) LTM Configuration Update REQ message to include:

  - (M) Target Cell Global ID (FFS, shall be removed for multi-cell handling)

  - (O) Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell (FFS, already by the respective Early Sync Information / SUL IEs, needs more discussions)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion and “request” is also missing if required)

  - (O) SSB / CSI resource configuration of all the candidate cells admitted by other CU(s) (already by LTM Information Request IE > CSI Resource Configuration IE, but needs more discussion)

  - (O) LTM Configuration ID Mapping List for each admitted cell with a unique LTM candidate ID (already by LTM Information Request IE > LTM Configuration ID Mapping List IE, but details of the list needs more discussion)

  - (O) Data Forwarding TNLs assigned by other involved CU(s) (not implemented yet)

  - (O) Data Forwarding proposal from the requesting CU (not implemented yet)

- (per-node) LTM Configuration Update ACK message to include:

  - (M) Target Cell Global ID (FFS, shall be removed for multi-cell handling)

  - (O) Updated LTM candidate configuration for the admitted cell, including the (updated) L1 measurement reporting configuration (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) Data Forwarding TNLs assigned by the requested CU (not implemented yet)

Other information may need further discussions, but the above information exchange over the first/second round-trips are deemed essential for Inter-CU LTM preparation across S-CU and C-CU(s).

## UE-associated Xn signalling connections between C-CUs

RAN3-125bis achieved one big milestone for subsequent LTM handling [6]:

- 3.1.6 UE association setup For subsequent LTM, after each cell switch, the UE association need to be established between the new serving gNB and candidate gNB(s). Based on companies’ contributions, there are following proposals: In [5040, Nokia]: Proposal 9: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM MODIFICATION REQUEST message to update the information related to a given UE context. In [5050, ZTE]: Proposal 12: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used. In [5202, HW]: One possible solution is that each candidate gNB allocates UE XnAP ID and send to other candidate gNBs in the preparation step. Another solution is that after each cell switch, the new serving gNB allocates the source node UE XnAP ID and send it to other candidate gNB-CUs, and then the candidate gNB-CUs reply with the target node UE XnAP ID. This UE XnAP ID negotiation can be done for example, during the handover success notification procedure. Proposal 14: RAN3 to discuss the UE associated Xn signalling connection setup between candidate gNBs in subsequent LTM case. In [5316, Ericsson]: Proposal 6: To support subsequent LTM in the RAN, the new XnAP procedure (e.g., LTM Configuration Update) is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs. In [5568, LGE]: Proposal 7: Discuss and consider allowing the UE associated XnAP signalling connections directly between C-CUs, for the unified/simplified specification handling and reduced signaling overhead. Moderator summarized above proposals in to following options to be further discussed: - Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and send to other candidate gNBs. [HW, LGE] - Option 2: After each cell switch, the new serving gNB signals XnAP ID to other candidate gNBs, and then the candidate gNB reply with the Target node UE XnAP ID. [Nokia, HW, Ericsson] - Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB. [ZTE] Question: Which option is agreeable? And which message to use? Moderator summary: Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM, when and how to establish the UE association is FFS.

From the nature of Inter-CU LTM, this agreement was natural since Inter-CU LTM can be viewed as a series of NW-triggered conditional handovers executed sequentially. In Rel-16, CHO was designed only for single execution from the source currently serving the UE toward one of the candidate targets. If executed serially, the selected target becomes the new radio access management point of the UE, and subsequently determines, as a “source”, for the next handover of the served UE toward another RAN node, and so on. This nature allows the XnAP descriptions based on the "source" and "target" to be applied successively for subsequent Inter-CU LTM.

From this sense, RAN3 acknowledged the fact that, once we have direct UE-associations in-between C-CUs, any procedure triggered by a C-CU at any point of time can be made to work as effective as from S-CU for initial PCell switch, and our specification efforts and signaling overhead can be greatly reduced as their stage-2/3 descriptions can remain universally applied regardless of how many CUs are involved for LTM.

Having this great milestone, however (after careful thoughts), we would like to recommend RAN3 not hasty to deep dive into the solution space. RAN3 just started thinking about the solutions, and the listed potential solutions in the SOD [6] are still not clear enough to discuss and compare pros and cons for down-selection.

In fact, there are two critical aspects to be further agreed and touched upon before deep dive.

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

### (1) How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

First, we would like to point out that this issue doesn’t exist when only one C-CU (other than S-CU) is involved from the beginning (i.e. a total of two CUs). In this case, there is already a direct UE-association between two CUs (S-CU <> C-CU) over Xn, and it can be re-used for subsequent LTM. Only when the number of CUs involved exceeds two (i.e. more than one C-CU), this issue becomes evident for establishing direct UE-associations in-between C-CUs.

Having that in mind, once an initial Inter-CU PCell switch happens from S-CU toward a candidate PCell in one C-CU, we first observe that this C-CU needs to know the existence of another CU(s) involved for the same UE’s LTM. Otherwise, establishing direct UE-associations (or even first seeking for Xn connectivity if not connected beforehand) would not be a question.

Overall, we think that leveraging the LTM Configuration ID Mapping List IE is the best way. This IE is used for assigning the unique identifiers for each candidate cell configuration, allocated and managed by S-CU (RAN3-125bis already agreed: The LTM Configuration IDs are allocated by the source CU). Moreover, similar to the (common) L1 resource configurations of candidate cells, this IE shall also be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations. Currently, this IE is captured at bare minimum in the current BL CR, i.e. a list of IDs (1 to 8) for each candidate cell. But if we can extend this IE to also include the information of which candidate cell belongs to which CU, any recipient CU of this IE would be able to know which CUs are involved for the UE’s LTM and which candidate cells are currently planned.

In fact, making involved entities visible with each other is nothing new. In Rel-18 of (Intra-CU) Inter-DU LTM, each involved DU was also visible to other DUs based on early UL sync information handling. The preamble indexes for early UL synchronization is allocated and shared per DU basis, and the early UL sync resource request from CU can indicate a list of involved DU IDs [7]. Though this aspect was not addressed by F1AP LTM Configuration ID Mapping List IE, we can say that the similar handling of letting each involved DU know the existence of other involved DU(s) was already in place in Rel-18.

Moreover, one advantage of this approach observed is that timing of information update (i.e. “when”) doesn’t need to be worried about. As mentioned above, the LTM Configuration ID Mapping List IE shall anyway be maintained up-to-date and synched to all the involved entities throughout LTM operations of a UE. Whenever there is a need for update, it will be updated in NW side and UE side (though synch to the UE may not be always required case-by-case).

### (2) How to identify the same UE?

In the past, identifying the same UE context already established in one entity upon request from other entity was addressed in RAN3 based on “UE XnAP ID” [8].

- During Inter-MN HO without SN change, the same SN identifies the already established UE context when the target MN decides to add the same SN, by the “SN UE XnAP ID” included in SN ADD REQ, which is forwarded from the source MN via HO REQ.

- Another practice was at Rel-17 CHO with NR-DC, where the same target SN identifies the same UE SN addition request from different target MN(s), based on the pair of the source MN node ID and the MN UE XnAP ID included in each SN ADD REQ, which is also forwarded from the source MN via HO REQ.

Two slightly different approaches were taken for each practice. In the first case, an entity requesting for the same UE identification directly provided the UE XnAP ID assigned by the recipient before. For the second case, the requesting entities provides the common UE XnAP ID for the recipient to identify that multiple requests (from different entities) are indeed for the same UE, where the common ID was selected by the identifier assigned at the source side.

When it comes to having direct UE-associations in-between C-CUs, the first approach can be applied as follows: Each C-CU would have already established a UE-association with S-CU and the corresponding UE XnAP ID has been anyway assigned for the interaction with S-CU (during the first round-trip HO preparation). When a C-CU initiating to establish direct UE-association with other C-CU, the requesting C-CU can use the UE XnAP ID (assigned by the receiving C-CU for interaction with S-CU), to be used as a reference to the UE context established in the receiving C-CU. Then, the pre-requisite will be for each C-CU to know the UE XnAP IDs assigned by other C-CUs (for interaction with S-CU), before it initiates direct UE-association with other C-CU(s).

The second approach can be worked out by a C-CU who initiating to establish direct UE-association with other C-CU uses the UE XnAP ID assigned by S-CU toward the receiving C-CU (during initial preparation). Then, the pre-requisite will be for each C-CU to know the UE XnAP IDs assigned by S-CU toward other C-CUs. In addition, this may require to include the S-CU node ID as well to refer to the right UE context, because from the receiving CU point of view, the ID used for identifying a UE is the UE XnAP ID assigned by the other node (here by S-CU).

Or, both approaches can be mixed up to provide both UE XnAP IDs assigned by S-CU and by C-CU (during preparation phase).

In fact, there could be several ways to address the issue of identifying the same UE. Regardless of which approach to take, a follow-up question will be about “timing”, i.e. when those UE XnAP IDs shall be shared across C-CUs. From the companies’ contributions, there seems three camps overall – (a) shared during the preparation phase; (b) shared after each cell switch (from the old serving CU?); (c) before cell switch?. But overall, their exact details are unclear and both approaches mentioned above could be mixed up.

Give this status, it may be better to agree the high-level methodology on how to identify the same UE (based on UE XnAP ID), and further discuss solution details including the exact timing of sharing the same UE identifiers across C-CUs.

Proposal 9: For direct UE-associations between C-CUs, first discuss two aspects before deep dive:

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 10: For (1), consider enhancing the LTM Configuration ID Mapping List IE to include the information of which candidate cell belongs to which CU, which shall be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations, thereby no need to worry about the timing of information update.

Proposal 11: For (2), agree to use “UE XnAP ID” as baseline for identifying the same UE, when establishing direct UE-associations in-between C-CUs.

- FFS on which UE XnAP ID to be used, either one allocated by S-CU during initial preparation toward the receiving C-CU, or allocated by the receiving C-CU during initial preparation (for interaction with S-CU), or both.

- FFS on when to be shared across C-CUs.

## E1AP impacts

E1AP was promoted as one of the impacted specs in the previously updated WID [9], based on the following observations [10]:

E1-AP potential impacts

1.       Add the abbreviations of LTM/Conditional LTM.

2.       Clarify that some procedures can be reused for LTM (e.g. Early Forwarding SN Transfer).

3.       The enhancement on the BEARER CONTEXT SETUP procedure to indicate the request concerns a conditional LTM cell switch.

4.       The security handling for inter-CU LTM (Maybe, depends on RAN2 solution).

5.       Some enhancement on the UP handling to support early data forwarding for LTM (Maybe, depends on RAN3 solution).

The current E1AP BL CR on the table [11] aims to address (1) and (2).

Proposal 12: Endorse the E1AP BL CR proposed in [11].

While (4) and (5) are further to discuss, we think (3) is also quite straightforward, given that

- LTM shares the similar design philosophy with conditional mobility, where serving cell switch does not happen immediately upon RRC configured. Moreover, the famous IE in the BEARER CONTEXT SETUP REQEUST message for one-bearer-context handling in CU-UP for multiple CHO requests applies the same.

- From CU-UP standpoint, no difference is observed between non-Conditional LTM and Conditional LTM for (3).

For (3), we just need to extend the usage of the famous CHO Initiation IE in the BEARER CONTEXT SETUP REQUEST message to cater for LTM as well (which was neglected in Rel-18), and their impact is small:

If the CHO Initiation IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall consider that the request concerns conditional handover or conditional PSCell change or conditional PSCell addition or subsequent CPAC or LTM and act as specified in TS 38.401 [2].

Proposal 13: Extend the usage of the famous CHO Initiation IE in the E1AP BEARER CONTEXT SETUP REQUEST message to cater for LTM as well.

## Others

Regarding TA handling, there were several additions worked out in Rel-18 that CU can share with a DU: (a) a known TA value (i.e. TA value = 0) between the current serving cell and a candidate cell; and (b) whether UE-based TA measurement can be applied (and configured to the UE) in-between the serving and candidate cells. The former was an optimization to cater a special deployment (co-located cells), while the latter was one of early UL synchronization mechanism specified that is utilized only when DU knows so.

While those TA handling perfectly fits within a single CU scope (as specified in Rel-18), a question is whether they can be applicable between cells under different CUs.

It is hard to imagine that either co-located cells are managed under different CUs, or the 260ns TAE (timing alignment error) requirement could be satisfied between cells under different CUs [12].

Before extending, the usage of those TA handlings across CUs shall be validated.

Proposal 14: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

# Conclusion

Based on the discussion in the present contribution and the observations above we propose:

## Inter-CU LTM preparation details

Proposal 1: The first round-trip (the existing HO procedure) is triggered per each candidate cell.

Proposal 2: To ease XnAP signalling management and error handling between CUs for subsequent LTM, during HO preparation, enforce only one UE association for parallel transactions. That is, parallel transactions are allowed using the same source UE AP ID only, each identified by the target cell ID, and a single UE-associated signaling connection is established/maintained (as shown below):

### 8.2.1 Handover Preparation #### 8.2.1.1 General This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same. If the procedure concerns an LTM, parallel transactions are allowed using the same source UE AP ID only. In this case, possible parallel requests are identified by the target cell ID, and only one UE-associated signaling connection is established and maintained between the source NG-RAN node and the target NG-RAN node. The procedure uses UE-associated signalling.

Proposal 3: The second round-trip (“LTM Configuration Update”) of Steps 6-7 should be marked as optional, following what we have done similarly for Rel-18 Inter-DU LTM. Even for “CSI-RS”-based L1 measurement (whose resource configuration is UE-specific), the second round-trip can be skipped if S-CU chooses to address subsequent LTM at a later stage.

Proposal 4: Design the stage-3 messages of the second round-trip (“LTM Configuration Update”) procedure to cater for the update of multiple candidate cells configurations simultaneously (i.e. not per-cell basis as in the existing HO procedure).

Proposal 5: Fix the second round-trip (“LTM Configuration Update”), to be initiated from the source toward the target (similar to the existing HO procedure). It is initiated from S-CU during preparation phase, and also expected to be initiated from CU who is currently serving the UE (i.e. “source”) in subsequent LTM.

Proposal 6: For LTM Cell ID in XnAP, use 9.2.3.5 Target Cell Global ID (instead of 9.2.2.7 NR CGI).

Proposal 7: During the first round-trip (the existing HO procedure), the below information should be exchanged (following similarly what we have done in Rel-18 Intra-CU LTM and S-CPAC):

- (per-cell) HO REQ message to include:

  - (O) LTM initiation indication (already by LTM Information Request IE, but requires the underlying (M) “LTM Information Setup” as in F1AP to avoid all optionals?)

  - (O) LTM Configuration ID Mapping List (already by LTM Information Request IE > LTM Configuration ID Mapping List IE, but details of the list need more discussion)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) SSB / CSI-RS resource configuration of candidate cells, if available (already by LTM Information Request IE > CSI Resource Configuration IE, but needs more discussion)

  - (O) Early Sync Information Request (agreed, already by Early Sync Information Request IE, needs more discussion and shall be fixed to refer to 9.2.1.xx3).

- (per-cell) HO REQ ACK message to include:

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) SSB / CSI-RS resource configuration allocated from the admitted cell (not implemented yet)

  - (O) Early Sync Information, which contains DL/UL early sync info allocated from the admitted candidate cell (agreed, already by Early Sync Information Response IE, needs more discussion)

  - (O) LTM candidate configuration for the admitted cell (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE, but the semantic of the mandatory existing container in HO REQ RESP shall be updated to be ignored if included)

  - (O) Data Forwarding proposal to be used for subsequent LTM (not implemented yet)

Proposal 8: During the second round-trip (new UE associated class-1 “LTM Configuration Update”), the below information should be exchanged:

- (per-node) LTM Configuration Update REQ message to include:

  - (M) Target Cell Global ID (FFS, shall be removed for multi-cell handling)

  - (O) Early Sync Information List, which contains DL/UL early sync info per each admitted candidate cell (FFS, already by the respective Early Sync Information / SUL IEs, needs more discussions)

  - (O) LTM reference configuration or its request (already by LTM Information Request IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion and “request” is also missing if required)

  - (O) SSB / CSI resource configuration of all the candidate cells admitted by other CU(s) (already by LTM Information Request IE > CSI Resource Configuration IE, but needs more discussion)

  - (O) LTM Configuration ID Mapping List for each admitted cell with a unique LTM candidate ID (already by LTM Information Request IE > LTM Configuration ID Mapping List IE, but details of the list needs more discussion)

  - (O) Data Forwarding TNLs assigned by other involved CU(s) (not implemented yet)

  - (O) Data Forwarding proposal from the requesting CU (not implemented yet)

- (per-node) LTM Configuration Update ACK message to include:

  - (M) Target Cell Global ID (FFS, shall be removed for multi-cell handling)

  - (O) Updated LTM candidate configuration for the admitted cell, including the (updated) L1 measurement reporting configuration (already by LTM Information Response IE > Target NG-RAN node To Source NG-RAN node Transparent Container IE)

  - (O) LTM reference configuration if requested (already by LTM Information Response IE > Reference Configuration IE, but FFS pending RAN2, needs more discussion)

  - (O) Data Forwarding TNLs assigned by the requested CU (not implemented yet)

## UE-associated Xn signalling connections between C-CUs

Proposal 9: For direct UE-associations between C-CUs, first discuss two aspects before deep dive:

- How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with?

- How to identify the same UE?

Proposal 10: For (1), consider enhancing the LTM Configuration ID Mapping List IE to include the information of which candidate cell belongs to which CU, which shall be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations, thereby no need to worry about the timing of information update.

Proposal 11: For (2), agree to use “UE XnAP ID” as baseline for identifying the same UE, when establishing direct UE-associations in-between C-CUs.

- FFS on which UE XnAP ID to be used, either one allocated by S-CU during initial preparation toward the receiving C-CU, or allocated by the receiving C-CU during initial preparation (for interaction with S-CU), or both.

- FFS on when to be shared across C-CUs.

## E1AP impacts

Proposal 12: Endorse the E1AP BL CR proposed in [11].

Proposal 13: Extend the usage of the famous CHO Initiation IE in the E1AP BEARER CONTEXT SETUP REQUEST message to cater for LTM as well.

## Others

Proposal 14: Do not consider additional TA handlings specified in Rel-18 across CUs (a known TA value and UE-based TA measurement, both of which were specified applicable within a single CU), unless their usage across CUs is validated.

The corresponding XnAP TP is also proposed in [13].

# Reference

[1] R3-247031, “(BL CR to 38.300) Support for Inter-CU LTM”, Nokia, Huawei, Google, China Telecom, NEC, Ericsson, LGE, ZTE, CATT, Samsung

[2] R3-244355, “Further discussion on XnAP signaling for inter-CU LTM”, Ericsson

[3] TS 38.423, NG-RAN; XnAP (Release 18), v18.3.0

[4] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[5] R3-247032, “(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics

[6] R3-245747, “Summary of offline on LTM procedures”, China Telecom, Lenovo

[7] TS 38.473, NG-RAN; F1 F1AP (Release 18), v18.3.0

[8] TS 37.340, E-UTRA and NR; Multi-connectivity; Overall Description; Stage 2 (Release 18), v18.3.0

[9] RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple, China Telecom

[10] RP-241516, “Discussion on rel-19 mobility enhancements WID revision”, Apple Inc, China Telecom

[11] R3-247590, “Baseline CR for introducing Rel-19 Mobility enhancement in E1AP”, LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC

[12] R3-235019/R4-2314455, Reply LS on beam application time and UE based TA measurement for LTM, RAN4

[13] R3-247592, “(TP for NR_Mob_Ph4 TS 38.423 BL CR)”, LG Electronics Inc.

# Annex A (RAN3 agreement for Inter-CU LTM)

## RAN3-125bis (Hefei, October 2024)

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: The source gNB-CU sends the CSI resource configuration of candidate cells to other candidate gNB-CUs via Handover Request message, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

## RAN3-125 (Maastricht, August 2024)

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

## RAN3-124 (Fukuoka, May 2024)

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

## RAN3-123bis (Changsha, April 2024)

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM. When to trigger can be further discussed.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

# Annex B (RAN2 agreement for Inter-CU LTM)

## RAN2-127bis (Hefei, October 2024)

Agreements on inter-CU LTM

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Agreements related to Inter-SN LTM

- The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3.

- Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline:

- The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part.

- FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations.

- The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message.

- Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed.

- The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration.

- The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part.

- The MN can request an SCG reference configuration from any of the involved SNs.

5.	For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7.	The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

9.	Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message.

10.	Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

11.	RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Agreements on L1 event triggered MR

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not).

N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12.	NW can configure a dedicated SR configuration for MR MAC CE transmission.

## RAN2-127 (Maastricht, August 2024)

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

## RAN2-126 (Fukuoka, May 2024)

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

## RAN2-125bis (Changsha, April 2024)

Agreements on scenarios:

- RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

- Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

- Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.