3GPP TSG-RAN WG3 Meeting #126	R3-247597

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.1

Source:	Nokia

Title:	Counting number of RRC_INACTIVE SDT UEs

Document for:	Discussion and Decision

# 1	Discussion

At RAN3#126, RAN3 receives the incoming LS in R3-24247010 concerning the counting the number of UEs in RRC_INACTIVE state with SDT data transmission.

In the new LS in R3-24247010, RAN2 gives their answer, indicating some issues with the accuracy of such counting:

RAN2 would like to thank SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. RAN2 confirms the network knows the number of UEs in RRC_INACTIVE state in a cell with an ongoing SDT session as the connection requests from such UEs are distinguished either based on dedicated RACH resources or based on a dedicated RRC resume cause. However, RAN2 would also like to inform SA5 that occasionally an SDT procedure may be unsuccessfully completed due to cell re-selection during SDT procedure, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold or integrity check failure while SDT procedure is ongoing. In these cases, the network would not be aware that the UEs SDT session is already terminated at the UE side, but would consider SDT session as ongoing until timer T319a expires. T319a is started when the SDT session starts and its maximum value is 4 seconds.

In this LS [1] RAN3 is in Cc but this is a mistake originating from the initial LS from SA5 in R3-245025 [2] which RAN3 received at RAN3#125bis.

The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.

In fact, this original LS is defining requirements for evaluation of cell load. This completely falls into RAN3 territory and competence. Moreover, it is important to recall that RAN3 is the group responsible for setting the O&M requirements for the RAN.

Observation 1: RAN3 is responsible for the O&M requirements and to decide the relevance of the requested counting.

Counting the number of UEs in RRC_INACTIVE state with SDT data transmission in a cell has multiple shortcomings:

First, it is not possible to locate RRC_INACTIVE UEs in a cell because the UEs move across the RNA: in general, an RNA comprises many cells and may even comprise cells of multiple gNBs. Imagine that 100 RRC_INACTIVE UEs are located in an RNA of 10 cells. In this case, taking an average value dividing the number of UEs in the RNA by the number of cells would make 100/10=10 UEs per cell in average. But this is very misleading because in reality it is possible than some cells have zero UEs.

If it is not possible to count the number of RRC_INACTIVE UEs in a cell, it is even more impossible to count the number of RRC_INACTIVE UEs in a cell which have been configured for SDT which is a subset of the former.

Observation 2: it is not possible to know the number of RRC_INACTIVE UEs in a cell configured with SDT.

When UE configured with SDT start to transmit, depending on the size of data they can transmit in two modes:

- They can switch to RRC connected state if more than a few packets are to be sent,

- They can transmit in SDT-mode if only a few packets are to be sent.

For the first case, the contribution to the cell load status is well known by current counters.

For the second case, an SDT transaction has very low volume of data exchanges which are a few packets; also the duration of this exchange is less than four seconds. Compared to the load induced by RRC connected traffic, this is negligible.

Observation 3: The contribution of SDT traffic to cell load status is negligible compared to the overall traffic.

Compared to the pain to collect such information in the gNB, we therefore conclude with the following proposal:

Proposal 1: RAN3 to conclude negatively on introducing a new counter for number of RRC_INACTIVE UEs in a cell with SDT transmission.

Proposal 2: send RAN3 conclusion and guidance to SA5 as per the reply LS provided in [3].

# 2	Conclusion and Proposals

This paper has analysed the proposal received in R3-247010 to count the number of RRC_INACTIVE UEs in a cell with SDT transmission and made the following observations:

Observation 1: RAN3 is responsible for the O&M requirements and to decide the relevance of the request counting.

Observation 2: it is not possible to know the number of RRC_INACTIVE UEs in a cell configured with SDT.

Observation 3: The contribution of SDT traffic to cell load status is negligible compared to the overall traffic.

The analysis leads to the following proposals:

Proposal 1: RAN3 to conclude negatively on introducing a new counter for number of RRC_INACTIVE UEs in a cell with SDT transmission.

Proposal 2: send RAN3 conclusion and guidance to SA5 as per the reply LS provided in [3].

# 3	References

- R3-247010, Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission

- R3-245025, LS on Number of UEs in RRC_INACTIVE state with data transmission

- R3-247599, Response LS on Number of UEs in RRC_INACTIVE state with data transmission