3GPP TSG-RAN WG3 Meeting #126	R3-247610

# Orlando, USA, November 18 – 22, 2024

Agenda item:	16.2

Source: 	Samsung

Title: 	Remaining issues on RAN architecture for Ambient IoT

Document for:	Discussion and decision

Introduction

Last meeting RAN3 continued to discuss the RAN architecture for Ambient IoT, and the following open issue is captured,

In Topology 1 the signalling transport for XXAP termination at the AIoT RAN node is SCTP/IP. Capture in the TP other transport layer options had been discussed in this study is not pursued.

To only consider no split architecture, and capture this in the TR.

An AIoT enabled gNB could support both topologies, this is an implementation matter.

The RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers, for both T1 and T2.

In T1, an A-IoT RAN node may serve one or multiple readers.

For reader selection:

in T1, reader selection may need the coordination between AIoT RAN and AIoT CN;

in T2, all options are still open, different alternatives are possible as per call flows.

Continue to discuss the architectures and protocol stack.

In this contribution, we further discuss the RAN architecture aspects for Ambient IoT.

Discussion

The RAN architecture aspect for Ambient IoT was discussed last several meetings. Till now, the most controversial issue is about how to capture the protocol stack and the call flow for Topology 2.

Note that there are basically three agreed solutions for Topology 2 which directly impact the design of the the protocol stack. The solutions are listed as follows,

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

Editor’s note 2: Depiction and further details of the options above are FFS

In addition, according to the interim conclusion drawn by SA2, the RRC based solution and the UP based solution will be specified while the NAS based solution is FFS.

8.1.3	Architecture to Support Topology 2

8.1.3.1	General

Two options will be specified for Topology 2:

-	User-plane option as defined in clause 8.1.3.2.

-	RRC-based option as defined in clause 8.1.3.3.

Editor's note:	Whether a NAS-based option is needed in addition is FFS.

When looking at the definition on the solutions to support T2 as captured in TR 38.769, in fact the definitions on RRC based solution and NAS based solution are quite vague. We are not sure all the companies are on the same page on the definition of RRC based solution and NAS based solution.

Firstly, the RRC based solution indicates that the AIoT upper layer information is transmitted explicitly over XXAP signalling, does it mean that all the AIoT upper layer information will be explicitly transmitted over XXAP signalling, or does it mean that only the assistance information which triggers the resource allocation by AIoT enabled gNB is explicitly transmitted over XXAP signalling. Depending on all the ongoing discussions, I think different companies have different interpretations.

Secondly, the NAS based solution indicates that the AIoT upper layer information is transmitted over NAS, and companies are pursuing to add UE requested method as well as CN requested method to solve the issue on how to perform resource allocation by RAN in the NAS based solution.

By the analysis above, actually RAN3 does not formally discuss the possibility that some assistance information can be provided from AMF/AIoTF to AIoT enabled gNB to trigger the resource allocation by the AIoT enabled gNB. And it is also probable that such assistance information can be part of (not necessarily all) the AIoT upper layer information. The assistance information can be AIoT device ID, Area Scope, Transaction/Session ID and/or the estimated number of AIoT devices, and we believe such assistance information is enough to trigger the AIoT enabled gNB to perform resource allocation. No matter which solution (RRC or NAS) the assistance information approach belongs to, we think it is worthy to be investigated.

Actually in our understanding, the first thing we need to understand is what is the detailed content to be included in the AIoT upper layer information, which still needs further progress in SA2, then we can decide which information can be made explicit over XXAP/NGAP.

Proposal 1: It is also possible that only part of the AIoT upper layer information are explicitly transmitted over XXAP/NGAP signaling for either RRC based solution or NAS based solution, and such explicitly transmitted information are used to trigger the AIoT enabled gNB to perform resource allocation.

Conclusion

In this paper, we provide further discussion on RAN architecture for Ambient IoT. The following proposals are provided,

Proposal 1: It is also possible that only part of the AIoT upper layer information are explicitly transmitted over XXAP/NGAP signaling for either RRC based solution or NAS based solution, and such explicitly transmitted information are used to trigger the AIoT enabled gNB to perform resource allocation.

Reference

[1] TR 38.769 v1.0.0