3GPP TSG-RAN WG3 Meeting #126	R3-247617

Orlando, USA, 18 – 22 November, 2024

Agenda item:	11.4

Source:	Nokia, Telecom Italia, Jio, Deutsche Telekom

Title:	Discussion on Energy Cost Reporting in Split Architecture

Document for:	Discussion and Decision

# 1	Introduction

In RAN3 #125bis the following was agreed related to the reporting of measured Energy Cost from gNB-DU to gNB-CU:

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In this contribution we provide our views regarding the F1AP Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface. We also provide some observations related to the Energy Cost index calculation in the gNB-DU, based on the existing measurements provided by SA5.

# 2	Discussion

In the last meeting we agreed to align F1AP signalling procedures for the reporting of Energy Cost from gNB-DU to gNB-CU to the XnAP signalling procedures we introduced in Rel-18.

A gNB-CU can request Energy Cost measurements from a gNB-DU. We propose to introduce a bit in Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message so that a gNB-CU can request the reporting of an Energy Cost measurement from a gNB-DU.

Proposal 1: Introduce a bit in Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message to allow gNB-CU to request the reporting of an Energy Cost measurement from a gNB-DU.

Similarly to the corresponding XnAP procedures, both one-time and periodic reporting over F1 interface should be supported. In case of periodic reporting, we think we should align the reporting periods over Xn and F1.

Proposal 2: Support periodic reporting by introducing in the DATA COLLECTION REQUEST message a Reporting Periodicity for Data Collection IE that is encoded as ENUMERATED and takes values “500ms, 1000ms, 2000ms, 5000ms, 10000ms, …”.

A corresponding F1AP CR is submitted to this meeting in [1].

The Energy Cost measurement is reported as an index that takes values in the range between 0 and 10000. A gNB can calculate the Energy Cost index by using an operator-defined mapping rule that maps an Energy Consumption value to an Energy Cost index. During the Rel-19 study item phase, we discussed which entities of the gNB in split architecture shall be considered for the Energy Cost measurement, namely gNB-DU, gNB-CU-CP, gNB-CU-UP. In the end, we concluded that the essential part of a node’s Energy Consumption is due to the radio and therefore we agreed to consider only the Energy Cost in the gNB-DU. Based on that, we agreed to transfer the measured Energy Cost from gNB-DU to gNB-CU.

One question that arises is how the gNB-DU can measure its Energy Consumption. In TS 28.554 [2] different Energy Consumption measurements are provided including NF Energy Consumption and Energy Consumption of a gNB. However, there is no measurement to describe the Energy Consumption of a gNB-DU. It is therefore currently unclear how a gNB-DU can map its Energy Consumption to an Energy Cost measurement.

Proposal 3: Introduce a new measurement for the Energy Consumption of a gNB-DU.

If RAN3 confirms the need to introduce a new Energy Consumption measurement of a gNB-DU, RAN3 needs to send an LS to SA5 to request SA5 to introduce a new measurement in TS 28.554.

We provide a draft LS to SA5 in [3].

# 3	Conclusion

In this paper we provide the following proposals:

Proposal 1: Introduce a bit in Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message to allow gNB-CU to request the reporting of an Energy Cost measurement from a gNB-DU.

Proposal 2: Support periodic reporting by introducing in the DATA COLLECTION REQUEST message a Reporting Periodicity for Data Collection IE that is encoded as ENUMERATED and takes values “500ms, 1000ms, 2000ms, 5000ms, 10000ms, …”.

Proposal 3: Introduce a new measurement for the Energy Consumption of a gNB-DU.

# References

[1]		R3-247618, “ Procedures over F1 for the exchange of Energy Cost index”, Nokia

[2]		3GPP TS 28.554, “5G end to end Key Performance Indicators (KPI)” (Rel.18)

[3]		R3-247619, “[Draft] LS on New Energy Consumption Measurement for gNB-DU”, Nokia