3GPP TSG-RAN WG3 Meeting #126	                                                             R3-247645

Orlando, USA, Nov 18 - Nov 22, 2024

Agenda item: 	    10.2

Source: 	China Unicom

Title: 	Discussion on MRO enhancements

Document for:	Discussion

1	Introduction

At RAN#102, the new Work Item on Data Collection for SON_MDT in NR standalone and MR-DC phase 4 was agreed [1] which introduced the following objectives in the core part:

The objective of this work item is to specify data collection enhancement in NR standalone and MR-DC for SON/MDT purpose. The specific objectives of this work item are: - MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] - Support of SON/MDT enhancements for [RAN3, RAN2]: - Intra-NTN mobility - Network Slicing - Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]: - RACH optimization for SDT - MHI Enhancement for SCG Deactivation/Activation - MRO for MR-DC SCG failure If needed, co-operate with RAN1, SA2, SA5, CT4.

This contribution provides our views on the scenarios of MRO enhancements which can be supported in Rel-19.

2	Discussion

2.1	LTM

MRO for LTM For TP to BL CR to TS 38.300: - Remove editor’s note 1 and 2. No need to capture LTM recovery in the failure definitions. - For detection mechanism, use same approach as for CHO i.e. reuse existing sub-bullets to add LTM. - Change“successful handover" definition, as proposed in R3-245230. For TP to BL CR to TS 38.401: - Capture wrong LTM prepared cell selection. Add failure type to F1AP ACCESS AND MOBILITY INDICATION message. CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell. What additional information need to be added to F1AP Access and Mobility Indication message? - C-RNTI? RACH information in LTM: RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

At the RAN3#125bis meeting, we have been made above agreements and introduced stage-2 definition of LTM detection mechanism.

Due to time constraints, we didn't discuss RACH-less and RACH-based access related topic at the last meeting, following the agreement from RAN3#124 meeting stated below.

RAN3 to discuss enhancements related to the information of RACH-less access and RACH-based access from UE related to MRO for LTM.

Regarding enhancements related to the information of RACH-less access and RACH-based access for LTM, it will be helpful for network to distinguish whether UE perform RACH-less LTM or RACH-based LTM for further optimization which can be included in the RLF report and SHR report.

Proposal 1: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

For legacy mobility, timer-based triggering conditions T304, T310 and T312 are introduced, which can be further reused in the LTM procedure in a straightforward way.

Proposal 2: The T304, T310 and T312 related triggering conditions can be reused for LTM SHR.

Regarding adding additional information to the F1AP Access and Mobility Indication message, we already agreed including failure type in the message for DU to do the root cause analysis. And for other additional information, there is no need to be added in the F1AP message which is already included in the RLF report container.

Proposal 3: No need to add C-RNTI in the F1AP Access and Mobility Indication message.

2.2	CHO with candidate SCGs

In last RAN3#125bis meeting, we have the following agreements.

MRO for CHO with Candidate SCG(s)

RLF report is enhanced to including

  - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered

  - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered

  - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

  - The fulfilled events before the RLF is encountered?

As MRO for CHO and MRO for CPAC are supported in previous release, which can be  considered as base line for R19 MRO for CHO with candidate SCG(s). RAN3 can reuse RRC recovery procedure, RRC reestablishment procedure, or RRC re-connected procedure and SCGFailureInformation message for R19 MRO for CHO with candidate SCG(s). To distinguish it from the previous cases, new IE can be introduced referring to the previous spec design.

Proposal 4: It is proposed to reuse same procedure and message for R19 MRO for CHO with candidate SCG(s) from MRO for CHO and MRO for CPAC.

2.3	Subsequent CPAC

A conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, or a SCG release, based on pre-configured subsequent CPAC configuration of candidate PSCell(s). UE would keep the subsequent CPAC configuration after a PSCell change, and if the execution condition of other one candidate PSCell is satisfied, UE may apply the stored configuration and synchronises to the candidate PSCell.

In that case, subsequent CPAC failure may occur at the first configured SN modification, as well as subsequent PSCell changes scenario. When subsequent CPAC fails, SCG FAILURE INFORMATION REPORT can be further enhanced to record the corresponding configurations for root cause analysis. Meanwhile, RAN3 needs to discuss how to trigger the forwarding of failure report to the configured node with the potential impacts on UE recording the mobility history information.

And regarding the differentiation between initial and following CPC, we don’ t see the strong intention to separate the two case.

Proposal 5: There is no need to differentiate between initial and following CPC.

For subsequent CPAC, network cannot log the UHI since no more SN adding procedure during the mobility phase based on the existing UHI mechanism,

Proposal 6: UHI mechanism can be enhanced for subsequent CPAC.

3	Conclusion

Proposal 1: Introduce an indication indicating the RACH-based or RACH-less in RLF report and SHR report.

Proposal 2: The T304, T310 and T312 related triggering conditions can be reused for LTM SHR.

Proposal 3: No need to add C-RNTI in the F1AP Access and Mobility Indication message.

Proposal 4: It is proposed to reuse same procedure and message for R19 MRO for CHO with candidate SCG(s) from MRO for CHO and MRO for CPAC.

Proposal 5: There is no need to differentiate between initial and following CPC.

Proposal 6: UHI mechanism can be enhanced for subsequent CPAC.

4   References

- RP-234038: New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4; TSG RAN Meeting #102, Edinburgh, Scotland, Dec 11-15, 2 023