3GPP TSG-RAN WG3 Meeting #126	R3-247648

Orlando , US, 18th – 22nd Nov 2024

Agenda Item:		12.3

Source:		Baicells

Title:		Discussion on NR Femto Architecture and Functionality

Document for:		Discussion and decision

- Introduction

In our #125-bis meeting, we have reached a consensus on the architecture for NR Femto, as outlined below:

# 4 Overall Architecture and Functional Split ## 4.X Support of NR Femtos ### 4.X.1 Architecture Figure 4.X.1-1 shows a logical architecture for the NR Femto that has a set of NG interfaces to connect the NR Femto to the 5GC. Figure 4.X.1-1: NR Femto Logical Architecture NOTE: the SeGW is out of RAN scope. The NR Femto may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto and the 5GC. Based on implementation, the transport of NG-U between the NR Femto and the 5GC may be optionally concentrated in the NR Femto GW. For NR Femto, the NG-C interface is defined as the interface: - Between the NR Femto GW and the Core Network; - Between the NR Femto and the NR Femto GW; - Between the NR Femto and the Core Network; The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto as an AMF. The NG interface between the NR Femto and the 5GC is the same regardless whether the NR Femto is connected to the 5GC via an NR Femto GW or not. The functions supported by the NR Femto shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto connects via the NR Femto GW) and the procedures run between an NR Femto and the 5GC shall be the same as those between a gNB and the 5GC. Xn-connectivity is supported between NR Femtos and between NR Femtos and gNBs, independent of whether any of the involved NR Femtos is connected to an NR Femto GW. <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

With this BLCR, we still have some architectural features and functionalities hosted by NR Femto and GW not discussed. Some are listed as below.

- Discussion

2.1 Architecture

As justification of the WID (see [2]), one of the motivations is to enable simple end user plug and play. To achieve this target, it is necessary that the NR Femto node should be simple and small.

Meanwhile we have agreed in [1] that an NR Femto GW allows the concentration of the NG-C interface between the NR Femto and the 5GC. From this perspective, it will introduce a tremendous number of NG-C connections to the NR Femto GW from NR Femto nodes and the support of NG-Flex for NR Femto node will cause even more NG connections.

Therefore, considering reasonable simplifications for the NR Femto node in a network-operation and configuration view, and targeting at a low-complexity implementation of NR Femto GW, the following proposal is envisaged:

Proposal 1: The NR Femto shall only connect to a single NR Femto GW at one time when the NR Femto connects via the NR Femto GW.

In order to enable NG-Flex connectivity for AMF load balancing, NR Femto GW should support connecting to multiple candidate AMF(s) at one time, namely NG-Flex fucntion shall be used at the NR Femto GW instead of NR Femto nodes.

Proposal 2: The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

Regarding the aforementioned proposals that pertain to architectural characteristics, it is necessary to clarify the characteristics of the connections within the Architecture chapter.

Proposal 3: We propose to incorporate the aforementioned two proposals into the Architecture chapter.

2.1 Functionalities with NR Femto GW

As the NR Femto GW supports to simultaneously connect with multiple AMFs, the proper AMF selection function, i.e.  the NAS node selection function (NNSF), should be supported by the NR Femto GW. Meanwhile, the NR Femto node is recommended only connecting to a single NR Femto GW at one time, that is the NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF, so it is not necessary for the NR Femto node to host the NNSF. Therefore, the NNSF should be located in the NR Femto GW for the UE instead of NR Femto node.

Proposal 4: The NR Femto GW shall host the NNSF function instead of the NR Femto nodes.

When a UE register itself to the network, the NG-RAN shall use the NAS Node Selection Function (if supported) to select a proper AMF for the UE as specified in the 3GPP TS 38.410 [3],

A NAS node selection function is located in the NG-RAN node to determine the AMF association of the UE, based on the UE's temporary identifier, which was assigned to the UE by the AMF. When the UE's temporary identifier has not been yet assigned or is no longer valid the NG-RAN node may instead take into account other information (e.g. slicing information, onboarding indication) to determine the AMF.

As analysis above, the NNSF shall be host by the NR Femto GW if present instead of NR Femto nodes. Therefore, the assistance information used assisting NR Femto GW to select a proper AMF for the UE, (e.g. UE's temporary identifier,GUAMI, slicing information, onboarding indication), shall be transferred from the NR Femto nodes to the NR Femto GW if present.

Proposal 5: The assistance information for NNSF if present should be transferred over NG interface from NR Femto node to NR Femto GW.

- Proposals

In this contribution, we provide our considerations on NR Femto Architecture, as well as NG connection and interface with NR Femto GW, and the following proposals were drawn from the analysis.

Proposal 1: The NR Femto shall only connect to a single NR Femto GW at one time when the NR Femto connects via the NR Femto GW.

Proposal 2: The NR Femto GW supports NG-Flex configuration and can simultaneously connect to multiple AMFs.

Proposal 3: We propose to incorporate the aforementioned two proposals into the Architecture chapter.

Proposal 4: The NR Femto GW shall host the NNSF function instead of the NR Femto nodes.

Proposal 5: The assistance information for NNSF if present should be transferred over NG interface from NR Femto node to NR Femto GW.

- Reference

- R3-245754

- RP-242395 New WID on additional topological enhancements for NR

- 38.410 NG-RAN; Architecture description (Release 18)

- TR 38.799 v19.0.0 Study on additional topological enhancements for NR (Release 19)