3GPP TSG-RAN WG3 Meeting #126				  	                  R3-247658

Orlando, USA, 18th - 22th Nov 2024

Agenda Item:	31.2

Source:	ZTE Corporation, China Telecom, China Unicom, CMCC

Title: 		Enhancing QoS Monitoring with Per-Packet Granularity

Document for:	Discussion and Approval

1	Introduction

In RAN3#125bis, based on our previous contribution, companies have discussed the QoS monitoring with per-packet granularity and make no consensus. In this meeting, we update extra explanation for the comments received from companies so far. In addition. we further discuss the necessity of supporting RAN part delay feedback with per-packet granularity and the updated potential solution. Based on this updated solution, the corresponding CR for TS38.415 for Release 19 are provided in [1], and a LS to RAN2 is provided in Annex.

- Discussion

2.1 Necessity

QoS monitoring, including UL/DL E2E delay monitoring, is supported in the current 5GS as specified in TS 23.501. This monitoring is requested per QoS flow towards the NG-RAN via NGAP signaling. According to Current TS 38.415, the RAN can only report the average delay of UL/DL packets per QoS Flow to the UPF via NG-U. However, E2E delay monitoring with per-packet granularity is not supported.

Observation 1: Current QoS monitoring metrics do not support E2E delay monitoring with per-packet granularity.

There are some drawbacks to monitoring only the average delay in the current 5G URLLC, such as the following:

- Inability to detect Individual anomalies: Monitoring only the average delay provides an overall view of the latency but fails to capture anomalies in individual data packets. Significant delays in specific packets are overlooked by the average value, making them harder to detect.

- Impact on service quality: Latency-sensitive applications, such as industrial control systems, UAV control, and autonomous driving, can be severely affected by high delays in individual packets, thereby degrading user experience and causing serious problems in some cases.

- Challenges in optimization: The average delay metric does not offer detailed information on the distribution of delays, making it difficult for operators to perform precise optimizations to enhance QoS performance.

Observation 2:  While monitoring average delay only provide an overall view of latency, it overlooks anomalies in individual data packets that can degrade user experience and cause serious problems in some cases.

In the current TS 28.552, the OAM measurement for the "Distribution of UL delay packets between PSA-UPF and UE (including/excluding D1)" is specified in sections 5.4.9.2.2 and 5.4.9.2.4, while the "Distribution of DL packet delay between PSA-UPF and UE" is specified in section 5.4.9.1.2. To accurately calculate these packet delay distributions, each sample packet delay must be measured. For example, as following highlighted in the DL delay distribution definition, the DL delay distribution can only be calculated if the DL delay result from the NG-RAN is provided for each requested sample packet.

5.4.9.1.2 Distribution of DL packet delay between PSA UPF and UE a) This measurement provides the distribution of DL packet delay between PSA UPF and UE. This measurement is split into subcounters per S-NSSAI. This measurement is only applicable to the case the PSA UPF and NG-RAN are time synchronised. b) DER (n=1). c) The measurement is obtained by the following method: The UPF performs QoS monitoring per the request received from SMF during PDU Session Establishment or Modification procedure. NOTE: The UPF may sample the GTP packets for QoS monitoring the specific sampling rate is up to implementation. For each received DL GTP PDU monitoring response packet (packet i) for QoS monitoring, the PSA UPF records the following time stamps and information included in the GTP-U header (see 23.501 [4] and 38.415 [31]): - T1 indicating the local time the DL GTP PDU monitoring packet was sent by the PSA UPF; - T2 indicating the local time that the DL GTP PDU monitoring packet was received by NG-RAN; - The DL Delay Result from NG-RAN to UE indicating the downlink delay measurement result which is the sum of the delay incurred in NG-RAN (including the delay at gNB-CU-UP, on F1-U and on gNB-DU) and the delay over Uu interface (see 38.415 [31],and the DL Delay Result is denoted by in the present document); - The S-NSSAI associated to the DL GTP PDU monitoring response packet. The PSA UPF 1) takes the following calculation for each GTP PDU monitoring response packet for each S-NSSAI, and 2) increment the corresponding bin with the delay range where the result of 1) falls into by 1 for the subcounter per S-NSSAI.

Currently, the partial UL/DL delay between the UPF and NG-RAN can be calculated on a per-packet basis using timestamps in NG-U packets. However, due to the RAN's inability to report RAN part delays with per-packet granularity, these existing OAM defined UL/DL delay distributions are not supported.

Observation 3: Due to the RAN's inability to report RAN part delays with per-packet granularity, the existing OAM measurements specified in the current TS 28.552 are not supported.

Furthermore, the E2E delay monitoring with per-packet granularity is already captured as a 5G requirement in 6.23.2 in TS 22.261, as outlined below:

Based on operator request, for direct network connection scenarios in non-public networks, the 5G system shall be able to activate/deactivate efficient QoS monitoring with a finer granularity (e.g. per data packet) in a specific QoS flow (e.g. supporting URLLC services) to report on data packets not meeting the required QoS level. NOTE 11: The QoS parameters to be monitored and reported can include latency (e.g. UL or DL). NOTE 12: The above requirement does not assume UE impacts.

Observation 4: The 5G system shall support efficient QoS monitoring with a finer granularity (e.g. per data packet). This requirement has already been captured in TS 22.261.

Based on the above discussion and observations, in order to support the existing measurements of the Distribution of UL/DL E2E delay of packets defined in TS 28.552, and meet the requirements for efficient QoS monitoring with per-packet granularity, as specified in TS 22.261, we propose the following proposals.

Proposal 1: In order to support the existing measurements of the Distribution of UL/DL E2E delay of packets defined in TS 28.552, it is proposed to enhance QoS monitoring by enabling RAN UL/DL delay feedback at per-packet basis. This enhancement will also meet the requirements for efficient QoS monitoring with per-packet granularity, as specified in TS 22.261.

2.2 Solution

2.2.1 RAN DL delay feedback with per-packet granularity

The solution for the RAN DL delay feedback with per-packet granularity is provided as the below.

Fig 1: RAN DL delay feedback with per-packet granularity

Since the RAN needs to measure EVERY downlink packet delay for involved QoS flow to calculate and report the average RAN DL delay based on legacy QoS monitoring, it is feasible for the RAN to directly report the RAN DL delay to the UPF on a per-packet basis.

For each downlink QoS Monitoring Packet (QMP), as shown in Step 5 of Figure 1, the RAN reports the RAN DL delay for the QMP packet using the existing UL NG-U frame. We only need to clarify that if the gNB supports per-packet delay feedback, the existing DL Delay Result IE in the UL NG-U frame may be reused for per-packet(per-QMP) DL delay feedback.

Proposal 2.1: RAN3 agrees to support RAN DL delay feedback with per-packet granularity. .

More specifically, to minimize the spec impact, RAN may report the downlink delay for each downlink QoS monitoring packet(QMP) by using existing DL Delay Result IE for per-packet granularity.

Proposal 2.2: It is proposed for RAN3 to add clarification that the existing DL Delay Result IE in the UL PDU SESSION INFORMATION may be reused for packet-level feedback.

2.2.2 RAN UL delay feedback with per-packet granularity (excluding D1):

D1 delay is UL PDCP packet delay occurred in the UE. Since not all UEs support reporting D1 delay to the RAN, and the distribution of UL delay packets between PSA-UPF and UE (excluding D1) is also defined in TS 28.522, the RAN should at least support UL delay feedback with per-packet granularity, excluding D1.

Proposal 3.1: RAN3 agrees to support RAN UL delay feedback with per-packet granularity, excluding D1.

The proposed solution for the RAN UL delay feedback with per-packet granularity (excluding D1) is provided as the below.

Fig 2: RAN UL delay feedback with per-packet granularity (excluding D1)

Similar to RAN DL average delay feedback, since the RAN needs to measure EVERY uplink packet delay (without D1) for involved QoS flow to calculate and report the average RAN UL delay based on legacy QoS monitoring, it is feasible for the RAN to directly report the RAN UL delay(without D1) to the UPF on a per-packet basis.

That is, the gNB may sample the UL packets for QoS monitoring, and the sampling rate is up to implementation. For each UL sample packet, the existing UL Delay Result IE in the UL PDU SESSION INFORMATION will be reused for packet-level feedback.

However, the presence of UL Sending Time Stamp IE is tied to the DL QoS monitoring packet, as shown highlight below, this IE is not always present for uplink packet. In order to allow the gNB to record the sending time for every UL sample packet to enable the UPF to calculate  the uplink delay between UPF and RAN, it is necessary to introduce a new IE, e.g., Enhanced ULSending Time Stamp . Specifically, the gNB may sample the uplink packets for QoS monitoring and report the uplink delay (without D1) for each sample packet.

5.5.3.8 QoS Monitoring Packet (QMP) Description: This parameter indicates that the transferred packet is used for QoS monitoring as described in clause 5.4.1.1 and clause 5.4.2.1. This parameter also indicates the presence of the DL Sending Time Stamp in the DL PDU Session Information frame and the presence of the DL Sending Time Stamp Repeated, the DL Receiving Time Stamp, the UL Sending Time Stamp in the UL PDU Session Information frame. If QoS monitoring has not been configured for the involved QoS flow, the QMP shall be ignored by the NG-RAN node. Value range: {0= not used for QoS monitoring, 1= used for QoS monitoring}. Field length: 1 bit.

Proposal 3.2:  Add clarification that the existing UL Delay Result IE in the UL PDU SESSION INFORMATION may be reused for packet-level feedback.

Proposal 3.3: Introduce a new IE, e.g., Enhanced ULSending Time Stamp IE to enable the UPF to calculate the uplink delay of UL sample packet between UPF and RAN.

2.2.3 RAN UL delay feedback with per-packet granularity (including D1):

The proposed solution for the RAN UL delay feedback with per-packet granularity (including D1) is provided as the below.

Fig 3: RAN UL delay feedback with per-packet granularity (including D1)

As shown in Step 3,and 5 of Figure 3, to support RAN UL delay feedback with per-packet granularity (including D1) while minimizing the impact on the UE, if the UE can report the average D1 with a shorter measurement/reporting period, the RAN can sum the latest average D1 reported by the UE and the calculated UL delay  to obtain the final UL delay (including D1) for the UL sample packet.

We also suggest sending a LS to RAN2 to ask about the feasibility of allowing UE to report D1 with a shorter measurement/reporting period. (Note: If more accurate results are needed, RAN3/RAN2 can consider further solutions in the future).

Observation 5: In order to support RAN UL delay feedback with per-packet granularity (including D1) while minimizing the impact on the UE, if the UE can report the average D1 with a shorter measurement/reporting period, the RAN can sum the latest average D1 reported by the UE and the calculated UL delay to obtain the final UL delay (including D1) for the UL sample packet.

Proposal 4: Send a LS to RAN2 to ask about the feasibility of allowing UE to report D1 with a shorter measurement/reporting period. The LS in Annex can be considered as baseline.

- Conclusion

The observations and proposals in this discussion paper is summarized below:

Observation 1: Current QoS monitoring metrics do not support E2E delay monitoring  with per-packet granularity.

Observation 2:  While monitoring average delay only provide a overall view of latency, it overlooks anomalies in individual data packets that can degrade user experience and cause serious problems in some cases.

Observation 3: Due to the RAN's inability to report RAN part delays with per-packet granularity, the existing OAM measurements specified in the current TS 28.552 are not supported.

Observation 4: The 5G system shall support efficient QoS monitoring with a finer granularity (e.g. per data packet). This requirement has already been captured in TS 22.261.

Proposal 1: In order to support the existing measurements of the Distribution of UL/DL E2E delay of packets defined in TS 28.552, it is proposed to enhance QoS monitoring by enabling RAN UL/DL delay feedback at per-packet basis. This enhancement will also meet the requirements for efficient QoS monitoring with per-packet granularity, as specified in TS 22.261.

Proposal 2.1: RAN3 agrees to support RAN DL delay feedback with per-packet granularity. .

Proposal 2.2: It is proposed for RAN3 to add clarification that the existing DL Delay Result IE in the UL PDU SESSION INFORMATION may be reused for packet-level feedback.

Proposal 3.1: RAN3 agrees to support RAN UL delay feedback with per-packet granularity,excluding D1.

Proposal 3.2:  Add clarification that  the existing UL Delay Result IE in the UL PDU SESSION INFORMATION may be reused for packet-level feedback.

Proposal 3.3: Introduce a new IE, e.g., Enhanced ULSending Time Stamp IE to enable the UPF to calculate  the uplink delay of UL sample packet between UPF and RAN.

Observation 5: In order to support RAN UL delay feedback with per-packet granularity (including D1) while minimizing the impact on the UE, if the UE can report the average D1 with a shorter measurement/reporting period, the RAN can sum the latest average D1 reported by the UE and the calculated UL delay to obtain the final UL delay (including D1) for the UL sample packet.

Proposal 4: Send a LS to RAN2 to ask about the feasibility of allowing UE to report D1 with a shorter measurement/reporting period. The LS in Annex can be considered as baseline.

- Reference

[1] R3-247657, Correction on QoS Monitoring with Per-Packet Granularity for TS38.415 (WID TEI19, Rel-19), ZTE Corporation

- Annex:  LS to RAN2

Title:	[draft] LS on QoS Monitoring with Per-Packet Granularity

Response to:

Release:	Rel-19

Work Item:	TEI 19

Source:	ZTE[shall be RAN3]

To:	RAN2

Cc:	SA1, SA2, SA5

Contact person:	Yansheng Liu

liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	[TBD]

1	Overall description

In the current TS 28.552, the OAM measurement for the "Distribution of UL delay packets between PSA-UPF and UE (including/excluding D1)" is specified in sections 5.4.9.2.2 and 5.4.9.2.4, while the "Distribution of DL packet delay between PSA-UPF and UE" is specified in section 5.4.9.1.2. To accurately calculate these packet delay distributions, each sample packet delay must be measured. Furthermore, the E2E delay monitoring with per-packet granularity is already captured as a 5G requirement in 6.23.2 in TS 22.261.

To meet the existing requirements,RAN3 is discussing how to improve the granularity of delay measurements(e.g., per-packet granularity), including:

- RAN DL delay feedback with per-packet granularity

- RAN UL delay feedback with per-packet granularity (excluding D1).

For this reason RAN3 would like to ask RAN2 whether it is feasible to improve the granularity with which UL PDCP delay measurements (i.e. D1 delay) can be measured. The current averaging window for D1 delay measurements is a minimum of 120ms, which deeply affects the granularity of RAN level delay monitoring.

Question: RAN3 want to inquire RAN2 about the feasibility of allowing UE to report D1 with a shorter measurement/reporting period.

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide feedback on the question above.

3	Dates of next RAN3 meetings

RAN3#127			17th - 21th Feb 2025 		Athens, GR