3GPP TSG-RAN WG2 Meeting #128																R3-247662

Orlando, USA, Nov. 18th – 22nd, 2024

Source			: 	      CEWiT

Agenda item	:    	20.2

Title 		:       Discussion on AI/ML-based Positioning Accuracy Enhancements

Document for: 	    Discussion and Decision

# 1.	Introduction

The Work Item on artificial intelligence/machine learning for NR air interface has the following objectives on AI/ML positioning [1]:

Provide specification support for the following aspects: - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

The positioning cases 3a and 3b with potential RAN3 specification impact are discussed in this document.

For the network-based positioning (Case 3a/3b), RAN3 made the following general agreements:

RAN3#125bis

The existing LCS framework is re-used for AI/ML positioning.

Case 3b:

Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1.

NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF.

The details of the measurements are still pending and subject to further discussion in RAN1.

Case 3a:

The AI/ML model (inference and training functions) is at the gNB.

LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN.

FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design.

How the LMF obtains the ground truth label and related data information is outside of RAN3 scope.

NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1).

The details of the measurements are still pending and subject to further discussion in RAN1.

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

# 2.	Ground Truth Label

The model training and model monitoring at the gNB-side model (Case 3a) require the ground truth label information for the UE(s) in the area of interest. Based on the RAN3 agreements, the general procedure for obtaining the ground truth label information is shown in Figure 1.

Figure 1: gNB Obtaining the Ground Truth Label from LMF

- A trigger for AMF to initiate the 5GC-NI-LR procedure happens from the serving gNB.

- The AMF selects an LMF based on NRF query or configuration in AMF and invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE as described in [2].

- The LMF performs one or more of the positioning procedures as described in [2]. If positioning estimate is available with LMF then step 3 is skipped e.g. PRU/Located UE.

- The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE as described in [2].

- The AMF sends the UE location information to the serving gNB.

NOTE: Step 5 is out of RAN3 scope.

- The serving gNB may need to extract the ground truth label information from the location information.

Proposal 1: Agree on the general procedure in Figure 1 for ground truth label signalling from the LMF.

# 3.	Model Monitoring

For gNB-side model (Case 3a), the gNB shall perform the model monitoring. Since the model is maintained at the gNB end, performance monitoring is preferred to be done at the gNB for minimal specification impact and reduced overhead signalling.

For LMF-side models (Case 3b), the LMF/MTLF is performing the model monitoring. No specification impact to RAN3 is expected.

Proposal 2:	For Case 3a, the gNB shall perform the model monitoring.

Proposal 3:	For Case 3b, RAN3 shall have no specification impact on the model monitoring aspect.

# 4.	Model Inference

For Case 3a, the model inference output is the AI/ML estimated measurements that are shared with the LMF for computing the UE location based on the legacy positioning methods. For Case 3b, the gNB provides the measurements to the LMF, using which the LMF runs the AI/ML model to compute the UE location. Hence, the existing positioning procedures can be used for both Case 3a and Case 3b.

Proposal 4:	For Case 3a/3b model inference, existing positioning procedures can be used.

# 5.	Conclusion

The following proposals are made in this document.

Proposal 1: Agree on the general procedure in Figure 1 for ground truth label signalling from the LMF.

Proposal 2: For Case 3a, the gNB shall perform the model monitoring.

Proposal 3: For Case 3b, RAN3 shall have no specification impact on the model monitoring aspect.

Proposal 4: For Case 3a/3b model inference, existing positioning procedures can be used.

# References

[1]	RP-242399, "Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface".

[2]	3GPP TS 23.273, "5G System (5GS) Location Services (LCS); Stage 2”.