3GPP TSG-RAN WG3 Meeting #126														R3-247679

Orlando, US, Nov 18th - Nov 22th, 2024

Agenda item:			11.3

Source:					CMCC

Title:						Discussion on AI/ML-based CCO

Document Type:		Discussion

Introduction

During RAN3#125bis, the following agreements have been captured for AI/ML enabled Coverage and Capacity Optimization.

Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message.

The Coverage Modification Cause reflects the predicted CCO issue.

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

Information to validate the prediction is needed.

In this paper, we provide our consideration of AI/ML based Coverage and Capacity Optimization.

Discussion

Xn interface enhancement

In RAN3#125bis, RAN3 has agreed to reuse NG-RAN NODE CONFIGURATION UPDATE message to transfer future CCO state and coverage modification cause to neighbour nodes, but how to implement needs further discussion.

Considering that the NG-RAN node can inform its neighbor nodes the cell/beam coverage configuration before and after the cell coverage is modified by using the Coverage Modification List IE in the current spec, we think the future CCO state exchanged between NG-RAN node is similar with the NG-RAN node inform its neighbor nodes the cell coverage state planned to be used at the next reconfiguration, the difference is that the future CCO state can be generated with AI/ML and it’s for the future use. Thus, two options for implementation:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

Observation 1: Two options for delivering future CCO state and coverage modification cause to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

From our perspective, both options can be discussed. If opt 2 is selected, the new IE can be used to only transmit AI/ML related predicted/future information, which can be distinguished from the existing IE to avoid neighbor node confusing about the received information is legacy CCO state or future CCO state.

For Opt 1, if reusing existing CCO cause and CCO state IE, an indication needs to be introduced to distinguish whether the CCO state is legacy or for future use.

In RAN3#125bis, RAN3 agreed to introduce separate time information for future coverage state and predicted CCO issue. We think the time information can be used to distinguish legacy CCO state from future CCO state. For example, when the CCO state is for legacy use, the time information is not included. When the CCO state is predicted or for future use, the future CCO state exchanges between nodes together with the time information. With this time information, Opt 1 can be selected, which also has the minimum standard impact as only the time information needs to be added. However, if opt 1 is agreed, the time information must be present if the CCO state is predicted or for future use. Otherwise, there is no difference between the legacy and AI/ML-based solution. Therefore, for Opt 1, RAN3 needs to specify that the time information must be present for further CCO state.

Observation 2: Opt 1 has minimum standard impacts if the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause. But RAN3 needs to specify that the time information must be present for further CCO state.

Proposal 1: If opt 1 is agreed, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 2: RAN3 agrees that the time information must be present if the CCO state is predicted or for future use.

F1 interface enhancement

For the split architecture, RAN3#125bis has the following WA:

WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1.

We think that RAN3 can follow the Xn enhancement principle that the GNB-CU CONFIGURATION UPDATE message can be reused to transfer the predicted CCO issue from gNB-CU to gNB-DU, and the GNB-DU CONFIGURATION UPDATE message can be reused to transfer the future CCO state from gNB-DU to gNB-CU.

Proposal 3: Turn the “WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1” into agreement.

Then, for how to introduce the predicted CCO issue and future CCO state together with the time information. We think it’s similar to the Xn enhancement that the time information is used to distinguish legacy CCO issue/CCO state from AI/ML-based predicted CCO issue and future CCO state. So, for F1 interface, we also suggested to reuse the current IE and introduce time information for predicted CCO issue and future CCO state.

Proposal 4: For F1 interface, same as Xn enhancement, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 5: RAN3 agrees that the time information must be present if the CCO state is predicted or for future use in F1 interface.

Time information

RAN3#125bis has agreed to introduce separate time information for future coverage state and predicted CCO issue. For the details of time information, from our perspective, it’s no need to include two kinds of time information in one message at the same time. For non-split architecture, it’s better for the neighbour nodes aware when the coverage configuration of local node is predicted to be modified, so that the neighbour nodes can make corresponding preparations in advance and adjust its own coverage configuration accordingly. So, for the Xn enhancement, we propose to introduce the time information that indicate the time when the coverage configuration of local node is predicted to be modified.

Observation 3: For non-split architecture, the time information of future CCO state is beneficial for the neighbour nodes aware when the local node coverage configuration will be modified. The neighbour nodes can make some preparations in advance and adjust its own coverage configuration accordingly.

Proposal 6: The time information transferred via Xn interface indicates when the local node coverage configuration will be modified.

For split architecture, considering that gNB-CU sends the predicted CCO issue to the gNB-DU, gNB-CU can indicate when the CCO issue is predicted to be happened. The gNB-DU provides the future CCO state to gNB-CU, gNB-DU can also indicate the time information of when the future CCO state will be modified. Thus, for split architecture, the time information of predicted CCO issue is transferred from gNB-CU to gNB-DU, and the time information of future CCO state is transferred from gNB-DU to gNB-CU.

Proposal 7: For split architecture, the time information of predicted CCO issue is sent from gNB-CU to gNB-DU.

Proposal 8: For split architecture, the time information of future CCO state is sent from gNB-DU to gNB-CU.

Feedback information

In the offline discussion of RAN3#125bis, RAN3 has discussed whether UE performance or measurements are needed to be collected as feedback. And the following FFS has been captured.

Information to validate the prediction is needed.

For the prediction information, except the predicted CCO issue, we think the future CCO state also needs to be considered as it can be predicted via AI/ML.

Observation 4: As the future CCO state can be generated via AI/ML, it also needs to be validate after the coverage modification.

When validate the future CCO state information, the specific UE(s)’ performance needs to be collected to see whether the UE experience severe performance degradation after coverage modification. This can be used to check the reasonability of the future CCO state. For example, as the UE(s) may locate in the gNB1 before coverage configuration changes, and locate in gNB2 after coverage configuration changes, if the UE experience severe performance degradation after coverage modification, it seems that the generated future CCO state is not very reasonable. If the future CCO state is inferred based on the AI/ML in the source node, the AI/ML model needs to be updated. Also, if the target node, i.e. the serving gNB, aware the performance degradation, it can also perform actions to reduce the impact, e.g. provide more resources for the UE.

In R18, the legacy UE performance feedback including average UE throughput UL/DL, average packet delay and average packet loss. For the coverage edge UE(s), i.e. the UE(s) serving gNB is changed after CCO, the UE performance collected before and after CCO can reuse the legacy UE performance feedback information.

Proposal 9: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

When the coverage configuration of local node changes, the coverage of neighbor nodes also need to be changed. Whether the coverage configuration modification caused by the local node results in severe performance degradation of neighbor node needs to be considered. And for the feedback information, except the UE performance, we also propose to collect the NG-RAN node performance. For example, after the coverage configuration modification, the number of RRC connections of neighbor node increases, which may cause the neighbor node overload, or the PRB usage decrease, which waste the neighbor node resources. The neighbor nodes need to inform the local node these information and the local node can update the AI/ML model with these information.

Also, we think the NG-RAN node performance can be used to validate whether the predicted CCO issue is resolved after the CCO, and whether new issue is predicted after CCO.

Proposal 10: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

Conclusion

In this contribution, we propose the following proposals:

Observation 1: Two options for delivering future CCO state and coverage modification cause to neighbor NG-RAN nodes:

Opt 1: Reusing the current defined CCO state and modification cause IE for future CCO state and coverage modification cause.

Opt 2: Introduce a new IE for AI/ML related information transmission, including future CCO state and coverage modification cause.

Observation 2: Opt 1 has minimum standard impacts if the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause. But RAN3 needs to specify that the time information must be present for further CCO state.

Proposal 1: If opt 1 is agreed, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 2: RAN3 agrees that the time information must be present if the CCO state is predicted or for future use.

Proposal 3: Turn the “WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1” into agreement.

Proposal 4: For F1 interface, same as Xn enhancement, the time information can be used to distinguish the legacy CCO state/cause from future CCO state and coverage modification cause.

Proposal 5: RAN3 agrees that the time information must be present if the CCO state is predicted or for future use in F1 interface.

Observation 3: For non-split architecture, the time information of future CCO state is beneficial for the neighbour nodes aware when the local node coverage configuration will be modified. The neighbour nodes can make some preparations in advance and adjust its own coverage configuration accordingly.

Proposal 6: The time information transferred via Xn interface indicates when the local node coverage configuration will be modified.

Proposal 7: For split architecture, the time information of predicted CCO issue is sent from gNB-CU to gNB-DU.

Proposal 8: For split architecture, the time information of future CCO state is sent from gNB-DU to gNB-CU.

Observation 4: As the future CCO state can be generated via AI/ML, it also needs to be validate after the coverage modification.

Proposal 9: It’s proposed to collect the performance of coverage edge UE(s) (e.g. the UE(s) serving gNB is changed after CCO) as feedback information.

Proposal 10: It’s proposed to collect the performance of neighbor node(s) as feedback information, such as the number of RRC connections and PRB usage before and after coverage configuration modification.

- References

[1] RP-242385, “New WID: Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, ZTE, NEC

- Annex: TP for 38.423

    - 9.1.3.4	NG-RAN NODE CONFIGURATION UPDATE

This message is sent by a NG-RAN node to a neighbouring NG-RAN node to transfer updated information for an Xn-C interface instance.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| TAI Support List | O |  | 9.2.3.20 | List of supported TAs and associated characteristics. | GLOBAL | reject |
| CHOICE Initiating NodeType | M |  |  |  | YES | ignore |
| >gNB |  |  |  |  |  |  |
| >>Served Cells To Update NR | O |  | 9.2.2.15 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| >>Served Cell Specific Info Request | O |  | 9.2.2.102 |  | YES | ignore |
| >ng-eNB |  |  |  |  |  |  |
| >>Served Cells to Update E-UTRA | O |  | 9.2.2.16 |  | YES | ignore |
| >>Cell Assistance Information NR | O |  | 9.2.2.17 |  | YES | ignore |
| >>Cell Assistance Information E-UTRA | O |  | 9.2.2.43 |  | YES | ignore |
| TNLA To Add List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Add Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | M |  | 9.2.3.84 |  | – |  |
| TNLA To Update List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Update Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| >> TNL Association Usage | O |  | 9.2.3.84 |  | – |  |
| TNLA To Remove List |  | 0..1 |  |  | YES | ignore |
| >TNLA To Remove Item |  | 1..<maxnoofTNLAssociations> |  |  | – |  |
| >>TNLA Transport Layer Information | M |  | CP Transport Layer Information 9.2.3.31 | CP Transport Layer Information of NG-RAN node1 | – |  |
| Global NG-RAN Node ID | O |  | 9.2.2.3 |  | YES | reject |
| AMF Region Information To Add | O |  | AMF Region Information 9.2.3.83 | List of all added AMF Regions to which the NG-RAN node belongs. | YES | reject |
| AMF Region Information To Delete | O |  | AMF Region Information 9.2.3.83 | List of all deleted AMF Regions to which the NG-RAN node belongs. | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |
| TNL Configuration Info | O |  | 9.2.3.96 |  | YES | ignore |
| Coverage Modification List |  | 0 .. 1 |  | List of cells with modified coverage. | GLOBAL | reject |
| >Coverage Modification Item |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>Global NG-RAN Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of the cell to be modified. | – |  |
| >>Cell Coverage State | M |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | – |  |
| >>Cell Deployment Status Indicator | O |  | ENUMERATED(pre-change-notification, ...) | Indicates the Cell Coverage State is planned to be used at the next reconfiguration. | – |  |
| >>Predicted Coverage Modification Time | M |  | FFS | Indicates the time in point of future coverage state to be applied |  |  |
| >>Cell Replacing Info | C-ifCellDeploymentStatusIndicatorPresent |  |  |  | – |  |
| >>>Replacing Cells |  | 0 .. <maxnoofCellsinNG-RAN node> |  |  | – |  |
| >>>>Global NG-RAN Cell Identity |  |  | Global NG-RAN Cell Identity 9.2.2.27 | NG-RAN Cell Global Identifier of a cell that may replace all or part of the coverage of the cell to be modified. | – |  |
| >>SSB Coverage Modification List |  | 0.. 1 |  | List of SSB beams with modified coverage. | – |  |
| >>>SSB Coverage Modification Item |  | 0..<maxnoofSSBAreas> |  |  | – |  |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the SSB beam to be modified. | – |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SSB beam is inactive. Other values Indicates that the SSB beam is active and also indicates the coverage configuration of the concerned SSB beam. | – |  |
| >>Coverage Modification Cause | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the reason for the coverage modification in NG-RAN node1. | YES | ignore |
| Local NG-RAN Node Identifier | O |  | 9.2.2.101 |  | YES | ignore |
| Neighbour NG-RAN Node List |  | 0..<maxnoofNeighbourNG-RAN nodes> |  |  | YES | ignore |
| >Global NG-RAN Node ID | M |  | 9.2.2.3 |  | – |  |
| >Local NG-RAN Node Identifier | M |  | 9.2.2.101 |  | – |  |
| Local NG-RAN Node Identifier Removal | O |  | Local NG-RAN Node Identifier 9.2.2.101 |  | YES | ignore |

- Annex: TP for 38.473

    - 9.3.1.211	CCO Assistance Information

This IE provides assistance information for the Capacity and Coverage (CCO) actions for specific CCO issues detected, and for network energy saving.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CCO issue detection | O |  | ENUMERATED (coverage, cell edge capacity, ..., network energy saving) | Indicates the type of CCO issue detected, or network energy saving cause. |
| Affected Cells and Beams | O |  | 9.3.1.212 |  |
| CCO issue prediction time | FFS |  | FFS | Indicates the time in point of when CCO issue predicted to be happened. |

    - 9.3.1.213	Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding coverage configuration selected by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Coverage Modification List |  | 1 |  |  |  |  |
| >Coverage Modification Item |  | 1 .. <maxCellingNBDU> |  |  | - |  |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Coverage State |  |  | INTEGER (0..63, …) | Value '0' indicates that the cell is inactive. Other values Indicates that the cell is active and also indicates the coverage configuration of the concerned cell. | - |  |
| >>SSB Coverage Modification List |  | 0..1 |  |  | - |  |
| >>>SSB Coverage Modification Item |  | 1..<maxnoofSSBAreas> |  |  | - |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>SSB Coverage State | M |  | INTEGER (0..15, …) | Value '0' indicates that the SS/PBCH block is inactive. Other values Indicates that the SS/PBCH block is active and also indicates the coverage configuration of the concerned SS/PBCH block. | - |  |
| >>Coverage Modification Cause | O |  | ENUMERATED(coverage, cell edge capacity, …, network energy saving) |  | YES | ignore |
| >>Future Coverage Modification Time | M |  | FFS | Indicates the time in point of future coverage state to be applied | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a NG-RAN node cell. Value is 64. |