3GPP TSG-RAN WG3 #126								R3-247694

Orlando, USA, 18th – 22th Nov 2024

Agenda Item:	10.4

Source:	China Telecom

Title: 	Discussion on RACH optimization for SDT

Document for:	Discussion

Introduction

RAN3 had discussed R18 leftovers for SON/MDT during the past serval meetings. For RACH enhancement for SDT, the following conclusions and FFS were made in RAN3#125 meeting:

The information that stored and reported in the RA report for SDT:

-	c. the time from the start of RA-SDT to the reporting of RA report

FFS on other SDT related information.

And at RAN2#127bis meeting, the following agreements on RACH Optimization for SDT had been made:

- For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log.

- RAN2 understands for SON/MDT R19 the SDT enhancements only relate to the RA-SDT procedure. This does not rule out the case when the UE falls back from RA-SDT.

In this contribution we will focus on FFS on other SDT related information.

Discussion

According to RAN2 agreement, CG-SDT related enhancements include the case CG-SDT fallback to RA-SDT is out of scope of R18 leftover discussion.

Proposal 1: the case CG-SDT fallback to RA-SDT is out of scope of R18 leftover discussion

In Rel-17, supplementary uplink can be supported in RA-SDT, and UE can select either NUL and SUL based on rsrp-ThresholdSSB-SUL information broadcast in SIB1:

1>	if the RSRP of the downlink pathloss reference is higher than RSRP_THRESHOLD or if RSRP_THRESHOLD is not set:

2>	if the Serving Cell is configured with supplementary uplink as specified in TS 38.331 [5]; and

2>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:

3>	select the SUL carrier.

2>	else:

3>	select the NUL carrier.

Since NUL and SUL may be configured with separated PRACH resource and separated carrier, the UL carrier information is needed in RA report for optimization purpose.

Proposal 2: the UL carrier information is needed in RA report for optimization purpose.

In addition, the SDT procedure may be failure for the following cases:

unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or integrity check failure while SDT procedure is ongoing, or expiry of SDT-specific timing alignment timer or configuredGrantTimer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

Therefore, it is need to introduce a set of new failure cause in RA report,e.g., T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, TATimer-Expiry

Proposal 3: it is need to introduce a set of new failure cause in RA report,e.g., T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, TATimer-Expiry

Proposal

Based on the above analysis, we have the following proposals:

Proposal 1: the case CG-SDT fallback to RA-SDT is out of scope of R18 leftover discussion

Proposal 2: the UL carrier information is needed in RA report for optimization purpose.

Proposal 3: it is need to introduce a set of new failure cause in RA report,e.g., T319a-Expiry, randomAccessProblem, rlc-MaxNumRetx, TATimer-Expiry.

Reference

- Draft_RAN2_127_Meeting_Report_v1

- TS38.300-i30, Stage 2 specification for NR

- TS38.331-i30, NR RRC specification