3GPP TSG-RAN WG3 Meeting #126	R3-247727

Orlando, USA, 18 – 22 November 2024

Agenda Item:	16.4

Source: 				LG Electronics Inc.

Title:			Further discussion on locating A-IoT Device

Document for:		Discussion and Decision

1.	Introduction

New Rel-19 SID on solutions for Ambient IoT (Internet of Things) in NR was agreed at RAN #102 [1]. One of the objectives in this SID is to identify potential solutions for locating an Ambient IoT device with no specification impact as follows:

… - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. …

In this contribution, we will discuss the issue on locating an Ambient IoT device and provide our view on it.

2.	Discussion

In last meetings, the RAN3 captured the following agreements in the Chair’s minute.

… Use cases for locating an AIoT device: - Find an appropriate “reader” close to the A-IoT device; - Find where the A-IoT device is. Support locating the A-IoT device at “reader” granularity. AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. … For topology 1, AIoT RAN node ID can be considered as a location of AIoT device. For topology 2, UE ID of the AIoT-enabled UE can be considered as a location of AIoT device. … The following principle applies to Topology1 and Topology2: - For both purpose a) and b), identifier of the reader may be reported to AIoT CN in the Inventory Report message. - While for purpose b), whether the reader ID based granularity of the AIoT device location is sufficient can be further checked. …

In Topology 1, it is agreed that one or more Readers with its own service area can be co-located with the A-IoT RAN and locating the A-IoT device is supported at “Reader” granularity. Therefore, if the A-IoT signaling from the A-IoT Device is received via one or more of Reader(s), then the A-IoT RAN and the A-IoT CN can deduce/derive the location of the A-IoT Device considering on the service area of each Reader. For example, if the A-IoT Device is in the area where the service areas of several Readers overlap, the A-IoT CN can locate the A-IoT Device at the finer granularity. To do so, the A-IoT CN needs to know the Reader related information such as the Reader ID, its corresponding service area, and the ID of the serving A-IoT RAN. Also, the A-IoT CN can use this information to select/determine an appropriate “Reader” close to the A-IoT Device. In the NG interface, the NG-RAN and AMF exchange the application level data to correctly interoperate on the NG interface. Similarly, the A-IoT RAN needs to provide the Reader related information to the A-IoT CN during the setup procedure for the XX interface.

Proposal 1: In Topology 1, the A-IoT RAN provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAN) to the A-IoT CN during the setup procedure for the XX interface.

In Topology 2, from the network point of view, the A-IoT enabled UE is the normal UE with the capability to act as the Reader. The A-IoT enabled UE is registered with the network before serving the A-IoT device. As in Topology 1, the A-IoT CN needs to know the Reader related information such as the Reader ID, its corresponding service area, the serving UE ID and the identifier of the serving A-IoT enabled gNB. One possible option is that during the UE’s Registration procedure, the information related to the Reader can be also provided to the A-IoT CN. That is, the A-IoT enabled UE acting as the Reader sends to the AMF the Registration Request message containing the Reader related information, and then, the AMF forwards this information to the A-IoT capable NF. However, the A-IoT enabled gNB may need to know the Reader related information because it may be also able to determine/select the A-IoT enabled UE (acting as the Reader) close to the specific A-IoT Device(s). Therefore, the A-IoT enabled UE should provide the Reader related information to the AIoT-capable NF via the A-IoT enabled gNB.

Proposal 2: In Topology 2, the A-IoT enabled UE provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the identifier of the serving A-IoT enabled gNB) to the AIoT-capable NF via the A-IoT enabled gNB during the UE’s registration procedure.

3.	Conclusion

In this contribution, we focused on the issue on locating an Ambient IoT device and provided our view on it. The following proposals are kindly suggested to RAN3:

Proposal 1: In Topology 1, the A-IoT RAN provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the RAN ID of the serving AIoT RAN) to the A-IoT CN during the setup procedure for the XX interface.

Proposal 2: In Topology 2, the A-IoT enabled UE provides the Reader related information (e.g., the Reader ID, its corresponding service area, and the identifier of the serving A-IoT enabled gNB) to the AIoT-capable NF via the A-IoT enabled gNB during the UE’s registration procedure.

4.	Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR,” CMCC, Huawei, T-Mobile USA, March, 2024.