3GPP TSG-RAN WG3 Meeting #126	R3-247739

# Orlando, USA, 18– 22 Nov 2024

Agenda Item:	9.2

Source:	ZTE Corporation, CATT, China Telecom, China Unicom

Title:	Discussion on criticality of measurement result

Document for:	Discussions & Approval

- Introduction

For introduced DATA COLLECTION RESPONSE message in NR, we identified an issue that the criticality of Node Measurement Initiation Result List IE and Cell Measurement Initiation Result List IE are set to “reject”. And also we checked that the criticality of Measurement Initiation Result IE are set of “ignore”. Since these IEs are introduced to support partial reporting mechanism.

In this contribution, we provide analysis on how to resolve this misalignment between NR and LTE.

2	Discussion

During the Rel-18 WI phase, it was agreed that partial reporting is supported for AI/ML related information report. Therefore, the Node Measurement Initiation Result and Cell Measurement Initiation Result were introduced into the DATA COLLECTION RESPONSE message to indicate list of measurement objects that failed to be initiated.

After reviewing this addition, in the DATA COLLECTION RESPONSE message [1], we identified that the criticality of Node Measurement Initiation Result List IE and Cell Measurement Initiation Result List IE are set to “reject”.

============================TS 38.423============================

9.1.3.27	DATA COLLECTION RESPONSE

This message is sent by NG-RAN node2 to NG-RAN node1 to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NG-RAN node1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node1 | YES | reject |
| NG-RAN node2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by NG-RAN node2 | YES | reject |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Energy Cost, Second Bit = Average UE Throughput DL, Third Bit = Average UE Throughput UL, Fourth Bit = Average Packet Delay, Fifth Bit = Average Packet Loss DL, Sixth Bit = Measured UE Trajectory. Other bits are ignored by the NG-RAN node1. | – |  |
| >>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Cell Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated per cell. | YES | reject |
| >Cell Measurement Initiation Result Item |  | 1 .. <maxnoofCellsinNG-RANnode> |  |  | – |  |
| >>Cell ID | M |  | Global NG-RAN Cell Identity 9.2.2.27 | Indicates an NR Cell Identity. | – |  |
| >>Cell Measurement Failure Cause List |  | 0..1 |  | Indicates that NG-RAN node2 could not initiate the measurement for at least one of the requested measurement objects in the cell. | – |  |
| >>>Cell Measurement Failure Cause Item |  | 1 .. <maxFailedCellMeasObjects> |  |  | – |  |
| >>>>Cell Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the NG-RAN node2. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections. Other bits are ignored by the NG-RAN node1. | – |  |
| >>>>Cause | M |  | 9.2.3.2 | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |

============================TS 38.423============================

And then, we also checked RESOURCE STATUS RESPONSE message [2], which also supports partial reporting mechanism. It is found that the criticality of Measurement Initiation Result List IE is set of “ignore”.

============================TS 36.423============================

9.1.2.12	RESOURCE STATUS RESPONSE

This message is sent by the eNB2 to indicate that the requested measurement, for all or for a subset of the measurement objects included in the measurement is successfully initiated.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Criticality Diagnostics | O |  | 9.2.7 |  | YES | ignore |
| Measurement Initiation Result |  | 0..1 |  | List of all cells in which measurement objects were requested, included when indicating partial success | YES | ignore |
| >Measurement Initiation Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – |  |
| >>Measurement Failure Cause List |  | 0..1 |  | Indicates that eNB2 could not initiate the measurement for at least one of the requested measurement objects in the cell | – |  |
| >>>Measurement Failure Cause Item |  | 1 .. <maxFailedMeasObjects> |  |  | EACH | ignore |
| >>>>Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object that failed to be initiated in the eNB2. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, Fifth Bit = ABS Status Periodic, Sixth Bit = RSRP Measurement Report Periodic, Seventh Bit = CSI Report Periodic, Eighth Bit = CAC for Possibly Aggregated Cells Periodic. Other bits shall be ignored by the eNB1. | – |  |
| >>>>Cause | M |  | 9.2.6 | Failure cause for measurement objects for which the measurement cannot be initiated | – |  |

============================TS 36.423============================

In LTE and NR, the same feature is supported across different messages, but the introduced IEs (Measurement Result) in the response message have different criticality value. We consider this a misalignment that needs to be addressed.

- Misalignment on criticality of measurement result between NR and LTE needs to be addressed.

There are two solutions to address this misalignment:

-	Solution 1: Modify the assigned criticality of Node Measurement Initiation Result List IE and Cell Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE to "ignore" to align with LTE.

-	Solution 2: Modify the assigned criticality of Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE to "reject" to align with NR.

- RAN3 is kindly requested to determine which solution should be accepted to address this misalignment—either aligning with LTE or aligning with NR.

The defintion of “ignore” & “reject” is desribed below:

Reject IE:

- If a response message is received containing one or more IEs marked with "Reject IE", that the receiving node does not comprehend, the receiving node shall consider the procedure as unsuccessfully terminated and initiate local error handling.

Ignore IE:

- If a response message is received containing one or more IEs/IE groups marked with "Ignore IE" which the receiving node does not comprehend, the receiving node shall ignore the content of the not comprehended IEs/IE groups and continue with the procedure as if the not comprehended IEs/IE groups were not received using the understood IEs/IE groups.

It means that if this IE is marked as "ignore" in the response message, the receiving node shall disregard the content of this IE and continue with the procedure. Returning to our Data Collection procedure, if the Node initiation result IE is not comprehended, the question is whether the entire process should be terminated or not e.g. whether other information, such as the cell initiation result, could still be regarded as correct. Take an example below:

Scenario 1:

First, gNB 1 initiates the request message to gNB 2, requesting the predicted resource status (cell level), predicted RRC numbers (cell level) and measured energy cost (node level) in the same message.

Then, gNB 2 responds to the gNB 1 which type of requested measurement is successfully initiated, including predicted RRC numbers (cell level) and measured energy cost are failed. And then gNB 1 can not comprehend the Node Measurement Initiation Result List IE if it really happens.

Scenario 2:

First, gNB 1 initiates the request message to gNB 2, requesting the predicted resource status (cell level)).

Then, gNB 2 responds to the gNB 1 which type of requested measurement is successfully initiated, including measured energy cost are failed. Therefore, gNB 1 can not comprehend the Node Measurement Initiation Result List IE because gNB 1 did not request it.

For the above two scenarios, if the conclusion reached in RAN3 is that no need for gNB1 to terminate the entire process, assigned criticality of the Measurement Initiation Result IE should be ‘ignore’. In this case, update on 38.423 is needed. On the contrary, if RAN3 believes that there is a significant issue on the requesting node’s side that should result in the failure of the entire procedure, the assigned criticality of the Measurement Initiation Result IE in the RESOURCE STATUS RESPONSE should be set to 'reject' which means correction to 36.423 should be implemented.

- RAN3 agree CR for either 38.423 or 36.423 after conclusion is reached.

3	Conclusion

Following is our proposals and observations:

- Misalignment on criticality of measurement result between NR and LTE needs to be addressed.

- RAN3 is kindly requested to determine which solution should be accepted to address this misalignment—either aligning with LTE or aligning with NR.

- RAN3 agree CR for either 38.423 or 36.423 after conclusion is reached.

4	Reference

[1] 38.423

[2] 36.423