3GPP TSG-RAN WG3 #126                                             	    R3-247747

Orlando, USA, Nov 18th –22nd, 2024

Agenda item:	   13.2

Source: 		Reliance Jio, NTT DOCOMO, Qualcomm Incorporated, Vodafone, Sony, Bharti Airtel, Telstra

Title: 	Inter-gNB/CU LTM typical deployment scenarios and operational requirements

Document for:	Discussion/Agreement.

# 1	Introduction

In this paper, we provide various typical Inter-gNB operator deployment scenarios that need to be supported by R19 Inter-CU/gNB LTM feature and some basic principles to allow operators to deploy Inter-CU/gNB LTM feature without causing too much signalling within network and also provide uniform LTM UE HO latency experience for both intra-gNB LTM and inter-gNB LTM scenarios.

# 2	Typical inter-gNB/CU LTM deployment scenarios

As we have closely observed that RAN3 is discussing 2 solutions for supporting R19 Inter-CU/Inter-gNB LTM and there are various technical arguments for different solutions. From operator perspective, first we prefer RAN3 to discuss key deployment scenarios that need to be supported in real operator networks.

Following are 3 key deployment scenarios to supported for R19 Inter-CU/gNB LTM independent of solution. We understand that during Inter-CU/gNB LTM preparation phase, multiple cells from multiple gNBs will be configured to LTM UEs and UE should be able to performs subsequent Inter-CU/gNB LTM across these pre-configured inter-gNB cells.

Deployment scenario 1: Centralized architecture CU deployment scenario - single gNB-CU with multiple distributed DUs and Cells.

In this deployment scenario, centralized CU with large number of DUs/Cells is considered. When UE is LTE pre-configured with cells from neighbour gNB-CU Cells, operator goal is to avoid ping-pong LTM preparation signalling when UE is freely moving across the neighbouring gNB cells.

Deployment scenario 2: Distributed architecture CU deployment scenario - multiple gNB-CUs with three-sector DUs.

In this deployment scenario, each gNB-CU is considered as supporting 3 sectors (i.e., 3 cells). In dense urban areas, when UE is in RRC_CONNECETD state, may move across cells belonging to multiple gNBs. In such scenario, when UE is LTE pre-configured with cells from neighbour gNB Cells, operator goal is to avoid ping-pong LTM preparation signalling when UE is freely moving across the small group of gNB cells.

Deployment scenario 3: Multi-layered architecture Heterogeneous CU deployment scenario – Large capacity gNB-CU with multiple DUs for Macro outdoor coverage and multiple small cells (CU with small capacity small cells) for Indoor coverage.

In this deployment scenario, centralized CU with large number of DUs/cells is considered for macro outdoor coverage. Multiple small cells with 1 to 3 cells max capacity is considered for indoor coverage. It is possible that LTM UE may move between Indoor and Outdoor coverage areas and LTM UE should be allowed to move across macro gNB cells and small cells via subsequent inter-gNB LTM procedures with incurring excessive backhaul co-ordination signalling. LTM UE pre-configured inter-gNB cells configuration should not be updated frequently to reduce very expensive over the air RRC signalling.

As observed, RAN3 is currently discussing two potential solutions for supporting Release 19 (R19) Inter-CU/Inter-gNB LTM, with various technical arguments favoring different approaches. From an operator's perspective, we recommend that RAN3 prioritize discussions around the primary deployment scenarios that must be supported in real-world operator networks to ensure practical applicability and alignment with operational needs. Establishing these deployment scenarios as a basis will help to clarify the technical requirements and guide the selection of the most suitable solution. We identify three key deployment scenarios that need to be supported for R19 Inter-CU/Inter-gNB LTM, independent of the specific solution chosen. During the Inter-CU/Inter-gNB LTM preparation phase, we anticipate that multiple cells from various gNBs will be configured for LTM-capable UEs, enabling seamless operations across a broader network area. This approach requires that UEs be capable of conducting subsequent Inter-CU/Inter-gNB LTMs across these pre-configured inter-gNB cells, allowing enhanced continuity and performance as UEs move through the network. Based on above typical operator deployment scenarios and discussion, we have following proposal. Proposal 1: RAN3 to support Inter-CU/gNB LTM design to meet typical operator deployment scenarios described in this document.

# 3	Operator Requirements for Inter-CU/gNB design

To deploy any feature by operator network, we would like to consider features which are providing benefits to both LTM UEs and network signalling efficiency as well.

Once an operators starts deploying R18 LTM solution, we expect that deployed R18 UEs should get benefit of lower HO interruption without additional signalling towards UEs for both Intra-gNB and Inter-CU/gNB scenarios for LTM Handover.

Proposal 2: Operator should have flexibility to re-use R18 Intra-gNB LTM UEs for R19 Inter-gNB LTM deployment as well.

For any 5G connectivity, for both HO and non-HO cases, operators are expected to provide strong secure connections and try to minimize or avoid frequent security updates if not needed and at the same time comply with WID objectives. Any security solution for LTM should avoid impact to our 5GC network. In order to meet this requirement, we propose

Proposal 3: RAN3 shall design inter-CU/gNB LTM without compromising security and comply with WID objectives.

Proposal 4: RAN3 solutions should avoid CN impact to support subsequent Inter-CU/gNB LTM HO security updates.

When any operators are deploying advanced mobility solutions in their network, we already have L3 mobility currently deployed and new 5G-Advanced R19 Inter-CU/gNB mobility solutions should not create additional signalling compared to R18 Inter-gNB LTM and legacy R15 L3 based HO. At the same time, within operator network we pay close attention how to minimize or even avoid any ping-pong signalling within network and towards UEs as well. Any solution which may cause ping-pong HO signalling and causing unwanted RLF declaration and RRC Re-establishment is undesirable. This needs to be considered seriously while designing any solution in 3GPP discussions.

Proposal 5: RAN3 shall consider Inter-CU/gNB LTM solutions without adding additional co-ordination signalling among pre-configured RAN nodes to support subsequent R19 Inter-CU/gNB LTM procedures.

Proposal 6: RAN3 shall consider Inter-CU/gNB LTM solutions to avoid any ping-pong network signalling, prevent Inter-CU LTM/gNB HO failures and RLF declaration by LTM UEs.

In RAN2/RAN3 discussions, we understand that there are 2 solutions are on the table.

Solution 1, which is requiring RRC/PDCP re-location from source CU to target CU for each Inter-CU/gNB LTM HO and causes security key update as well.

Solution 2, which does not requires RRC/PDCP re-location from source CU to target CU during Inter-CU/gNB LTM and no security update needed at the time of cell switch but Xn interface is used for signalling interaction between CU1 and CU2.

In different operators networks deployments, Xn latencies are variable depending on transport network deployment which is completely under operational control and is not a problem for 3GPP solution to discuss. Most of 5G operators have deployed advanced transport networks and we do not expect large Xn signalling latencies between neighbour gNBs.

Proposal 7: Xn-interface latency is under operators transport network design choice and is not expected to be  large in most of typical inter-gNB deployments.

Since there are 2 solutions on the table and we propose RAN3 to discuss and compare both solutions and make decisions which will help not just some implementations but also MUST help operator network deployments and our end user experience for LTM UEs. We operators do not prefer to make hasty decisions without proper analysis of different solutions.

Proposal 8 : RAN3 shall evaluate different solutions and design inter-CU/gNB LTM feature to meet operator network deployment scenarios and provide benefit to both network and LTM UEs as well.

# Conclusion

This contribution discussed inter-gNB LTM general deployment aspects and various operator requirements. We thus have following proposals to be agreed by RAN3 to make it commercially viable for operators.

Proposal 1: RAN3 to support Inter-CU/gNB LTM design to meet typical operator deployment scenarios described in this document.

Proposal 2: Operator should have flexibility to re-use R18 Intra-gNB LTM UEs for R19 Inter-gNB LTM deployment as well.

Proposal 3: RAN3 shall design inter-CU/gNB LTM without compromising security and comply with WID objectives.

Proposal 4: RAN3 solutions should avoid CN impact to support subsequent Inter-CU/gNB LTM HO security updates.

Proposal 5: RAN3 shall consider Inter-CU/gNB LTM solutions without adding additional co-ordination signalling among pre-configured RAN nodes to support subsequent R19 Inter-CU/gNB LTM procedures.

Proposal 6: RAN3 shall consider Inter-CU/gNB LTM solutions to avoid any ping-pong network signalling, prevent Inter-CU LTM/gNB HO failures and RLF declaration by LTM UEs.

Proposal 7: Xn-interface latency is under operators transport network design choice and is not expected to be large in most of typical inter-gNB deployments.

Proposal 8 : RAN3 shall evaluate different solutions and design inter-CU/gNB LTM feature to meet operator network deployment scenarios and provide benefit to both network and LTM UEs as well.