3GPP TSG-RAN WG3 Meeting #126                                                              R3-247749

Orlando, USA, 18-22 November 2024

Agenda item:	 11.3

Source:		Rakuten

Title:		Discussion of AI/ML based CCO

Document for:		Discussion and Decision

# 1	Introduction

RAN3#125-bis meeting agreed that:

Timing information:

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

# 2	Discussion

# In the legacy CCO mechanism:

Step 1: The gNB-CU detects the CCO issue (i.e., either coverage issue or cell edge capacity issue) and affected cells/SSBs;

Step 2: The gNB-CU sends the CCO Assistance Information IE (within the GNB-CU CONFIGURATION UPDATE message over F1) containing the CCO issue type and affected cells/SSBs to the gNB-DU;

Step3: The gNB-DU uses the received CCO Assistance Information IE to determine the new coverage state (CCO state) for the affected gNB-DU’s cells/SSBs;

Step 4: The gNB-DU sends the Coverage Modification Notification IE (within the GNB-DU CONFIGURATION UPDATE message over F1) containing the new CCO state for the affected gNB-DU’s cells/SSBs to the gNB-CU;

Step 5: The gNB-CU sends the Coverage Modification List IE (within the NG-RAN NODE CONFIGURATION UPDATE message over Xn) containing the detected CCO issue type and the new CCO state for the affected gNB-DU’s cells/SSBs to the neighbour gNB to allow for the neighbour gNB to determine a new CCO state for its own cells/SSBs that best matches the new CCO state of the gNB-CU’s cells/SSBs resulting from the detected CCO issue.

For the AIML-based CCO mechanism:

-	Step 0: AIML training entity generates predicted CCO issue

-	Step 1: gNB-CU predicts the CCO issue.

-	Step 2: gNB-CU sends the predicted CCO issue to gNB-DU.

-	Step 3: gNB-DU generates the future coverage status based on the predicted CCO issue and other local information, whether only local information is used can be further discussed.

-	Step 4: gNB-DU sends the future coverage status to gNB-CU.

-	Step 5: gNB-CU sends the future coverage status to neighbour gNBs (say gNB2).

- 	Step 6: gNB2-CU uses the predicted issue, the future coverage state, the respective timing information, and infers its own CCO issue

- 	Step 7: gNB2-CU sends this information to gNB2-DU which decides whether any changes are required in its own coverage/capacity configuration

Observation 1: Since gNB2-CU gets the information about the predicted CCO issue, and the future CCO state of gNB1 ahead of the prediction time and the updated configuration time, it is likely that it will deduce its own CCO issue ahead of that time. And similarly, it is likely that gNB2-DU will also recommend its own future CCO state ahead of the time.

Therefore, the timing information agreed in the previous meeting needs to extend to gNB2 also, wherein the predicted CCO issue and future CCO state timing of gNB1 and gNB2 might differ.

Proposal 1: We propose that the timing information agreed in the previous meeting needs to extend to gNB2 also, wherein the predicted CCO issue and future CCO state timing of gNB1 and gNB2 might differ.

Let us assume that at time t0, gNB1-CU predicts the CCO issue to occur at time t0+x. gNB1-CU sends the predicted issue with the predicted time to the gNB1-DU. gNB1-DU generates the future coverage status based on the predicted CCO issue and other local information and decides the time to+y when this future coverage status will be applied.  Based on this, gNB2-CU predicts the CCO issue to occur at t0+p and gNB-DU recommends the future CCO state to be applied at t0+q based on the input from gNB1-CU

In this paper, we’d also like to bring to notice of RAN3 a scenario where a potential conflict might occur between the legacy and the AI/ML based CCO mechanisms. It is understood that both the mechanisms will co-exist, therefore, we should study the interaction between the two and avoid possibility of scenarios where the two might conflict.

Consider the scenario where the future CCO state of gNB1-DU is to be applied at time t0+y, and in the vicinity of that moment of time, the legacy CCO mechanism detects a CCO issue. This could happen is the prediction at gNB1 due was not accurate, or the prediction time of the CCO issue was not accurate.

Case 1: The proposed “future” CCO state has not been applied by either gNB1 or gNB2

- gNB1-DU has not yet applied the proposed configuration change which it had derived in response to the predicted CCO issue, however, that change might no longer be valid.

- gNB1-DU determines the new coverage state and informs gNB1-CU

- gNB1-CU informs gNB2-CU of cancellation of the earlier predicted CCO issue and the future coverage state. It also includes the information of currently detected CCO issue and the CCO state applied by the gNB1-DU in response to the CCO issue detected by gNB1-CU.

- gNB2-DU derives a new CCO issue based on the information received from the gNB1-CU and informs gNB2-DU of the change.

- gNB2-DU determines the new coverage state in response to the latest information and informs the gNB2-CU

Case 1: The proposed “future” CCO state has been applied by either gNB1 or gNB2

- gNB1-DU has not yet applied the proposed configuration change which it had derived in response to the predicted CCO issue, however, that change might no longer be valid.

- gNB1-DU determines the new coverage state and informs gNB1-CU

- gNB1-CU informs gNB2-CU of cancellation of the earlier predicted CCO issue and the future coverage state. It also includes the information of currently detected CCO issue and the CCO state applied by the gNB1-DU in response to the CCO issue detected by gNB1-CU.

- gNB2-CU derives a new CCO issue based on the information received from the gNB1-CU and informs gNB2-DU of the change.

- gNB2-DU determines the new coverage state in response to the latest information and informs the gNB2-CU

Case 2a: If gNB1-DU has applied the “future” CCO state configuration and a new CCO issue is detected at gNB1-CU

- gNB1-CU informs the detected CCO issue to gNB2-DU with an additional information to reverse the recently applied change to the CCO configuration.

- The rest of the flow will follow the legacy mechanism.

- This will result in gNB2-CU and gNB2-DU to also cancel the prediction based CCO configuration, and follow the legacy mechanism.

Case 2b: If gNB1-DU has applied the “future” CCO state configuration and a new CCO issue is detected at gNB2-CU

- This might require a careful handling. It is possible that the CCO issue has been detected at gNB2-DU in response to the CCO state configuration at gNB1-DU.

- RAN3 could recommend a hysteresis period after a CCO state configuration change during which no CCO detection is possible.

Case 3a: If gNB2-DU has applied the “future” CCO state configuration and a new CCO issue is detected at gNB1-CU

- gNB1-CU will not be aware that the gNB2-DU has applied the future CCO state configuration.

- Therefore, the flow will be same as Case 1.

Case 3b: If gNB2-DU has applied the “future” CCO state configuration and a new CCO issue is detected at gNB2-CU

- This might require a careful handling. It is possible that in response to the configuration change at gNB2-DU, this issue has been detected, but it’s possible that gNB1-DU has not yet applied its own CCO configuration change.

- RAN3 could recommend that t0+q is greater than t0+y then gNB2-DU could potentially treat this like a legacy CCO issue.

There are other permutations/combinations of the above, and we would like to propose to study them carefully to avoid corner cases which might result in frequency changes of configuration and an unstable system behavior

Proposal 2: RAN3 to consider the interaction of legacy CCO mechanism and the AI/ML based CCO mechanism and decide on recommendations to avoid conflicts between the two.

# 3	Conclusion

Observation 1: Since gNB2-CU gets the information about the predicted CCO issue, and the future CCO state of gNB1 ahead of the prediction time and the updated configuration time, it is likely that it will deduce its own CCO issue ahead of that time. And similarly, it is likely that gNB2-DU will also recommend its own future CCO state ahead of the time.

Proposal 1: We propose that the timing information agreed in the previous meeting needs to extend to gNB2 also, wherein the predicted CCO issue and future CCO state timing of gNB1 and gNB2 might differ.

Proposal 2: RAN3 to consider the interaction of legacy CCO mechanism and the AI/ML based CCO mechanism and decide on recommendations to avoid conflicts between the two.

# Reference

[1] TR 38.743

[2] TS 38.473