3GPP TSG-RAN WG3 #126	R3-247751

Orlando, US, 18 – 22 Nov. 2024

Agenda Item:	13.2. Support for inter-CU LTM

Source:	NTT DOCOMO INC.

Title:	Discussion on inter-CU LTM DC case

Document for:	Discussion, agreement

Introduction

At the last meeting, some papers for inter-CU LTM with DC case were submitted, but they were not treated. In the WID [1], following are stated:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

In this contribution, we discuss open points to be discussed for support of inter-CU SCG and MCG LTM.

Discussion

Based on the WID, the scenarios to be discussed as inter-CU LTM DC cases are 1) Inter-CU MCG LTM without SCG change and 2) Inter-CU SCG LTM without MCG change. On the other hand, the case where MCG LTM and SCG LTM are configured simultaneously is excluded. In this contribution, open issues in each scenario are discussed.

Inter-CU SCG LTM without MCG change

In Inter-CU SCG LTM without MCG change, two cases should be considered: 1) MN initiated SN change and 2) SN initiated SN change.

In the MN initiated case, the MN can trigger the preparation of Inter-CU SCG LTM without MCG change using the SN Addition Request for the same purpose as the Handover Request in the Inter-CU LTM non-DC case. In the non-DC inter-CU LTM, RAN3 agreed on the LTM Configuration Update procedure to be performed for the subsequent LTM as a 2nd round trip, and a similar procedure is needed for the SN Addition. In the case of MN initiated, it is FFS whether the legacy procedure (e.g. SN Modification Request) is reused for this signaling or not. In the case of MN initiated, the MN can handle to avoid simultaneous MCG inter-CU LTM and SCG inter-CU LTM configuration, so a solution for the simultaneous configuration seems not needed.

In the case of SN initiated, a procedure similar to the MN initiated case can be initiated based on a request from the SN to the MN. This request can reuse an existing SN Change Required. Again, in the SN initiated case, if MCG LTM is configured when the SN requests SCG LTM from the MN, the MN can refuse this request to avoid the simultaneous configuration.

Observation 1:	The simultaneous configuration of Inter-CU MCG LTM and SCG LTM can be handled by the MN.

Proposal 1:	For MN initiated inter-CU SCG LTM without MCG change, SN Addition Request can be reused for 1st round trip between MN and candidate SN(s) to trigger the LTM configuration preparation.

Proposal 2:	For MN initiated inter-CU SCG LTM without MCG change, 2nd round trip between MN and candidate SN(s) to transfer LTM candidate configuration for subsequent cell switch is needed.

Proposal 3:	For SN initiated inter-CU SCG LTM without MCG change, SN Change Required can be reused for triggering inter-CU SCG LTM followed by a procedure similar as MN initiated inter-CU CSG LTM without MCG change.

Inter-CU MCG LTM without SCG change

In Inter-CU MCG LTM without SCG change case, multiple candidate MCGs and the same SCG for each MCG are configured, so the CHO with SCG can be considered as a baseline. In CHO with SCG, the source MN makes a mobility decision and sends a request to each candidate MN. Based on this request, the candidate MN selects a single associated candidate SN/SCG and sends SN Addition Request. Similarly, in this case, when the candidate MN receives a request from the source MN, it performs SN addition to the SN with which the UE is currently establishing DC and includes the SCG configuration in each LTM candidate configuration. Also, since the SN receives SN Additions to configure LTM from multiple candidate MNs and the SN can aware that those requests concern a single UE based on CHO with SCG mechanism, so that it can handle the simultaneous configuration issue.

Proposal 4:	Legacy procedure for CHO with SCG can be a baseline for inter-CU MCG LTM.

SN security key handling

SN security key handling in the Inter-CU SCG LTM can support the subsequent LTM by reusing the solution supported in the subsequent CPAC. That is, the list of sk counter is pre-configured for the UE in the LTM preparation phase. Also, the source MN generates the list for all candidate SNs and forwards it to all candidate SNs.

SN security key handling in the Inter-CU MCG LTM cannot use the solution in the subsequnent CPAC when the MN changes (i.e., when the KgNB changes). This is because when the KgNB changes, the KSN must also change, so unless a list of future KgNBs is known in advance, it is not possible to generate configuration for KSN changes in advance. In other words, how to refresh the SN security key depends on how security key handling is performed in the Inter-CU LTM non-DC case, and this point is still under discussion, so that it should be suspended.

Proposal 5:	For inter-CU SCG LTM without MCG change, SN security key refresh mechanism for subsequent CPAC can be reused.

Proposal 6:	For inter-CU MCG LTM without SCG change, SN security key refresh mechanism would involve gNB security key refresh, and this is under discussion on other WG and not finished, so this point should be suspended.

Conclusions and proposals

Our proposals are summarized below.

Observation 1:	The simultaneous configuration of Inter-CU MCG LTM and SCG LTM can be handled by the MN.

Proposal 1:	For MN initiated inter-CU SCG LTM without MCG change, SN Addition Request can be reused for 1st round trip between MN and candidate SN(s) to trigger the LTM configuration preparation.

Proposal 2:	For MN initiated inter-CU SCG LTM without MCG change, 2nd round trip between MN and candidate SN(s) to transfer LTM candidate configuration for subsequent cell switch is needed.

Proposal 3:	For SN initiated inter-CU SCG LTM without MCG change, SN Change Required can be reused for triggering inter-CU SCG LTM followed by a procedure similar as MN initiated inter-CU CSG LTM without MCG change.

Proposal 4:	Legacy procedure for CHO with SCG can be a baseline for inter-CU MCG LTM.

Proposal 5:	For inter-CU SCG LTM without MCG change, SN security key refresh mechanism for subsequent CPAC can be reused.

Proposal 6:	For inter-CU MCG LTM without SCG change, SN security key refresh mechanism would involve gNB security key refresh, and this is under discussion on other WG and not finished, so this point should be suspended.

References

- RP-242356, WID “NR mobility enhancements Phase 4”