3GPP TSG-RAN WG3#126 	R3-247108

Orlando, USA, 18-22 November 2024

Agenda item:	13.2

Source:	Nokia

Title:	Semi-persistent CSI-RS support for LTM

Document for:	Discussion and Decision

# 1	Introduction

RAN1 has sent an LS to RAN3 (R1-2409283) to inform about the working assumption made in the last RAN1 meeting.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective

The above working assumption has a clear RAN3 impact and in this paper, we will explain the impacts and the related proposals.

# 2   Discussion

Below we describe the RAN3 impacts:

First impact:

The first and foremost impact of the above working assumption is to provide the configuration of CSI-RS resource either in non-UE associated signalling or UE associated signalling. This is also discussed in our companion contribution (R3-247102).

Proposal: (Semi-persistent) CSI-RS resource configuration is provided to source CU/DU.

Second impact:

Furthermore, semi-persistent CSI-RS can be activated and de-activated. Therefore, RAN3 will have to decide whether the (de-)activation should be done by source DU or target DU(s).  In our opinion, it should be the source DU triggering the activation and de-activation of semi-persistent CSI-RS. This is mainly because source DU can select appropriate SP CSI-RS(s)  based on either L1 or L3 measurements. For example, based on L1 measurements (SSB based or periodic CSI-RS based), source DU can decide whether additional CSI-RSs need to be activated for some candidate cells to perform beam measurements with more refined beams.

Proposal: Source DU can (de-)activate semi-persistent CSI-RS resource measurement and reporting.

The above can be done by specifying a new procedure over F1 and Xn interface. Once, the (de-)activation request is accepted by the target DU, an acknowledgement message should be sent to the source DU which in turn will trigger CSI measurement and reporting on the UE side.

Based on the above, we do not see any showstopper and semi-persistent CSI-RS can be supported from RAN3 point of view. Likewise,  a reply LS to RAN1 can be sent with the above conclusion and the impacts.

Proposal: Semi-persistent CSI-RS resources can be supported from RAN3 signalling point of view.

Proposal: Send a reply to LS from RAN1 capturing the above mentioned conclusion and impacts.

# References

R3-247102 Discussion on Inter-gNB-CU LTM Procedure, Nokia

# Annex: Reply-LS to RAN1

3GPP TSG-RAN WG3#126 	R3-24XXXX

Orlando, USA, 18-22 November 2024

# Title:	(draft- Reply) LS on the support of semi-persistent CSI-RS resource for LTM candidate cells

# Response to:	R1-2409283

# Release:	Rel-19

# Work Item:	NR_Mob_Ph4-Core

Source:	Nokia [To be RAN3]

To:	RAN1

Cc:	RAN2

Contact Person:

Name: Andres Arjona

E-mail Address: andres.arjona@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	none

1. Overall Description:

Working Assumption

In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective

The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS:

From RAN3 point of view, the above working assumption can be supported without any big RAN3 workload.

RAN3 foresee the following impacts:

- The first and foremost impact of the above working assumption is to provide the configuration of CSI-RS resource as part of preparation phase.

- Furthermore, semi-persistent CSI-RS can be activated and de-activated. Therefore, RAN3 will have to decide whether the (de-)activation should be done by source DU or target DU(s) and define the related F1 and Xn signalling.

To RAN1 group.

ACTION: 	RAN3 respectfully asks RAN1 to take the above into account.

3. Date of Next TSG RAN WG3 Meetings:

TSG RAN WG3 Meeting #127	17th– 21st February 2025	Athens, GR

TSG RAN WG3 Meeting #127-bis	7th-11th April 2025		China, CN