3GPP TSG-RAN WG3#126 	R3-247112

Orlando, USA, 18-22 November 2024

Agenda item:	13.2

Source:	Nokia, KDDI, LG Electronics, ZTE, Rakuten, Fujitsu

Title:	Support of Multiple Reference Configurations

Document for:	Discussion and Decision

# 1	Introduction

RAN2 has sent an LS to RAN3 (R2-2409378) to inform about RAN2 agreements for Inter-CU LTM. The outcome regarding multiple reference configurations has substantial RAN3 impacts. This contribution describes the impacts and provides related proposals.

# 2   Discussion

Support of multiple reference configuration has been a controversial topic in RAN2. Despite having almost equal support, it was decided in RAN2 that single reference configuration is supported for inter-CU LTM. The exact agreement made in RAN2 is as follows:

Agreement: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

It is to be noted that RAN2 decision on multiple reference configuration was done on voting basis without inquiring RAN3 on the benefit of having multiple reference configurations. Also, the support on multiple reference configuration in RAN2 was not ignorable as can be seen below.

Companies position in RAN2: - Multiple reference configurations: LG, Samsung, CMCC, Nokia, Fujitsu, Rakuten, ITRI, KDDI, ZTE, BT, IDC, KT, ETRI (13) - Single reference configuration: Google, Honor, CATT, Lenovo, Vivo, Apple, OPPO, Spredtrum, Xiaomi, NEC, Intel, MediaTek, Qualcomm, China Telecom, Ericsson, Sharp (16)

Therefore, we believe that this topic needs to be revisited from RAN3 point of view, as we believe the main benefit would be the simplification of RAN3 signalling. Based on RAN3 discussion outcome, an LS should be sent to RAN2 if arise the need to revisit RAN2 decision.

Proposal 1: RAN3 to discuss the need of multiple reference configuration from RAN3 signalling point of view.

In our view, supporting on single reference configuration in case of inter-CU LTM brings more complexity in RAN3 signaling than benefits. We list below the challenges that RAN3 will face in designing an inter-CU handover with single reference configuration.

Challenge #1: First, it needs to be decided in RAN3 who generates the reference configuration. Is it the source CU or one of the candidate target CU?

Challenge # 2: If one of the candidate target CUs generates the reference configuration, it incurs additional signaling to first fetch the reference configuration at source CU and then use it to get delta configurations of other candidate cells. This would cause additional signaling overhead over F1 and Xn.

Challenge # 3: If source CU generates the reference configuration based on its own cells, it may not be the optimum choice as source CU may only contain one cell and all other candidate cells may belong to different CU. This would result in sub-optimum use of reference configuration with no clear benefit of reducing signaling overhead.

Challenge # 4: Commonality between cell configurations belonging to different CUs may be limited,  especially in case of multi-vendor CUs and different frequency ranges. Hence, this makes the reference configuration feature not useful and also not feasible for actual deployments.

Challenge # 5: If RAN3 decides to go in the path that one CU can generate reference configuration based on its own cell configurations but other cells belonging to different CU use complete configuration, this results in additional complexity in RRC signaling and structure from both RAN2 and RAN3 point of view.

Challenge #6: Mechanism for updating reference each time a a new candidate cell or candidate CU is prepared.

Observation 1: It is observed that the use of single reference configuration brings more challenges in RAN3 than actual advantages.

Proposal 2: RAN3 to discuss the above-mentioned challenges with respect to the use of single reference configurations.

We believe that supporting multiple reference configurations for inter-CU LTM is a straightforward solution for the above challenges and with clear advantages. In addition, it is a simple extension with reduced effort from RAN3 point of view. The justification of this is as follows:

Justification # 1: Cell configurations have the maximum similarities when they belong to same CU (and/or frequency ranges). Hence, generating one reference configuration per CU makes more sense.

Justification # 2: Reference configuration per CU will results in reduced signaling from RAN3 point of view. Meaning, every CU can generate reference configuration and provide it as part of HANDOVER REQUEST ACKNOWLEDGEMENT message.

Justification # 3: It gives the network flexibility to configure the number of reference configurations which provides the maximum benefits (subject to UE capability).

Justification # 4: It helps the modularity in RRC structure design where LTM Config is generated per CU (subject to further RAN2 discussions).

Justification #5: Addition and removal of candidate gNBs does not require an update procedure related to the reference configuration.

Based on the above, we propose the following.

Proposal 3: RAN3 to agree that multiple reference configurations provide more benefits compared to single reference configuration from RAN3 signaling and standardization effort point of view.

Proposal 4: Send an LS to RAN2 requesting to revisit the agreement on reference configuration considering RAN3 view.

# 3	Conclusions

Proposal 1: RAN3 to discuss the need of multiple reference configuration from RAN3 signalling point of view.

Observation 1: It is observed that the use of single reference configuration brings more challenges in RAN3 than actual advantages.

Proposal 2: RAN3 to discuss the above-mentioned challenges with respect to the use of single reference configurations.

Proposal 3: RAN3 to agree that multiple reference configurations provide more benefits compared to single reference configuration from RAN3 signaling and standardization effort point of view.

Proposal 4: Send an LS to RAN2 requesting to revisit the agreement on reference configuration considering RAN3 view.