3GPP TSG-RAN3 Meeting #126	R3-247153

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	10.3.2

Source:	Qualcomm Incorporated

Title:	SON MDT for network slicing

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss aspects related to SON/MDT for network slicing based on agreements and open issues from previous meetings.

# Discussion

  - Logged MDT enhancements for optimizing slice-based cell reselection

RAN3 to work on logged MDT enhancement for slice-based cell reselection.

Two use cases were captured for slice-based cell reselection

- First use case is about optimizing mapping between the slice groups and frequency priority using logged MDT

- Second use case is informing the network that UE performs slice-based re-selection instead of general reselection

### Use case 1 (Optimizing mapping between slice groups and frequency priority)

In the previous meetings, there were proposals for UE to log the slice group(s) i.e., NSAGs or frequency priorities in the logged MDT report and report it to the gNB to optimize the mapping of S-NSSAI to NSAG or optimize the frequency priorities.

We think adding these network configurations in the logged MDT report is not appropriate as this adds burden for the UE to store these configurations in RRC_IDLE and adds to the Uu overhead in logged MDT report. gNB should be able to store this information if needed.

And it is be noted that the gNB can’t identify the UE across state transitions, so even if these network configurations are reported back, there can’t be any UE-specific optimizations.

Requiring the UE to log the NSAGs or frequency priorities in the logged MDT report adds burden for UE to store these configurations in RRC_IDLE and adds to Uu overhead in logged MDT report. gNB should be able to store this information if needed

gNB can’t identify the UE across state transitions, so even if UE reports NSAGs or frequency priorities, there can’t be any UE-specific optimizations.

Use case 1 is not considered in Rel-19 i.e., there is no need to optimize the mapping between NSAGs and frequency priorities

### Use case 2 (Slice-based reselection from general reselection mapping between slice groups and frequency priority)

In order to support slice-based cell reselection, a UE may be provided with slice-based cell reselection information via RRCRelease or SIB16 e.g., reselection priorities per NSAG per frequency and corresponding list(s) of cells where the slices of the NSAG are supported or not supported.

When a UE supports slice-based cell reselection, and when slice-based cell reselection information is provided to the UE, then the UE uses the slice-based cell reselection information.

When no slice-based cell reselection information is provided for any NSAG that was determined to be considered during cell reselection, then the UE uses the general cell reselection information, i.e., without considering the NSAG(s) and their priorities.

A UE provided with slice-based cell reselection information may reselect different cells compared to a UE not provided with slice-based cell reselection information parameters because of different reselection priorities and based on a bias introduced by whether the cell supports a certain slice or not. An operator trying to build coverage maps via logged MDT might not be able to build a true coverage map if it doesn’t know why UEs traversing a similar path see a different coverage (i.e., connect to different cells) even though the radio conditions are similar.

There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

For use case 2, UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

  - Slice-aware MRO

RAN3 agreements:

Work on the Slice aware MRO scenarios.

For successful handover, FFS if solutions are needed to optimize handovers in the case of slicing discontinuity, e.g., target cell not supporting slice.

The following is captured in TS 38.300

To make mobility slice-aware in case of Network Slicing, S-NSSAI is introduced as part of the PDU session information that is transferred during mobility signalling. This enables slice-aware admission and congestion control.

Both NG and Xn handovers are allowed regardless of the slice support of the target NG-RAN node i.e. even if the target NG-RAN node does not support the same slices as the source NG-RAN node.

Even though handovers are allowed regardless of the slice support of the target NG-RAN node, this leads to discontinuity in the slice service and interruption, if UE moves towards an area that does not support one of UE's ongoing slices (e.g., during inter-RA mobility).

To detect such scenarios and take correction action in the future, it was proposed by few companies last meeting to add slice-related information in SON reports (e.g., in UHI or SHR) e.g., include UE’s ongoing slice(s) in the source and target cell in UHI, collect SHR when a certain handover resulted in a slice service interruption.

Currently, a gNB already knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn. During handover preparation, the source gNB will know that a PDU session (which is associated to a slice) it is offloading to a certain target gNB might get interrupted due to non-support of that slice in the target gNB.

We therefore are not sure whether any enhancements to SON reports (e.g., collecting SHR in case of such handover to target cell supporting the ongoing slice) to do non-real optimizations are really needed when such things are known already at the gNB.

Also, a certain gNB via implementation can already take slice aware mobility decisions and mitigate slice service discontinuity. For example, a gNB can prioritize a target cell that supports UE’s ongoing slice in the source cell over another target cell that doesn’t support the UE’s ongoing slice (if radio conditions of the two target cells are comparable).

A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# Conclusion

Observation 1.	Requiring the UE to log the NSAGs or frequency priorities in the logged MDT report adds burden for UE to store these configurations in RRC_IDLE and adds to Uu overhead in logged MDT report. gNB should be able to store this information if needed

Observation 2.	gNB can’t identify the UE across state transitions, so even if UE reports NSAGs or frequency priorities, there can’t be any UE-specific optimizations.

Proposal 1:	Use case 1 is not considered in Rel-19 i.e., there is no need to optimize the mapping between NSAGs and frequency priorities

Observation 3.	There are benefits in making the OAM aware that the logged MDT reports were collected by a UE using slice-based cell reselection information (and not general cell reselection information).

Proposal 2:	For use case 2, UE can indicate via a flag in logged MDT report that it collected logged MDT using the slice-based cell reselection parameters (and not general cell reselection parameters).

Observation 4.	A gNB knows the slices supported by its served cells and the slices supported in the cells served by its neighbor gNBs via Slice Support List exchange in F1 and Xn.

Observation 5.	Based on the knowledge of slice(s) supported at neighboring gNBs, a source gNB has knowledge whether a PDU session (which is associated to a slice) will get interrupted or not upon offloading to a target gNB.

Proposal 3:	There are no enhancements needed from UE in SON reports to assist the gNB to optimize handovers with slice service discontinuity (e.g., due to target cell not supporting UE’s ongoing slice). gNB already has sufficient knowledge to identify such scenarios.

# 4. References