3GPP TSG-RAN WG3 Meeting #126	R3-247191

Orlando, USA, 18 - 22 Nov, 2024

Title:	(TP to TR 38.769) A-IoT Signalling and Procedures

Agenda Item:	16.3

Source:	Huawei

Document for:	other

# 1	Introduction

In RAN3#125bis meeting, the candidate procedures for A-IoT Inventory for Topology 1 and Topology 2 were captured in endorsed RAN3 pCR [1].

In this paper, we futher discuss the leftover aspects about A-IoT signalling and procedure.

# 2	Discussion

## 2.1	Information exchanged with A-IoT CN

In RAN3#124 meeting, there were two items captured in the pCR, i.e. A-IoT Device Identification, and Scope of inventory request. For the A-IoT Device Identification included in the Inventory Request, there is an Editor’s note captured in pCR section 6.5.1.1:

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

In the RAN2#127 meeting, it has been agreed that reader is able to trigger multiple/subsequent AIoT paging messages associated with the same request from the CN.  Based on RAN2 agreement, we can see that the A-IoT RAN needs to store the A-IoT Device Identification in order to allow the paging repetition over the A-IoT radio. Besides, based on current progress in SA2, RAN2 and RAN3, there is no need for the A-IoT RAN to interpret/process it.

- Proposal 1-1: The A-IoT RAN needs to store the A-IoT Device Identification in order to allow the paging repetition over the A-IoT radio, but there is no need for the A-IoT RAN to interpret/process it.

- In RAN2#127 meeting, it has been agreed that “targeted for one or more than one device” and “approximate number of target devices” are useful to be visible to the reader from CN. To support RAN2 agreement, the Inventory Request from A-IoT CN should be able to include “targeted for one or more than one device” and “approximate number of target devices”.

- In RAN2#127bis meeting, there is the following agreements:

Need of MSG Size Agreements - From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size. Two options can be captured: 1) from the CN and 2) from the device (simple message size indication). Capture the advantages/disadvantages. - The D2R message size would be beneficial but it is not essential. - Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size. Is it always available, sometimes, or never.

- Proposal 1-2: To support RAN2 agreement, the Inventory Request from A-IoT CN should be able to include “targeted for one or more than one device”, “approximate number of target devices”, and it is subject to SA2 on the inclusion of “estimate of expected D2R message size”.

- Besides of these, other information to be carried in Inventory Request can be discussed in normative phase. In our view, it is also needed to include Inventory Session ID in the Inventory Request message from A-IoT CN, to enable the A-IoT RAN to distinguish different requests from A-IoT CN(s). While from A-IoT CN point of view, upon receiving the Inventory Response/Failure/Report message from the A-IoT RAN, it also needs to get the Session ID to distinguish the Inventory procedures.

- Proposal 1-3: Other information to be carried in Inventory Request can be discussed in normative phase, e.g., Session ID.

- For the A-IoT Device ID provided in the Inventory Report, there is an Editor’s note captured in endorsed RAN3 pCR section 6.5.1.1:

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

- As mentioned above, there is no need for the A-IoT RAN to interpret/process A-IoT Device ID. Whether it sent transparent or not is subject to SA2, there is nothing to be done for this by RAN3, and thus it should be a NOTE.

- Proposal 1-4: Clean up the Editor’s Notes in section 6.5.1.1

- In Topology 2 NAS/UP based solution, the interactions between the A-IoT-enabled UE and A-IoT CN are transparently forwarded by the gNB, and the information will be the same or quite similar to the information (to be) listed in TR 38.769 section 6.5.1.1 [2]. In Topology 2 RRC based solution, upon receiving the information (to be) listed in TR 38.769 section 6.5.1.1, the A-IoT-enabled gNB will forward the received information to the A-IoT-enabled UE.

- Proposal 1-5: The information listed in TR 38.769 section 6.5.1.1 will be transmitted to the A-IoT-enabled UE from the A-IoT-enabled gNB (RRC based solution) or from the A-IoT CN (NAS/UP based solutions).

## 2.2	A-IoT-enabled UE selection

In RAN3#125bis meeting, regarding to reader selection in Topology 1, there is an Editor’s Note listed in RAN3 pCR section 6.5.2

Editor’s Note 1: Reader selection is FFS.

In RAN3#125bis meeting, there are the following agreements about reader selection:

For reader selection: in T1, reader selection may need the coordination between AIoT RAN and AIoT CN; in T2, all options are still open, different alternatives are possible as per call flows.

In current RAN3 pCR, it has also been captured that for Topology 1 reader selection may need coordination between A-IoT RAN node and A-IoT CN. Based on above RAN3 agreements and current description in RAN3 pCR, in our view, the Editor’s Note 1 should be removed.

Regarding to reader selection in Topology 2, there are two Editor’s Notes captured in RAN3 pCR section 6.4.2 and 6.5.3, respectively:

Editor’s Note 5: In Topology 2, it is FFS on reader selection, may need coordination between A-IoT RAN node and A-IoT CN.

Editor’s Note: Reader selection is FFS.

In Topology 2 NAS/UP based solution, considering the A-IoT procedures are terminated between the A-IoT-enabled UE and the A-IoT CN, the A-IoT Inventory/Command procedures are transparent to the gNB, therefore, the A-IoT-enabled UE reader selection should be performed by the A-IoT CN.

In Topology 2 RRC based solution, similar to Topology 1 the A-IoT RAN can be aware of AIoT services and reader selection may need the coordination between AIoT RAN and AIoT CN, the A-IoT-enabled gNB in Topology 2 is also able to be aware of the ongoing A-IoT services, and the A-IoT-enabled UE selection may need coordination between A-IoT-enabled gNB and A-IoT CN.

- Proposal 2-1: In Topology 2 NAS/UP based solution, A-IoT CN selects the A-IoT-enabled UE.

- Proposal 2-2: In Topology 2 RRC based solution, the A-IoT-enabled UE selection may need coordination between A-IoT-enabled gNB and A-IoT CN.

- Proposal 2-3: Clean up the Editor’s Notes Releated to Reader Selection.

## 2.3	Command operation

  - There are two Editor’s notes about Command captured in endorsed RAN3 pCR section 6.5.1.2:

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory)

  - For Editor’s Note 1, there is no clear requirement to support command on a group of devices or all devices. In our view, the Editor’s Note 1 should be removed.

  - For Editor’s Note 2, RAN2 has already agreed that the service type of A-IoT (e.g. inventory, command) is useful to be visible to the reader from CN. From RAN2 agreement, we can see that it is useful for A-IoT RAN to differentiate command and inventory. Regarding to whether the A-IoT RAN is aware of different types of command, there is so far no clear requirement for the A-IoT RAN to understand the different type of command, this detail aspect is pending to RAN2/SA2 and can be further discussed in normative phase.

- Proposal 3-1: The A-IoT RAN to differentiate command and inventory (already agreed in RAN2), and it can be further discussed in normative phase on whether RAN is aware of different types of command.

- Proposal 3-2: Clean up the Editor’s Notes in section 6.5.1.2

  - RAN2 has agreed that “inventory and command” case was agreed as the baseline. In this baseline, network first performs “paging” over A-IoT radio, the device sends its device ID to the network, and then the network sends command data to the device. In our view, the network should send DL data to the single device after the security checking at the A-IoT CN, therefore, the following command procedure should be adopted.

  - Proposal 3-3: Support Command Transmission after Inventory procedure for command operation.

# 3	Conclusion

- In this contribution, we further discussed the signaling and procedures over CN-RAN interface 1, and got the following proposals:

- On the Information exchanged with A-IoT CN

- Proposal 1-1: The A-IoT RAN needs to store the A-IoT Device Identification in order to allow the paging repetition over the A-IoT radio, but there is no need for the A-IoT RAN to interpret/process it.

- Proposal 1-2: To support RAN2 agreement, the Inventory Request from A-IoT CN should be able to include “targeted for one or more than one device”, “approximate number of target devices”, and it is subject to SA2 on the inclusion of “estimate of expected D2R message size”.

- Proposal 1-3: Other information to be carried in Inventory Request can be discussed in normative phase, e.g., Session ID.

- Proposal 1-4: Clean up the Editor’s Notes in section 6.5.1.1

- Proposal 1-5: The information listed in TR 38.769 section 6.5.1.1 will be transmitted to the A-IoT-enabled UE from the A-IoT-enabled gNB (RRC based solution) or from the A-IoT CN (NAS/UP based solutions).

- On the A-IoT-enabled UE selection

- Proposal 2-1: In Topology 2 NAS/UP based solution, A-IoT CN selects the A-IoT-enabled UE.

- Proposal 2-2: In Topology 2 RRC based solution, the A-IoT-enabled UE selection may need coordination between A-IoT-enabled gNB and A-IoT CN.

- Proposal 2-3: Clean up the Editor’s Notes Releated to Reader Selection.

- On the Command operation

- Proposal 3-1: The A-IoT RAN to differentiate command and inventory (already agreed in RAN2), and it can be further discussed in normative phase on whether RAN is aware of different types of command.

- Proposal 3-2: Clean up the Editor’s Notes in section 6.5.1.2

  - Proposal 3-3: Support Command Transmission after Inventory procedure for command operation.

The corresponding TP is provided in section 5.

# 4. Reference

- [1] R3-247036 (BL pCR to TR 38.769) Study on solutions for Ambient IoT in NR, Huawei, CMCC

- [2] 3GPP TR 38.769 Study on solutions for ambient IoT (Internet of Things) (Release 19) V1.0.0 (2024-09)

# 5. TP for TR 38.769 v1.0.0

----------------Start of the Change----------------

## 6.5	Impacts on CN-RAN interface

### 6.5.1	Information exchanged with A-IoT CN

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

NOTE 1:	The definition of this identification is out of RAN3 scope.

NOTE 2: The A-IoT RAN needs to store this identification to enable paging repetition over the A-IoT radio, but the A-IoT RAN does not need to interpret/process this identification.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

(3) Inventory Session ID (to distinguish different requests from A-IoT CN(s).)

(4) Assistance Information, which may include:

-	targeted for one or more than one device

-	approximate number of target devices

-	estimate of expected D2R message size

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

NOTE 3: It is up to SA2 whether device ID is sent transparent or not.

#### 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device. The A-IoT RAN is able to differentiate the command and inventory request from the A-IoT CN.

NOTE: Whether the A-IoT RAN can differentiate the different type of command can be furether discussed in normative phase.

#### 6.5.1.3	A-IoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, A-IoT radio resources can be requested in advance to the NAS/UP based communication with the A-IoT device or can be requested along with the NAS/UP based communication. There are different ways to trigger A-IoT radio resource allocation, e.g., upon CN request or upon UE request, etc. Some aspects of A-IoT radio allocation may be preconfigured by OAM.

#### 6.5.1.4	Releder Selection

In Topology 1, Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

In Topology 2 NAS/UP based solution, A-IoT-enabled UE is selected by the A-IoT CN.

In Topology 2 RRC based solution, A-IoT-enabled UE selection may need coordination between A-IoT RAN node and A-IoT CN.

----------------End of the Change----------------