3GPP TSG-RAN WG3 Meeting #126                                                              R3-247227

Orlando, Florida, USA, November 18 – 22, 2024

Agenda item: 		12.2

Source: 		Qualcomm Incorporated

Title: 	(draft Reply LS to SA2 – TP to BL CR 38.401) Discussion on SA2 questions on multi-hop WAB and UE ULI

Document for:		Other

# 1	Introduction

SA2 sent an LS to RAN3 on questions regarding FS_VMR_Ph2 in R3-244019/S2-2407345 with the following content [1]:

SA2 would like to inform RAN3 that SA2 had progressed the Rel-19 FS_VMR_Ph2 study and documented the conclusions in clause 8 in TR 23.700-06. Issue-1: SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases: a) MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and b) MWAB-UE accessing MWAB-gNB belonging to same MWAB. SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback. Issue-2: SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

In the last meeting, RAN3 discussed issue-1 and issue-2. For issue-1, RAN3 achieved the following agreement [2]:

The scenario where a WAB-gNB serves a WAB-MT(s) should be preventable by means of different standard based solutions RAN3 to specify solutions to prevent the multi-hop WAB topology, where multi-hop WAB means that a WAB MT connects to a WAB gNB. Discussions on multi hop WAB topology are out of scope

RAN3 further identified a set of candidate solutions for further down-selection:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCIs. FFS on any other legacy OTA parameters. Solution 2: Use the slice dedicated for backhauling, i.e. use a list of S-NSSAIs in RRCsetupcomplete to do access control and/or use a list of S-NSSAIs in handover signalling. No CN upgrade is needed. Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. Solution4: BH-gNB broadcasts a new indicator “WAB allowed” in SIB. WAB-gNB does not broadcast “WAB allowed”. Solution5: In case of handover for a WAB-node, the new WAB-node indication is included in the HO request, then the target BH-RAN node can perform access control for this WAB-node.

For issue-2, RAN3 observed that SA2’s LS points to LCS as the purpose of the “additional ULI”. Several companies believed that further clarification was needed from SA2 whether this “additional ULI” would have other purposes. Since SA2 discussed the same issue in a parallel meeting, RAN3 did not agree to sends an LS at that time.

This contribution aims to make progress in the discussion on SA2’s questions, and it contains a draft LS in Annex 1 and a TP for BL CR to 38.401 in Annex 2.

# 2	Discussion

## Issue 1a: WAB-MT connecting to non-collocated WAB-gNB

SA2 defined CN-based solutions to prevent the WAB-MT from connecting to a non-collocated WAB. These solutions have been captured in clause 6.2.3.6 of TS 23.700-6. They address the following scenarios:

- Preventing a WAB-MT from obtaining access to a WAB-gNB.

- Preventing an NG handover of a connected WAB-MT to a WAB-gNB.

The solutions proposed do not handle the following scenarios:

- Preventing a WAB-MT to (re)select a cell of a WAB-gNB

- Preventing Xn handover of a WAB-MT to a WAB-gNB.

Observation 0: Preventing the formation of multi-hop WAB topologies implies that:

- the WAB-MT in RRC IDLE cannot (re)select cells of a WAB-gNB

- the WAB-MT in RRC CONNECTED is not handed over to a WAB-gNB

In the last meeting, RAN3 identified five solution candidates to address SA2’s issue 1a. RAN3 has the goal to further down select among these solutions in the next meeting.

### Solution 1: Based on dedicated frequencies/PCIs for WAB-gNB cells

In case the WAB-gNB uses dedicated frequencies and/or PCI values, the WAB-MT can be configured to avoid (re)selection of cells with these frequencies and PCI values. It can further be configured to avoid inclusion of these cells in measurement reports. The configuration of the WAB-MT with this behavior can be based on implementation. In this manner, all the above issues can be addressed. This solution does not require any stage-3 enhancement.

The allocation of dedicated frequencies for the access to the WAB-gNB represents a likely deployment scenario, especially if access PLMN and backhaul PLMN are different.

In case the same frequencies are used for access to WAB-gNBs and backhaul, it is beneficial to allocate a dedicated set of PCI values for WAB-gNB cells for other reasons, e.g., to avoid PCI collisions between WAB-gNB-cells and cells of the stationary network. This approach has already been captured in TS 38.401, clause 7.8, as a solution for PCI collision avoidance of mobile IAB.

Observation 1: Solution 1, i.e., preventing multi-hop WAB topology based on the dedicated frequencies/PCIs for WAB-gNB cells, addresses most deployment scenarios and does not require stage-3 enhancements.

### Solution 2: Based on signalling of dedicated WAB slice ID

SA2 agreed that WAB authorization of the WAB-MT is based on a dedicated slice ID and DNN (CR 5688 to TS 23.501 in S2-2411238). Solution 2 uses the signaling of this WAB-dedicated slice ID to prevent formation of multi-hop WAB topologies.

Since the dedicate WAB slice ID is not broadcast by the BH RAN, the solution does not prevent the WAB-MT from (re)selecting WAB-gNB cells.

Observation 2a: In solution 2, the RRC-IDLE WAB-MT cannot know the gNB’s support for a specific slice ID, and it may therefore (re)select a WAB-gNB cell and perform an access attempt.

In case the WAB-MT selects such a WAB-gNB cell and performs initial access, it will include the WAB-specific slice ID in MSG.5. At this point, the WAB-gNB can release the RRC connection. However, this may not prevent the WAB-MT from performing access attempts to the same cell since the legacy RRC Release procedure does not allow including a cell-barring indication.

Observation 2b: In solution 2, the WAB-gNB can release the RRC connection of a WAB-MT after it has received the WAB-specific slice ID in MSG.5. However, there is no legacy mechanism to prevent the WAB-MT from conducting a new access attempt.

Observations 2a and 2b indicate that based on legacy procedures, the signalling of the dedicated WAB slice ID can neither prevent (re)selection of WAB-gNB cells not access attempts to WAB-gNB cells.

When the WAB-MT’s serving BH gNB initiates an Xn handover request toward a target gNB, it includes the WAB-specific slice ID in the HO Request message. In case the target gNB is a WAB-gNB, it can reject the handover request in compliance with TS 38.423, Clause 8.2.1:

If the target NG-RAN node does not admit at least one PDU session resource, or a failure occurs during the Handover Preparation, the target NG-RAN node shall send the HANDOVER PREPARATION FAILURE message to the source NG-RAN node. The message shall contain the Cause IE with an appropriate value.

The legacy Cause IE “Slice(s) not supported by NG-RAN” can be used. This implies that Xn handover of a WAB-MT to a WAB-gNB cell can be prevented by legacy means based on the dedicated WAB slice ID. There is no need for stage-3 enhancements.

Observation 2c: In solution 2, when the BH RAN initiates an Xn handover request to a WAB-gNB cell, the WAB-gNB can reject the handover preparation using legacy behaviour and legacy cause value.

### Solution 3: Based on “WAB-barred” broadcast by WAB-gNB cells

When WAB-gNB cells broadcasts an OTA “WAB-barred” indicator, WAB-MTs can consider such cells as barred during cell (re)selection. Obviously, this solution has stage-3 impact on network and on WAB-MTs. However, this solution does not have any impact on the BH RAN.

Observation 3a: In solution 3, the broadcast of “WAB-barred” by WAB-gNB cells can prevent WAB-MT to perform cell (re)selection of WAB-gNBs.

The OTA “WAB barred” indicator can also be used to prevent handovers of a WAB-MT to a WAB-gNB. When measuring candidate cells, the WAB-MT needs to decode SIB to detect the “WAB barred” indicator, and then exclude cells broadcasting this indicator from the measurement report. While a legacy UE, and therefore also the WAB-MT, can be configured to read SIB1 of candidate cells, such procedure creates significant overhead, and it should therefore not be assumed as a generic procedure to avoid multi-hop WAB.

Observation 3b: In solution 3, to avoid handover to WAB-gNBs, the WAB-MT can exclude candidate cells that broadcast “WAB-barred” from the measurement reports, but this requires decoding of SIB1 for all candidate cells which is not be desirable.

### Solution 4: Based on “WAB supported” broadcast by BH RAN

The “WAB-supported” broadcast by the BH RAN represents the mirror solution to the “WAB barred” broadcast by WAB-gNBs. It can therefore be used by the WAB-MT to avoid (re)selection of WAB-gNB cells, but it is not suitable to avoid handover of the WAB-MT to WAB-gNBs.

Solution 4 has stage-3 impact, and this stage-3 impact affects the BH RAN and the WAB-MT.

Observation 4: In solution 4, the “WAB supported” broadcast by BH RAN provides the same functionality as the “WAB barred” broadcast by WAB-gNB, but it has stage-3 impact on the BH RAN, which is undesirable.

### Solution 5: Based on new “WAB” indication in MT Xn handover request

This solution is functionally identical to solution 3, where the WAB-specific slice ID is used as such a WAB indicator. It would therefore work equally well to prevent multi-hop WAB topologies due to Xn handovers of WAB-MTs to WAB-gNBs. However, solution 5 has stage-3 impact, and this stage-3 impact affects the BH RAN.

Observation 5: In solution 5, including a new WAB indication into the MT’s Xn handover request has the same functionality as provided by the WAB slice ID in the MT’s Xn handover request, but it has stage-3 impact, and this stage-3 impact affects the BH RAN.

### Summary of solutions to prevent multi-hop WAB

Solution 1 can prevent (re)selection of WAB-gNB cells by WAB MTs, and it can prevent Xn handover of WAB-MTs to WAB-gNBs. Solution 1 by itself is therefore sufficient to prevent the formation of a multi-hp WAB topology. This solution further has no stage-3 impact. Solution 1 should therefore be supported.

Solution 1 relies on using dedicated frequencies and/or PCIs for WAB-gNB cells. While this condition may be met by most deployment scenarios, it may be desirable to have alternatives in scenarios where this condition cannot be met.

Solution 2 can prevent Xn handover of WAB-MTs to WAB-gNB’s. This solution has no stage-3 impact. Solution 2 should therefore also be supported.

Solution 3 can prevent (re)selection of WAB-gNB cells by WAB MTs. This solution has stage-3 impact, but this stage-3 impact only affects the WAB-gNB and WAB-MT, i.e., it does not affect the BH RAN.

Solutions 2 and 3 together are therefore sufficient to prevent the formation of a multi-hop WAB topology. Opposed to solution 1, solutions 2 and 3 can be applied to all deployment scenarios. Solutions 2 and 3 should therefore be supported.

Solution 4 can prevent (re)selection of WAB-gNB cells by WAB MTs, and solution 5 can prevent Xn handover of WAB-MTs to WAB-gNB’s. However, opposed to solutions 2 and 3, solutions 4 and 5 have stage-3 impact on the BH RAN. Such impact is undesirable since it requires BH RAN upgrade to prevent formation of a multi-hop WAB topology. There is further not need for solutions 4 and 5 since solutions 1, 2, and 3 can sufficiently address all issues. Solutions 4 and 5 should therefore not be supported.

Proposal 1: RAN3 to only support solutions 1, 2 and 3, and to down select solution 4 and 5.

Proposal 2: Capture on stage-2 that the formation of a multi-hop WAB-topology can be prevented by allocating dedicated frequencies and PCI values to WAB-gNB cells.

Proposal 3: Capture on stage-2 that to the formation of a prevent multi-hop WAB topology, the target WAB-gNB can reject handover requests based on the WAB-specific slice ID using legacy behaviour and signaling.

Proposal 4: Capture on stage-2 that to prevent formation of a multi-hop WAB topology, the WAB-gNB can broadcast a “WAB-barred” indicator in SIB, and the WAB-MT may avoid (re)selection of cells broadcasting this indicator.

Proposal 5: Capture solutions 1, 2 and 3 in the Reply LS to SA2 as a RAN-based remedy to issue-1a.

## Issue 1a: WAB-MT connecting to collocated WAB-gNB

Since the WAB-MT can be configured with the frequencies and PCIs of the WAB-gNB cells, it can avoid (re)selection of these cells, and it can avoid including these cells into measurement reports. The latter avoids WAB-MT handover to its collocated WAB-gNB. This configuration can be based on implementation.

Observation 6a: Preventing a WAB-MT to (re)select the cells of the co-located WAB-gNB and being handed over to (Re)selection of WAB-gNB cells by WAB-MT and Xn handover of the WAB-MT .

Proposal 6a: Capture on stage-2 that the WAB-MT can be prevented to connect to its collocated WAB-gNB based on implementation, where the WAB-MT avoids (re)selecting cells of its collocated WAB-gNB and including these cells in measurement reports.

Proposal 6b: Capture P6 in the Reply LS to SA2 as a RAN-based remedy to issue-1b.

Proposal 6c: Send LS to RAN2 to request support of “WAB barred” in SIB1 for WAB-gNB cells.

## Issue 2: Additional ULI

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

TS 23.700-06, clause 8.5 captures the following:

The following aspects are taken as way forward during normative work: - Existing principles (e.g. the two options for LMF to retrieve the MWAB location either via NRPPa procedure or via GMLC) are re-used for UE location service when MWAB is involved as described in sol#11. - The MWAB-gNB provides to the AMF an Additional ULI. The Additional ULI is: - TAC/Cell ID of the cell serving the MWAB-UE if the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are the same PLMN. - If the PLMN serving the MWAB-UE and the PLMN broadcasted by the MWAB-gNB are not the same PLMN, which below option used for the Additional ULI will be determined in normative phase. - option 1: mapped TAC/Cell ID based on geo-location of the MWAB based on input from OAM. - option 2: geo-location of the MWAB. - option 3: TAC/Cell ID of the cell serving the MWAB-UE in other PLMN. - The AMF may provide the received Additional ULI to LMF in LCS request. NOTE: If option 2 is selected in normative phase, support of passing Additional ULI to LMF needs to be reviewed.

In the last meeting, RAN3 discussed options 1, 2 and 3 provided in TR 23.700-06, clause 8.5, for this additional ULI. In this discussion, RAN3 was wondering how this additional ULI was used for LCS in case the WAB-gNB’s access PLMN is different than the backhaul PLMN. RAN3 was further wondering whether there were other purposes of this additional ULI.

In the last meeting, SA2 agreed on CRs to TS 23.501 and TS 23.502 in S2-2411113 and S2-2411114, respectively [2], which address some of RAN3’s concerns. The CR to TS 23.501 in S2-2411113 contains the following:

The AMF may consider the 'Additional ULI' when it determines UE location and manages the UE location related functions (e.g. Mobility Restrictions). Based on operator policy, the AMF may send the Additional ULI when the AMF provides user location information it has received over the N2 interface to a NF (e.g. the LMF as specified in clause 5.19.1 of TS 23.273 [87]).

Observation 7: SA2 decided that the additional ULI related to the WAB-MT serves for the support of LCS, establishment of mobility restrictions, and potentially other reasons, i.e., the same purposes as the additional ULI related to the mIAB-MT introduced for mobile IAB.

In case the WAB-gNB’s access and backhaul PLMNs are the same, the additional ULI related to the WAB-MT should include the same information as the additional ULI related to mIAB-MT introduced for mobile IAB.

Proposal 7b: In case the WAB-node’s access PLMN and backhaul PLMN are the same, the UE’s ULI to include the cell ID and TAC of MT’s served cell for the same reasons and in the same manner as defined for mobile IAB.

In case the WAB-gNB’s access and backhaul PLMNs are different, it needs to be understood how the additional ULI related to the WAB-MT is used. The following scenarios can be considered:

Scenario 1: the WAB-gNB’s access PLMN has other RAN node in vicinity of the WAB-node

Scenario 2: the WAB-gNB’s access PLMN has no other RAN nodes in vicinity of the WAB-node

In Scenario 1, the UE’s CN may use the information related to the WAB-MT’s physical location contained in this additional ULI to perform LCS and to provide mobility restrictions for this UE.

In Scenario 2, the UE’s CN can still perform LCS for the UE, but it can only use the TRP for the WAB-gNB. There is also no need for mobility restrictions since the UE cannot be handed over to any other RAN node of the same PLMN. However, SA2 does not preclude other purposes for the UE’s CN to use this additional ULI information.

While there may be no obvious benefit to include the WAB-MT’s additional ULI for Scenario 2, there is certainly benefit for Scenario 1.

Observation 8: When the WAB-node’s access PLMN and backhaul PLMN are different, the UE’s CN can make use of the WAB-MT’s additional ULI in case the WAB-gNB’s access PLMN has other RAN node in vicinity of the WAB-node.

When the WAB-gNB’s access and backhaul PLMNs are different, the WAB-MT’s served cell ID and TAC may or may not be meaningful to the backhaul CN. In case it is meaningful to the backhaul CN, it can be explicitly signalled to the UE’s CN following option 3 in SA2’s LS and as proposed in P7. How the UE’s CN interprets the WAB-MT’s served cell ID and TAC is up to implementation.

Observation 9: When the WAB-node’s access PLMN and backhaul PLMN are different and the WAB-MT’s served cell ID and TAC are meaningful to the UE’s CN, they can be explicitly signalled to the UE’s CN following option 3 in SA2’s LS.

In case the WAB-MT’s served cell ID and TAC is not meaningful to the UE’s CN, the WAB-gNB can perform a mapping to information that is meaningful to the UE’s CN. The mapped information can also represent the format of a cell ID and TAC. The mapping can be based on implementation, e.g., OAM configuration.

Observation 10: When the WAB-MT’s served cell ID and TAC are not meaningful to the UE’s CN, they can be mapped by the WAB-gNB to a cell ID and TAC that are meaningful to the UE’s CN in accordance with option 1 in SA2’s LS. The mapping can be left up to implementation.

SA2’s LS also proposed option 2, i.e., where the WAB-gNB includes the WAB-node’s geolocation into the UE’s ULI. This approach would imply that the WAB-gNB is capable to derive a geolocation for the WAB-node, and that the UE’s CN can interpret geolocation information. Options 1 and 3 do not have these requirements.

Observation 11: Including the WAB-node’s geolocation information would require that the WAB-node can obtain its geolocation information, and that the UE’s CN can interpret such geolocation information.

Based on the above discussion, RAN3 should recommend using options 1 and 3 provided by SA2, i.e., the WAB-gNB includes a cell ID and TAC related to the WAB-MT’s served cell. Whether this information is the explicit served cell/TAC information or a mapped cell/TAC information can remain up to implementation.

Proposal 7b: In case the WAB-node’s access PLMN and backhaul PLMN are the different, the UE’s ULI to also include a cell ID and TAC related to the MT’s served cell (SA’2 options 1 and 3). Whether this information represents the WAB-MT’s explicit or mapped cell/TAC mapped is up to implementation.

Proposal 8: RAN3 to not support including the WAB-MT’s geolocation information into the UE’s ULI (SA2’s option 2) since this requires additional enhancements on WAB-node and AMF.

## Draft Reply LS

Based on the above discussion and proposals, a draft Reply LS to SA2 has been provided in Annex 1.

Proposal 9: RAN3 to approve the draft Reply LS provided in the Annex 1.

## TP to BL CR of TS 38.401

Based on the above discussion and proposals, a TP to BL CR of TS 38.401 has been provided in Annex 2.

Proposal 10: RAN3 to approve the draft TP to BL CR of TS 38.401 in Annex 2.

# 3	Conclusion

This contribution discussed questions on issues raised by SA2’s LS related to multi-hop WAB and an additional ULI to be included into the UE’s ULI. The following observations and proposals have been made:

Issue 1a: WAB-MT connecting to non-collocated WAB-gNB

Observation 0: Preventing the formation of multi-hop WAB topologies implies that:

- the WAB-MT in RRC IDLE cannot (re)select cells of a WAB-gNB

- the WAB-MT in RRC CONNECTED is not handed over to a WAB-gNB

Observation 1: Solution 1, i.e., preventing multi-hop WAB topology based on the dedicated frequencies/PCIs for WAB-gNB cells, addresses most deployment scenarios and does not require stage-3 enhancements.

Observation 2a: In solution 2, the RRC-IDLE WAB-MT cannot know the gNB’s support for a specific slice ID, and it may therefore (re)select a WAB-gNB cell and perform an access attempt.

Observation 2b: In solution 2, the WAB-gNB can release the RRC connection of a WAB-MT after it has received the WAB-specific slice ID in MSG.5. However, there is no legacy mechanism to prevent the WAB-MT from conducting a new access attempt.

Observation 2c: In solution 2, when the BH RAN initiates an Xn handover request to a WAB-gNB cell, the WAB-gNB can reject the handover preparation using legacy behaviour and legacy cause value.

Observation 3a: In solution 3, the broadcast of “WAB-barred” by WAB-gNB cells can prevent WAB-MT to perform cell (re)selection of WAB-gNBs.

Observation 3b: In solution 3, to avoid handover to WAB-gNBs, the WAB-MT can exclude candidate cells that broadcast “WAB-barred” from the measurement reports, but this requires decoding of SIB1 for all candidate cells which is not be desirable.

Observation 4: In solution 4, the “WAB supported” broadcast by BH RAN provides the same functionality as the “WAB barred” broadcast by WAB-gNB, but it has stage-3 impact on the BH RAN, which is undesirable.

Observation 5: In solution 5, including a new WAB indication into the MT’s Xn handover request has the same functionality as provided by the WAB slice ID in the MT’s Xn handover request, but it has stage-3 impact, and this stage-3 impact affects the BH RAN.

Proposal 1: RAN3 to only support solutions 1, 2 and 3, and to down select solution 4 and 5.

Proposal 2: Capture on stage-2 that the formation of a multi-hop WAB-topology can be prevented by allocating dedicated frequencies and PCI values to WAB-gNB cells.

Proposal 3: Capture on stage-2 that to the formation of a prevent multi-hop WAB topology, the target WAB-gNB can reject handover requests based on the WAB-specific slice ID using legacy behaviour and signaling.

Proposal 4: Capture on stage-2 that to prevent formation of a multi-hop WAB topology, the WAB-gNB can broadcast a “WAB-barred” indicator in SIB, and the WAB-MT may avoid (re)selection of cells broadcasting this indicator.

Proposal 5: Capture solutions 1, 2 and 3 in the Reply LS to SA2 as a RAN-based remedy to issue-1a.

Issue 1b: WAB-MT connecting to collocated WAB-gNB

Observation 6a: Preventing a WAB-MT to (re)select the cells of the co-located WAB-gNB and being handed over to (Re)selection of WAB-gNB cells by WAB-MT and Xn handover of the WAB-MT .

Proposal 6a: Capture on stage-2 that the WAB-MT can be prevented to connect to its collocated WAB-gNB based on implementation, where the WAB-MT avoids (re)selecting cells of its collocated WAB-gNB and including these cells in measurement reports.

Proposal 6b: Capture P6 in the Reply LS to SA2 as a RAN-based remedy to issue-1b.

Proposal 6c: Send LS to RAN2 to request support of “WAB barred” in SIB1 for WAB-gNB cells.

Issue 2: Additional 2a

Observation 7: SA2 decided that the additional ULI related to the WAB-MT serves for the support of LCS, establishment of mobility restrictions, and potentially other reasons, i.e., the same purposes as the additional ULI related to the mIAB-MT introduced for mobile IAB.

Observation 8: When the WAB-node’s access PLMN and backhaul PLMN are different, the UE’s CN can make use of the WAB-MT’s additional ULI in case the WAB-gNB’s access PLMN has other RAN node in vicinity of the WAB-node.

Observation 9: When the WAB-node’s access PLMN and backhaul PLMN are different and the WAB-MT’s served cell ID and TAC are meaningful to the UE’s CN, they can be explicitly signalled to the UE’s CN following option 3 in SA2’s LS.

Observation 10: When the WAB-MT’s served cell ID and TAC are not meaningful to the UE’s CN, they can be mapped by the WAB-gNB to a cell ID and TAC that are meaningful to the UE’s CN in accordance with option 1 in SA2’s LS. The mapping can be left up to implementation.

Observation 11: Including the WAB-node’s geolocation information would require that the WAB-node can obtain its geolocation information, and that the UE’s CN can interpret such geolocation information.

Proposal 7a: In case the WAB-node’s access PLMN and backhaul PLMN are the same, the UE’s ULI to include the cell ID and TAC of MT’s served cell for the same reasons and in the same manner as defined for mobile IAB.

Proposal 7b: In case the WAB-node’s access PLMN and backhaul PLMN are the different, the UE’s ULI to also include a cell ID and TAC related to the MT’s served cell (SA’2 options 1 and 3). Whether this information represents the WAB-MT’s explicit or mapped cell/TAC mapped is up to implementation.

Proposal 8: RAN3 to not support including the WAB-MT’s geolocation information into the UE’s ULI (SA2’s option 2) since this requires additional enhancements on WAB-node and AMF.

Draft Reply LS

Observation 5: In solution 5, including a new WAB indication into the MT’s Xn handover request has the same functionality as provided by the WAB slice ID in the MT’s Xn handover request, but it has stage-3 impact, and this stage-3 impact affects the BH RAN.

Proposal 9: RAN3 to approve the draft Reply LS provided in the Annex 1.

TP to BL CR of TS 38.401

Proposal 10: RAN3 to approve the draft TP to BL CR of TS 38.401 in Annex 2.

# 4	References

[1] R3-244019 (S2-2407345): LS from SA2 on questions regarding FS_VMR_Ph2, TSG RAN WG3 Meeting #125, Maastricht, The Netherlands, August 224.

[2] Chair notes, TS RAN WG3 Meeting #125bis, Hefei, China, October 2024

# Annex 1: Draft Reply LS to SA2

3GPP TSG RAN WG3#126	R3-24xxxx

Orlando, Florida, November 18 – 22, 2024

Title:	[Draft] Reply LS on questions regarding FS_VMR_Ph2

Response to:	LS in R3-244019 (S2-2407345) on questions regarding FS_VMR_Ph2

Release:	Rel-19

Study Item:	FS_VMR_Ph2

Source:	Qualcomm [to be: RAN3]

To:	SA2

Cc:	RAN2

Contact person:

Name:	Georg Hampel

Tel. Number:

E-mail Address:	ghampel@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

RAN3 would like to thank SA2 for their LS on FS_VMR_Ph2 solution impacts to RAN (R3-244019/S2-2407345).

On issue 1:

SA2’s question:

SA2 has discussed the access control (to restrict MWAB-UE accessing MWAB-gNB cell) for below two cases:

a)	MWAB-UE accessing any MWAB-gNB in IDLE and CONNECTED mode (documented as one candidate solution in clause 6.2.3.6 of TS 23.700-06); and

b)	MWAB-UE accessing MWAB-gNB belonging to same MWAB.

SA2 assume that those scenarios will be discussed by RAN WGs, and ask for RAN feedback.

RAN3’s answer:

RAN3 observed that SA2’s CN-based solutions discussed in clause 6.2.3.6 of TS 23.700-06 cannot prevent the following scenarios:

- The WAB-MT in RRC IDLE (re)selects cells of a WAB-gNB.

- The WAB-MT in RRC CONNECTED is handed over via Xn to a WAB-gNB.

RAN3 believes that these scenarios should be addressed in addition to issues 1a and 1b raised by SA2. RAN3 identified the following RAN-based solutions to address issues 1a and 1b raised by SA2 and the above two scenarios identified by RAN3:

Solution 1: The WAB-gNB uses dedicated frequencies and/or PCI values, which are avoided by the WAB-MT for cell-reselection and measurement reports.

Solution 2: The WAB-gNB rejects handover requests that include the dedicated WAB slice ID agreed by SA2 in CR 5688 to TS 23.501 (S2-2411238).

Solution 3: WAB-gNB cell broadcasts a “WAB barred” indicator, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

RAN3 is planning to specify these solutions in collaboration with RAN2 as part of the WI on Additional Topological Enhancements.

On issue 2:

SA2’s question:

SA2 has discussed and concluded that Additional ULI format will be determined by RAN WG and SA2 will align based on RAN feedback. Please refer to related conclusion in clause 8.5 of TR 23.700-06.

RAN3’s answer:

RAN3 believes that option 3, i.e., including TAC/Cell ID of the WAB-MT’s served cell, needs to be supported in case the WAB-node’s access PLMN and backhaul PLMN are the same. In case the PLMNs are different, the WAB-gNB should also include a TAC/Cell ID that represents the location of the WAB-MT’s served cell. Whether this information is mapped (option 1) or explicit (option 3) can remain up to implementation. RAN3 does not see a reason to support option 2, i.e., including the WAB-node’s geolocation.

2. Actions:

To RAN2 group.

ACTION: 	RAN3 asks SA2 to take the above feedback into account.

3. Date of Next RAN3 Meetings:

TSG-RAN3 Meeting	 #126, November 18 to 22, 2024       Orlando, FL, USA

TSG-RAN3 Meeting	 #127, February 17 to 21, 2024       Athens, Greece

# Annex 2: TP for BL CR to 38.401

START OF CHANGE

## X.Y	WAB-node mobility

SKIP =========== >

### X.Y.Z+1	Preventing formation of a multi-hop WAB topology

A multi-hop WAB topology may form in case a WAB-MT connects to a WAB-gNB. This may occur under the following conditions:

- The WAB-MT (re)selects a cell of a WAB-gNB and performs connection establishment at this cell.

- The WAB-MT is handed over to a WAB-gNB.

The formation of a multi-hop WAB topology can be prevented in the following manner:

- The WAB-MT is configured to avoid (re)selection of cells with frequencies and/or PCI values that are only used by WAB-gNBs. The WAB-MT is also configured to avoid including measurements of these cells into measurement reports.

- The WAB-gNB is configured to reject handover requests that include the S-NSSAI configured for WAB as defined in TS 23.501 clause 5.49.

- WAB-gNBs are configured to broadcast a “WAB-barred” indicator in SIB1, and the WAB-MT is configured to avoid (re)selection of cells that broadcast this indicator.

These configurations are based on implementation.

END OF CHANGE