3GPP TSG RAN WG3 126	R3-247236

Orlando, USA, Nov 18th –22nd, 2024

Title: 	Further Enhancements on AIML support for CCO

Source: 	Jio

Agenda item:	11.3

Document Type:	Discussion and decision

# 1. Introduction

During last RAN3#125-bis meeting discussions on timing information follow:

Introducing separate time information for future coverage state and predicted CCO issue.

Time for future coverage state: The point in time when the future coverage state will be applied.

Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

The following question arises in the content:

How CU can deal with different time information generated in multiple DUs towards the same predicted CCO issue?

In this Discussion contribution, we provide further discussion related the aforementioned open issue to AI/ML based Coverage and Capacity Optimization (CCO). This document is a complement on the R3-247233 Discussion document,

# 2. Discussion

## gNB CU Future CCO state per Content

Following figure 2, gNB2 CU will receive a future CCO issue from gNB1 CU for the two different coverage holes which most probably will be related to different SSB beams. If the contents of the future CCO issue include only the related cells, gNB2 CU would have limited information on which SSB beams are affected from the future CCO issue. , It might be really difficult then for the gNB2 CU to interpret the reported issue and conclude on the concerned SSB beams, forcing it to go through exhaustive brute force algorithms to figure out, if possible, the corresponding SSB beams.

Figure 2: Example of two coverage holes in one neighbor cell CCO issue scenario

Observation 1: If the received future CCO issue, signalled via Xn interface to a neighbour gNB CU, does not include the SSB beams, the information is incomplete and vague forcing the gNB CU to undergo into complicated and processing consuming algorithms to estimate the related SSB beams.

Proposal 1: Any future CCO issue signalled via Xn interface to a single or group of neighbour gNB CUs should include among others the related cells and the corresponding SSB beams.

Following the legacy CCO, an AI/ML predicted CCO issue is always associated with a list of concerned neighbour cells and related group of SSB beams, which typically the gNB-CU communicates via F1 interface to the gNB-DU. It is then the most logical conclusion to extend this functionality to the aforementioned discussed scenario of receiving a future CCO from a neighbour cell. This is because sould the gNB CU send the received future CCO via F1 interface to the gNB DU with only the involved cells but without the corresponding SSB beams, would be again very difficult and somehow unclear on the SSB beams, forcing the DU to undergo through heavy and complicated brute force algorithms to conclude on the related SSB beams since it lacks the AI/ML intelligence.

Observation 2: If the gNB CU after receiving a future CCO issue, signalled via Xn interface from a neighbour gNB CU, concludes on a predicted CCO issue and forwards it to the gNB DU without including the SSB beams, the information would be incomplete and unclear forcing the gNB DU (due to lack of AI/ML inference and intelligence) to undergo into complicated and processing consuming algorithms to conclude on the related SSB beams.

Proposal 2: Any predicted CCO issue, concluded after receiving a future CCO issue signalled via Xn interface from a neighbour gNB CU, should include among others the related cells and the corresponding SSB beams when signalled via F1 interface to the gNB DU.

# 3. Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1: If the received future CCO issue, signalled via Xn interface to a neighbour gNB CU, does not include the SSB beams, the information is incomplete and vague forcing the gNB CU to undergo into complicated and processing consuming algorithms to estimate the related SSB beams.

Proposal 1: Any future CCO issue signalled via Xn interface to a single or group of neighbour gNB CUs should include among others the related cells and the corresponding SSB beams.

Observation 2: If the gNB CU after receiving a future CCO issue, signalled via Xn interface from a neighbour gNB CU, concludes on a predicted CCO issue and forwards it to the gNB DU without including the SSB beams, the information would be incomplete and unclear forcing the gNB DU (due to lack of AI/ML inference and intelligence) to undergo into complicated and processing consuming algorithms to conclude on the related SSB beams.

Proposal 2: Any predicted CCO issue, concluded after receiving a future CCO issue signalled via Xn interface from a neighbour gNB CU, should include among others the related cells and the corresponding SSB beams when signalled via F1 interface to the gNB DU.

# References

[1] 3GPP TR 38.743 V2.0.0 (2024-08)