3GPP TSG-RAN WG3 Meeting #126	R3-247240

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.1

Source:	Nokia

Title:	Recovery of N3mb failure for Multicast Session

Document for:	Discussion and Decision

# 1	Discussion

At RAN3#125, RAN3 agreed a RAN3 CR to address the N3mb failure case for the unicast transport of Broadcast service upon request from CT4. In the reply LS in R3-243888 RAN3 also hinted on some possible other recovery scenarios to address as follows:

- Recovery of N3mb path failure for unicast transport of multicast session,

- Recovery of N3mb path failure for multicast transport of broadcast session,

- Recovery for Uplink GTP-U Error Indication received by MB-UPF (loss of contexts in NG-RAN node without full RAN node failure/restart).

At this RAN3#126 meeting, RAN3 receives the LS from CT4 in C4-244467 informing that CT4 has agreed the attached 23.527 CR defining a new stage 2 restoration procedure in Rel-19 for the first scenario above.

More precisely, CT4 has agreed the attached 23.527 CR 0082 defining a new stage 2 restoration procedure in Rel-19 for recovery of N3mb path failure for unicast transport of multicast session. In this LS, CT4 asks RAN3 group to consider alignment to support the new Rel-19 restoration procedure.

The solution designed by CT4 is similar to the broadcast solution earlier agreed with a detection of the path failure in the MB-UPF, then MB-UPF informing MB-SMF of the detection of the failure with relevant IP address, then MB-SMF converting the IP address into a gNB ID and sending to AMF a notification of the failure including the gNB ID to route the message to the involved gNB.

In the new Multicast service case, the NGAP message reaching the gNB from AMF is however different, it is the NGAP Multicast Session Update Request. But the failure indication which contains the failed tunnel endpoints can therefore reuse the NGAP MBS NG-U Failure Indication IE which was already introduced for the broadcast case.

Proposal 1: agree the NGAP CR in [3] reusing the NGAP MBS NG-U Failure Indication IE and adding it into the Multicast Session Update Request Transfer IE of the Multicast Session Update Request message to indicate the failure to gNB over NGAP.

Another difference compared with the earlier Broadcast case is the action taken by the gNB upon receiving the failure notification. In CT4 tdoc C4-244212 which contains the CR 0082 for TS 23.527 the gNB does not provide the new tunnel endpoint in the response message because for multicast the dedicated procedure of Distribution Setup Request initiated by gNB is used. CT4 actually distinguishes two cases:

- Case 1: gNB-CU CP ask gNB-CU UP for a new tunnel endpoint and gNB-CU UP can provide one. This is the nominal scenario whereby the gNB-CU CP can further trigger the NGAP Distribution setup request to deliver this new tunnel endpoint to the MB-SMF.

- Case 2: gNB-CU CP cannot get a new tunnel endpoint from gNB-CU UP. The reason may be that the gNB-CU UP has total failure but the gNB-CU CP also fails to retrieve a new tunnel endpoint from another gNB-CU UP. Even though this scenario can be more seldom, in this case the gNB-CU CP should trigger the NGAP Distribution Release Request in order to avoid that the old tunnel endpoint keeps hanging. Indeed, the MB-SMF will not take the initiative to release that tunnel endpoint because it doesn’t know whether the interruption of path has self-recovered.

The above shows that it is important to well specify the actions that a gNB supporting this new feature should follow and especially the interaction expected with the Distribution setup request and Distribution release request procedure.

Proposal 2: it is important to clearly specify in the NGAP CR the interaction in gNB with the Distribution Setup Request and Distribution Release Request procedure.

Concerning the E1 aspects, the E1AP MC Bearer Context Modification Request message already enables the gNB-CU CP to request the gNB-CU UP for a tunnel endpoint, but it is specified when it does not yet exist. The restoration is a new use case which leads to new behavior whereby the gNB-CU CP should newly be able to request a replacement of an existing tunnel endpoint: this means that the gNB-CU UP receiving the request must consider that the current tunnel endpoint is implicitly released and the new tunnel endpoint is for replacement. We think that it is more appropriate to have a new Information Element for backwards compatibility. The E1AP CR is provided in [x].

Proposal 3: agree the E1AP CR in [4].

Finally, the TS 23.527 is currently not referenced in TS 38.300 and it is worth introducing such reference with a statement as a pointer to the restoration scenarios.

Proposal 4: agree the TS 38.300 CR in [5].

Proposal 5: agree the reply LS in [6].

# 2	Conclusion and Proposals

This paper has analysed the new restoration solution for unicast transport of multicast service designed by CT4 for release 19 and the foreseen impacts to NGAP specification. CT4 ask RAN3 to make the corresponding RAN3 alignment CRs and the following proposals result from our analysis:

Proposal 1: agree the NGAP CR in [3] reusing the NGAP MBS NG-U Failure Indication IE and adding it into the Multicast Session Update Request Transfer IE of the Multicast Session Update Request message to indicate the failure to gNB over NGAP.

Proposal 2: it is important to clearly specify in the NGAP CR the interaction in gNB with the Distribution Setup Request and Distribution Release Request procedure.

Proposal 3: agree the E1AP CR in [4].

Proposal 4: agree the TS 38.300 CR in [5].

Proposal 5: agree the Reply LS in [6].

# 3	References

- R3-243888, Restoration of N3mb Failure for MBS Broadcast

- C4-244467, LS on Multicast MBS Session Restoration procedure for N3mb path failure

- R3-247247, Recovery of N3mb path failure for unicast transport of multicast session, CR TS 38.413

- R3-247248, Recovery of N3mb path failure for unicast transport of multicast session, CR TS 37.483

- R3-247249, Recovery of N3mb path failure for unicast transport of multicast session, CR TS 38.300

- R3-247250, Reply LS on Multicast MBS Session Restoration procedure for N3mb path failure