3GPP TSG-RAN WG3 Meeting #126	             R3-247242

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:	17.3

Source:	CMCC

Title:	Support On-Demand SIB1 for UEs

Document for:	Discussions & Approval

# Introduction

In the RAN3 last # 125 Bis meeting, it is agreed [1]:

- The UL WUS configuration will be transferred over Xn via a new class1 defined procedure. - Cell A gNB can decide and signal to NES Cell gNB that it stops the UL WUS configuration broadcast in its SIB.

# Discussion on OD-SIB1 Mode

The following was discussed in RAN3 127 Bis meeting. There are three options were discussed in [4].

## Option 1

Option 1: After the UL WUS Configuration Provision request and Positive Response (confirm), NES Cell gNB goes to OD-SIB1 Mode. Cell A gNB broadcasts UL WUS configuration via its SIB.

XnAP: Cell A gNB “confirm” means that it will broadcast received UL WUS configuration via its SIB right away, Cell A gNB “reject” means it will not perform the request.

Figure 1 Option 1

In this option, UL WUS Configuration Provision Request from the NES Cell means that such request indicates the needs from the NES Cell to request the Cell A broadcast the UL WUS configuration. After the NES Cell receives the UL WUS Configuration Provision Response, if the response is “confirm”, the NES cell goes to OD-SIB1 Mode after a certain amount of time indicated in the ‘Timer”, which is responded from the Cell A for it to get ready and broadcast the UL WUS Configuration for the NES Cell.

The UL WUS Configuration Provision Request is sent from the NES Cell gNB to the Cell A gNB, including NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Start (UL WUS Configuration Broadcast), Pause (UL WUS Configuration Broadcast), Resume (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast).

- Start (UL WUS Configuration Broadcast): it means the NES Cell gNB wants the Cell A gNB to broadcast the UL WUS configuration for a list of NES Cells. Each NES Cell might be associated with a UL WUS configuration, time of period that the UL WUS configuration broadcast is requested by the NES Cell.

- Pause (UL WUS Configuration Broadcast): it means that the NES Cell gNB wants the Cell A gNB to temporally stop the UL WUS configuration broadcast for a list of NES Cells. Such pause might be associated with a time of period (timer) for each requested NES Cell. The Cell A gNB needs to maintain the UL WUS configuration and status for the list of requested NES Cells.

- Resume (UL WUS Configuration Broadcast): it means that the NES Cell gNB wants the Cell A gNB to resume the UL WUS configuration broadcast for a list of NES Cells that was paused before. Such resume might be associated with a time of period (timer) for each requested NES Cell.

- Cancel (UL WUS Configuration Broadcast): it means that the NES Cell gNB wants the Cell A gNB to completely cancel the UL WUS configuration broadcast for a list of NES Cells. The cancellation will remove the configuration and state that was maintained by the Cell A gNB for the list of NES Cells.

The UL WUS Configuration Provision Request is sent from the NES Cell gNB to the Cell A gNB, including NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Start (UL WUS Configuration Broadcast), Stop (UL WUS Configuration Broadcast), Resume (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast).

The action of “Start UL WUS Configuration Broadcast” requests the Cell A to broadcast the UL WUS Configuration for the requested NES Cell, which might be associated with a time of period that such request is effective.

The action of “Pause UL WUS Configuration Broadcast” requests the Cell A to pause the UL WUS configuration broadcast for the requested NES Cell, which might be associated with a time of period that such request is effective. The Cell A needs to maintain the configuration and status for the Cell A.

The action of “Resume UL WUS Configuration Broadcast” requests the Cell A to resume the UL WUS configuration broadcast for the requested NES Cell, which might be associated with a time of period that such request is effective and an updated UL WUS Configuration.

The action of “Cancel UL WUS Configuration Broadcast” requests the Cell A to completely cancel the UL WUS configuration broadcast for the requested NES Cell. The Cell A removes the configuration and status for the Cell A.

For the request that contains the action of “Start UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” or “Reject” response:

- The “Confirm” response could be associated with timer, which indicates the time of period that the NES Cell needs before the UL WUS configuration broadcast is effective. The NES Cell could use the timer to configure when to go to the OD SIB1 mode.

- The “Reject” response tells the NES Cell the requested Cell A refuses to broadcast the UL WUS configuration for the NES Cell.

For the request that contains the action of “Pause UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” response:

- The “Confirm” response confirms that the UL WUS configuration broadcast is paused at the Cell A.

For the request that contains the action of “Resume UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm” or “Reject” response:

- The “Confirm” response could be associated with timer, which indicates the time of period that the NES Cell needs before the UL WUS configuration broadcast is effective. The NES Cell could use the timer to configure when to go to the OD SIB1 mode.

- The “Reject” response tells the NES Cell the requested Cell A refuses to resume the UL WUS configuration broadcast for the NES Cell.

For the request that contains the action of “Cancel UL WUS Configuration Broadcast”, the Cell A gNB might return a “Confirm”:

- The “Confirm” response confirms that the UL WUS configuration broadcast is completely cancelled at the Cell A.

## Option 2

Option 2: After the UL WUS Configuration Provision request and positive response (confirm), Cell A gNB broadcast received UL WUS configuration via its SIB. NES cell goes to OD-SIB Mode upon reception of the explicit “UL config provided” message from Cell A gNB.

The reason is, it may take up to 41 seconds to broadcast UL WUS even Cell A broadcast immediately, NES cell can stay as legacy until it knows that Cell A has broadcast its UL WUS.

Compared to the Option 1, an additional message of “UL WUS Configuration Is Broadcast” is sent from the Cell A gNB to the NES Cell gNB in step 4 as shown in Figure 2. Upon receiving this message, the NES Cell enters the OD SIB1 mode.

Cell A gNB might return a “Confirm” response which means that it has the intention to broadcast UL WUS configuration for the Cell A, or return a “Reject” response which means that it will not perform the request.

The actions that were discussed in the Option 1 are also applicable for Option 2.

An additional message of “UL WUS Configuration State” is sent from the Cell A gNB to the NES Cell gNB, which might include the state of the UL WUS Configuration Broadcast, i.e. started, paused and resumed.

The “stated” UL WUS Configuration State is to tell the NES Cell to safely enter OD SIB1 mode, which could be associated with the list of NES Cells that the UL WUS Configuration is broadcast for.

The “paused” UL WUS Configuration State is to tell the NES Cell that the Cell A initiates the pausing of UL WUS configuration broadcast, which could be associated with the list of NES Cells that the UL WUS Configuration is paused.

The “resumed” UL WUS Configuration State is to tell the NES Cell that the Cell A initiates the continuation of UL WUS configuration broadcast, which could be associated with the list of NES Cells that the UL WUS Configuration is resumed.

Figure 2 Option 2

## Option 3

Option 3: After the UL WUS Configuration Provision request and positive response (confirm), no change in NES Cell or Cell A gNBs.

The Request message is to send the UL WUS Configuration.

The positive Response is to indicate that Cell A has received the UL WUS configuration and has intension to broadcast it (but it will not do it yet.)

The second message is sent from NES Cell gNB to Cell A gNB to indicate it is time to broadcast UL WUS configuration. And NES cell goes to OD-SIB Mode.

If there are multiple Cell A that accept the UL WUS Configuration Provision Request from the NES Cell, this option gives flexibility to the NES Cell gNB to decide the timing for the Cell A to broadcast the UL WUS configuration for it.  The NES Cell could be the SCell for multiple UEs, depending on the PCell that active UEs connecting to, the NES Cell could switch to a different Cell A and request the Cell A to broadcast the UL WUS Configuration for it.

The Cell A as PCell could initiate the UL WUS Configuration broadcast for a NES Cell as a SCell for a UE if the Cell A accepts the UL WUS Configuration Provision Request from the NES Cell.

In Option 3, RAN3 needs to discuss whether an additional step is needed for the Cell A as PCell to notify the NES Cell as a SCell for a UE that the UE is connected to the PCell, such that the NES Cell would indicate the Cell A to broadcast UL WUS Configuration for it.

Figure 3 Option 3

## Split Architecture

If the CU/DU split architecture is adopted in gNB, a NES Cell and Cell A could be in different DU(s). In order to transit the UL WUS Configuration, both F1 interface and Xn interface could be impacted.

- The NES Cell and Cell A are on the same gNB, but on different DU(s). gNB-CU is in the middle to forward the UL WUS configuration between two gNB-DUs on the same gNB.

- The NES Cell and Cell A are on different gNB(s). Two gNB-CU(s) are in the middle to forward the UL WUS Configuration between two gNB-DUs on different gNB(s).

The procedures and messages that are discussed above are also applicable to F1 interface.

All procedures and messages are applicable to F1 interface when the NES Cell and Cell A are on the same gNB, but on different DU(s); or the NES Cell and Cell A are on different gNB(s).

## UL WUS Configuration Details

For discussion on specific content for on-demand SIB1, the following agreements are agreed during RAN1#118bis meeting [3] and RAN1#117 meeting [2]:

Agreement in RAN1#118bis: For the purpose of “to which cell does the configuration applies”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. To which cell does the configuration applies; NES-CellId; PhysCellId. ARFCN-ValueNR. Note: ARFCN-ValueNR is used to indicate the absolute radio frequency channel number (ARFCN) for SSB of NES cell. Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration: - rsrp-ThresholdSSB - prach-RootSequenceIndex - msg1-SubcarrierSpacing - restrictedSetConfig Note: In legacy spec, the parameters above are under the IE RACH-ConfigCommon Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters to indicate “frequency information for UL-WUS transmission” are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz (working assumption)ULSubCarrierSpacing. Agreement in RAN1#118bis: For the purpose of “WUS transmission”, at least the following parameters are included in the UL-WUS configuration. Purpose; Parameters. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. SSB-positionInBurst. tdd-UL-DL-ConfigurationCommon. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResources; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. Note: - In legacy spec, ss-PBCH-BlockPower/SSB-positionInBurst/tdd-UL-DL-ConfigurationCommon are under the IE ServingCellConfigCommon/ServingCellConfigCommonSIB - Msg1-FrequencyStart is with respect to the first RB of the UE carrier determined by offsetToCarrier and absoluteFrequencyPointA Agreement in RAN1#117: For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

From RAN3’s perspective, a new IE to contain the UL WUS configuration is needed. Based on the above agreements, the following sub-elements shall be included in the new IE:

- NES-CellId: it indicates to which cell does the WUS configuration apply to.

- WUS transmission: it contains the configuration of WUS transmission of the NES Cell.

- SIB1 reception: it is the configuration of SIB1 reception, which is used to instruct the UEs where to receive the SIB1 from the Nes Cell.

- RAR Reception: the configuration of RAR reception, which is used to instruct UEs where and how to receive the RAR from the NES Cell. RAR reception allows the UE to synchronize its timing with the network and proceed with the connection setup or re-establishment process.

A new IE that contains UL WUS configuration of the NES Cell needs to be defined, which includes NES-CellId, WUS Transmission, SIB1 Reception, RAR Reception. The details are still upon RAN1’s progress.

## Cell A Capability Notification

To help the NES Cell gNB to make a decision on which neighbouring cells are suitable to be Cell A and broadcast UL WUS configuration for the NES Cell, the Cell A capability of the neighbouring cells could be notified to the NES Cell gNB.

Cell A capability of neighbouring cells could be indicated from one gNB to another gNB, or from one gNB-CU to another gNB-CU, or/and from one gNB-DU to its CU, which could have Xn and F1 interface impact.

## Decision of NES Cell

This subsection mainly focuses on the split architecture.  The CU manages signalling between the network and the UE, including connection setup, release, mobility management, and security functions. And it has more global picture of the neighbouring cell deployment, inference level, signal strength and quality, energy consumption, power status, etc.  The CU can be helpful by providing information before the DU initiates the on-demand SIB1 operation for some NES cells. For instance, the CU could supply a list of cells where the on-demand SIB1 operation should be enabled.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled, and a list of corresponding cells could be requested as Cell A) before a DU initiates the OD SIB1 operation.

# Conclusions

In this submission, we discussed the procedures of the OD-SIB1 Case 2 that is agreed to be focused on in RAN 3. The following proposals are presented in the paper:

- The UL WUS Configuration Provision Request is sent from the NES Cell gNB to the Cell A gNB, including NES Cell List, corresponding UL WUS configuration for each NES Cell, the action associated with the NES cell, i.e., Start (UL WUS Configuration Broadcast), Stop (UL WUS Configuration Broadcast), Resume (UL WUS Configuration Broadcast), Cancel (UL WUS Configuration Broadcast).

The action of “Start UL WUS Configuration Broadcast” requests the Cell A to broadcast the UL WUS Configuration for the requested NES Cell, which might be associated with a time of period that such request is effective.

The action of “Pause UL WUS Configuration Broadcast” requests the Cell A to pause the UL WUS configuration broadcast for the requested NES Cell, which might be associated with a time of period that such request is effective. The Cell A needs to maintain the configuration and status for the Cell A.

The action of “Resume UL WUS Configuration Broadcast” requests the Cell A to resume the UL WUS configuration broadcast for the requested NES Cell, which might be associated with a time of period that such request is effective and an updated UL WUS Configuration.

The action of “Cancel UL WUS Configuration Broadcast” requests the Cell A to completely cancel the UL WUS configuration broadcast for the requested NES Cell. The Cell A removes the configuration and status for the Cell A.

An additional message of “UL WUS Configuration State” is sent from the Cell A gNB to the NES Cell gNB, which might include the state of the UL WUS Configuration Broadcast, i.e. started, paused and resumed.

The “stated” UL WUS Configuration State is to tell the NES Cell to safely enter OD SIB1 mode, which could be associated with the list of NES Cells that the UL WUS Configuration is broadcast for.

The “paused” UL WUS Configuration State is to tell the NES Cell that the Cell A initiates the pausing of UL WUS configuration broadcast, which could be associated with the list of NES Cells that the UL WUS Configuration is paused.

The “resumed” UL WUS Configuration State is to tell the NES Cell that the Cell A initiates the continuation of UL WUS configuration broadcast, which could be associated with the list of NES Cells that the UL WUS Configuration is resumed.

The Cell A as PCell could initiate the UL WUS Configuration broadcast for a NES Cell as a SCell for a UE if the Cell A accepts the UL WUS Configuration Provision Request from the NES Cell.

In Option 3, RAN3 needs to discuss whether an additional step is needed for the Cell A as PCell to notify the NES Cell as a SCell for a UE that the UE is connected to the PCell, such that the NES Cell would indicate the Cell A to broadcast UL WUS Configuration for it.

All procedures and messages are applicable to F1 interface when the NES Cell and Cell A are on the same gNB, but on different DU(s); or the NES Cell and Cell A are on different gNB(s).

A new IE that contains UL WUS configuration of the NES Cell needs to be defined, which includes NES-CellId, WUS Transmission, SIB1 Reception, RAR Reception. The details are still upon RAN1’s progress.

Cell A capability of neighbouring cells could be indicated from one gNB to another gNB, or from one gNB-CU to another gNB-CU, or/and from one gNB-DU to its CU, which could have Xn and F1 interface impact.

The CU of the NES Cell could provide some assistance information (a list of cells where the on-demand SIB1 operation should be enabled, and a list of corresponding cells could be requested as Cell A) before a DU initiates the OD SIB1 operation.

# Reference

- draft_MeetingReport_RAN3#125-bis_vOct25

- Draft_Minutes_report_RAN1#117_v030

- Draft_Minutes_report_RAN1#118b_v010

- Draft-R3-245737_V2