3GPP TSG-RAN WG3 Meeting #126	R3-247251

Orlando, US, 18 - 22 Nov, 2024

Title:	Consideration on Multicast MBS session restoration procedure for N3mb path failure

Agenda Item:	8.1

Source:	Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated

Document for:	Discussion

# Introduction

In previous RAN3 meetings, the support of Restoration of N3mb Failure for MBS Broadcast was discussed and supported by RAN3, and in the LS from RAN3 to CT4, RAN3 further asked several scenarios, including:

- Recovery of N3mb path failure for unicast transport of multicast session,

- Recovery of N3mb path failure for multicast transport of broadcast session,

- Recovery for Uplink GTP-U Error Indication received by MB-UPF (loss of contexts in NG-RAN node without full RAN node failure/restart).

Based on the CT4 replied LS C4-244467, CT4 further agreed a CR to support the first scenario above.

In this paper, we further discuss the RAN3 impact for the first scenario.

# 2	Discussion

For unicast transport of multicast session, in case of N3mb path failure, as there is only DL NG-U tunnel established, the gNB is not able to know it.

In the attached CR from CT4 in the LS, the agreed solution is:

The MB-SMF can notify the N3mb path failure to the NG-RAN, to enable the NG-RAN to assign a new DL F-TEID for N3mb.

And it is noticed that in the “other comments” part within the CT4 CR cover page, it is said that: “The CR requires NGAP to be enhanced to support signaling a UP Path Failure in the NGAP Multicast Session Update Request message.”

Observation 1: in CT4 agreed CR, the MB-SMF informs the N3mb path failure indication to the gNB in Multicast Session Update Request, and the RAN tiggers another Distribution Setup procedure towards the CN to re-establish the shared NG-U tunnel.

In case of Broadcast N3mb path failure, different handling was agreed upon receiving the N3mb failure indication, i.e., upon receiving the N3mb failure indication in the BROADCAST SESSION MODIFICATION REQUEST message, the gNB replies the CN a new DL TNL information in the RESPONSE message.

Observation 2: in broadcast N3mb path failure, upon receiving the N3mb failure indication in the BROADCAST SESSION MODIFICATION REQUEST message, the gNB replies the CN a new DL TNL information in the RESPONSE message.

From RAN3 point of view, there are two possible approaches can be used to support N3mb path failure for multicast:

- Option 1: align with CT4 LS, by only adding N3mb path failure indication in the Multicast Session Update Request Transfer IE.

- Option 2: align with Broadcast solution, not only adding N3mb path failure indication in the Multicast Session Update Request Transfer IE, but also introducing an Multicast Session Update Response Transfer IE to provide new DL TNL information.

Considering that this restoration scenario will not happen frequently, it is slightly prfered to adopt Option1 to align with CT4 reply LS.

Proposal 1: align with CT4 LS, in NGAP, adding N3mb path failure indication in the Multicast Session Update Request Transfer IE.

On the other hand, it is noticed that in 8.17.2.2, the following was added for broadcast:

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", …

But the correct codepoint should be “NG-U path failure detected”.

Proposal 2: in NGAP, correct the description of the codepoint in 8.17.2.2, from “N3mb path failure” to “NG-U path failure detected”.

On the other hand, in case of CP UP separation, upon receiving the N3mb path failure indication from 5GC, the gNB-CU-CP should provide such information to the gNB-CU-UP, to trigger the gNB-CU-UP to allocate new unicast transport resources accordingly.

In current E1AP, the gNB-CU-CP is able to request the gNB-CU-UP to allocate DL TNL information:

If the MC Bearer Context NG-U TNL Info at NG-RAN Request IE is contained in the MC BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the MC Bearer Context NG-U TNL Info at NG-RAN Modify Response IE in the MC BEARER CONTEXT MODIFICATION RESPONSE message.

But this approach currently only can be used in case the TNL information is not yet available at the gNB-CU-CP, we need to update E1AP specification to extend the scenario of using MC Bearer Context NG-U TNL Info at NG-RAN Request IE, and clarify the UP handling accordingly.

#### 9.3.3.34	MC Bearer Context To Modify

This IE contains MBS session resource related information used to request a modification of a multicast MC Bearer Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.122 |  | - |  |
| MC Bearer Context NG-U TNL Info at NG-RAN Request | O |  | 9.3.1.123 | To request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP | - |  |
| MBS Multicast F1-U Context Descriptor | C-ifSetupOrRemove |  | 9.3.1.125 |  | - |  |

Proposal 3: in case of CP UP separation, in E1AP: extend the scenario of using MC Bearer Context NG-U TNL Info at NG-RAN Request IE, and clarify the gNB-UP handling accordingly.

# 3	Conclusion

In this contribution, we discussed the Multicast MBS session restoration procedure for N3mb path failure topic, and get the following proposals:

Proposal 1: align with CT4 LS, by adding N3mb path failure indication in the NGAP: Multicast Session Update Request Transfer IE.

Proposal 2: in NGAP, correct the description of the codepoint in 8.17.2.2, from “N3mb path failure” to “NG-U path failure detected”.

Proposal 3: in case of CP UP separation, in E1AP: extend the scenario of using MC Bearer Context NG-U TNL Info at NG-RAN Request IE, and clarify the gNB-UP handling accordingly.

The corresponding CRs are provided in [1] and [2], and the Draft Reply LS is provided in [3].

# 4	Reference

[1] R3-247252 Support of Multicast MBS session restoration procedure for N3mb path failure, NGAP CR, Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated

[2] R3-247253 Support of Multicast MBS session restoration procedure for N3mb path failure, E1AP CR, Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated

[3] R3-247254 [Draft] Reply LS on Multicast MBS session restoration procedure for N3mb path failure (Reply to C4-244467), Huawei