3GPP TSG-RAN WG3 Meeting #126                                                                    R3-247271

Orlando, USA, November 18th– 22nd, 2024

Source:	CATT

Title:	Impacts to NGAP for NR Femto

Agenda Item:	12.3

Document for:	Discussion and Decision

1 Introduction

In this contribution, we will discuss the impacts to NGAP for NR Femto.

2 Discussion

In the TP [1], the NR Femto logical architecture has been shown. Some NR Femto nodes connect to the 5GC directly as a gNB by means of the NG interface, other NR Femto nodes connect to the 5GC though the NR Femto Gateway (NR Femto GW). It appears that the introduction of the 5G Femto architecture does not affect the signaling of the NG setup procedure between the NR Femto node and the NR Femto GW.

The NR Femto GW can connect to multiple AMFs. AMFs and Femto nodes transmit NG signal via the NR Femto GW which they connect with. The AMF needs the NR Femto GW to transfer NGAP signal of a specific UE to NR Femto, and the Femto nodes also need the GW to transfer NGAP signal to AMF. Therefore, how to find path is an issue that needs to be solved in 5G Femto.

In LTE, HeNB is specified as in TS 36.300 [2]. The architecture of the 5G Femto node deployment is similar with the architecture of HeNB specified in TS 36.300 section 4.6.1. In TS 36.300, as GUMMEI IE is the globally unique MME identity, upon reception of the GUMMEI from a UE, the HeNB shall include it in the INITIAL UE MESSAGE message and send the message to HeNB GW. The HeNB GW selects the MME based on the GUMMEI in the message, then routes the received message to the selected MME.

In 5G, GUAMI IE includes AMF Region ID IE, AMF Set ID IE and AMF Pointer IE, which can uniquely identify an AMF [3]. Thus, selection of an AMF is hosted by the NR Femto GW instead of the NR Femto node. Upon reception of the GUAMI from a UE, the NR Femto node shall include it in the INITIAL UE MESSAGE message. It is proposed to add GUAMI IE in the INITIAL UE MESSAGE message.

Proposal 1: Add GUAMI IE in the INITIAL UE MESSAGE message for NR Femto GW to select AMF.

HeNB Signals the GUMMEI of the source MME to the HeNB GW in the S1 PATH SWITCH REQUEST message. The HeNB GW routes the S1 PATH SWITCH REQUEST message towards the MME based on the GUMMEI of the source MME received from the HeNB. Similar to the HeNB, the NR Femto node needs a function to indicate the source AMF to the NR Femto GW in the PATH SWITCH REQUEST message, so as to rout the PATH SWITCH REQUEST message towards the AMF indicated by the NR Femto node. In LTE, the HeNB signals the GUMMEI of the Source MME to the HeNB GW in the S1 PATH SWITCH REQUEST message via the Source MME GUMMEI IE.

For NR Femto GW to rout the PATH SWITCH REQUEST message to AMF, the NR Femto node can signal the GUAMI of the source AMF to the NR Femto GW in the PATH SWITCH REQUEST message. Source AMF GUAMI IE can be added in the PATH SWITCH REQUEST message.

Proposal 2: Add Source AMF GUAMI IE in the PATH SWITCH REQUEST message to indicate the GUAMI of the source AMF to the NR Femto GW.

In TS 36.413 [4], the MME UE S1AP ID 2 IE is used to indicate the MME UE S1AP ID assigned by the MME to HeNB. The information element is included in the INITIAL CONTEXT SETUP REQUEST message, the HANDOVER REQUEST message, and the PATH SWITCH REQUEST ACKNOWLEDGE message. If HeNB architecture uses those messages, that IE will be mandatory. And it is used for subsequent X2 handovers from the HeNB.

Since there is no existing IE for subsequent Xn handover handover of UE from the Femto node to other NG-RAN nodes in TS 38.413 [3], a new IE needs to be added to the corresponding messages for subsequent Xn for the similar functionality.

Observation: INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message should introduce a new IE to indicate the AMF UE NGAP ID allocated by AMF for subsequent Xn handover.

Since the MME UE S1AP ID IE is similar with the AMF UE NGAP ID IE in TS 38.413 [3], and the new IE indicates the AMF UE NGAP ID assigned by the AMF, the new IE in stage-3 spec can be named AMF UE NGAP ID 2 IE, which is added into the INITIAL CONTEXT SETUP REQUEST message, the HANDOVER REQUEST message, and the PATH SWITCH REQUEST ACKNOWLEDGE message for subsequent Xn handovers.

Proposal 3: Introduce AMF UE NGAP ID 2 IE, to the INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message to indicate the AMF UE NGAP ID assigned by the AMF.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: Add GUAMI IE in the INITIAL UE MESSAGE message for NR Femto GW to select AMF.

Proposal 2: Add Source AMF GUAMI IE in the PATH SWITCH REQUEST message to indicate the GUAMI of the source AMF to the NR Femto GW.

Observation: INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message should introduce a new IE to indicate the AMF UE NGAP ID allocated by AMF for subsequent Xn handover.

Proposal 3: Introduce AMF UE NGAP ID 2 IE, to the INITIAL CONTEXT SETUP REQUEST message, HANDOVER REQUEST message and PATH SWITCH REQUEST ACKNOWLEDGE message to indicate the AMF UE NGAP ID assigned by the AMF.

4 Reference

- R3-245754, TP for BL CR on Introduction of NR Femto Architecture and Protocol Aspects, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, RAN3#125bis

- 3GPP TS 36.300, Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 18), V18.2.0 (2024-06)

- 3GPP TS 38.413, NG-RAN, NG Application Protocol (NGAP), V18.2.0 (2024-06)

- 3GPP TS 36.413, Evolved Universal Terrestrial Radio Access Network(E-UTRAN); S1 Application Protocol (S1AP) (Release 18), V18.2.0 (2024-06)