3GPP TSG RAN WG3 #126		R3-247308

Orlando, US, Nov 18th – 22th, 2024

Title:	(TP for SON BL CR for TS 38.473) Discussion on LTM MRO

Agenda item:	10.2

Source:	Google

Document for:	Discussion and Decision

# Introduction

In RAN3#125bis meeting, there were some agreements and FFSs for LTM MRO.

MRO for LTM:

Add failure type to F1AP ACCESS AND MOBILITY INDICATION message.

CU does not forward the RLF Report to DU if failure is due to wrong selection of prepared LTM cell.

What additional information need to be added to F1AP Access and Mobility Indication message?

- C-RNTI?

RAN3#123bis meeting, it was agreed to study the near failure LTM:

Work on scenarios of near failure LTM

In this paper, we further discuss the remaining issues of LTM MRO and give our considerations.

# Discussion

## LTM RLF report

### Delivery of RLF report

According to the agreements in RAN3#125bis meeting, it was agreed that CU performs the initial analysis. During the initial analysis phase, the CU will determine whether the failure is due to wrong selection of prepared LTM cell and the failure type. The failure types can be too late LTM, too early LTM or LTM to wrong cell.

If the failure is not due to wrong selection of prepared LTM cell, the CU forward the RLF report to DU for root cause analysis. Before the forwarding of RLF report, the CU firstly need to decide the DU responsible for the failure. One possible solution is based on the failure type. For the too late LTM, the CU forwards the RLF report to the failure DU controlling the cell where the sub-optimal LTM configuration resulted in the RLF. For too early LTM or LTM to wrong cell, the CU at least forwards the RLF report to the source DU, which inappropriately initiated the LTM cell switch procedure. If there is random access information and the connection failure type is set to HOF in the RLF report, the CU can further delivery the RLF report to the target DU.

Proposal 1: CU forwards RLF report to the correct DU based on LTM failure type:

- The failure DU in case of too late LTM

- The source DU in case of too early LTM, or LTM to wrong cell

- The target DU if random access information exists in RLF report for HOF case

The receiving DU can then perform a root cause analysis based on the RLF report and the LTM failure type. By addressing the issues, the DU can optimize LTM related parameters and enhance the robustness of LTM.

Proposal 2: DU performs root cause analysis to optimize the LTM related parameters.

### Additional information

To analyze the potential additional information, we prefer to classify the LTM MRO into two cases. In case 1, the UE provides the RLF report. In case 2, the UE may not provide the RLF report. We will give our considerations on the additional information provided from CU to DU for the two cases.

Case 1: MRO with RLF report

During online discussion in RAN3#125bis meeting, it was suggested to check the C-RNTI information provided in the RLF report. The related descriptions are quoted from the TS 38.331 [1].

RLF-Report-r16 ::=                   CHOICE {

nr-RLF-Report-r16                    SEQUENCE {

measResultLastServCell-r16           MeasResultRLFNR-r16,

measResultNeighCells-r16             SEQUENCE {

measResultListNR-r16                 MeasResultList2NR-r16       OPTIONAL,

measResultListEUTRA-r16              MeasResultList2EUTRA-r16    OPTIONAL

}                                                OPTIONAL,

c-RNTI-r16                           RNTI-Value,

previousPCellId-r16                  CHOICE {

nrPreviousCell-r16                   CGI-Info-Logging-r16,

eutraPreviousCell-r16                CGI-InfoEUTRALogging

}                                                                    OPTIONAL,

failedPCellId-r16                    CHOICE {

nrFailedPCellId-r16                  CHOICE {

cellGlobalId-r16                     CGI-Info-Logging-r16,

pci-arfcn-r16                        PCI-ARFCN-NR-r16

},

eutraFailedPCellId-r16           CHOICE {

cellGlobalId-r16                 CGI-InfoEUTRALogging,

pci-arfcn-r16                    PCI-ARFCN-EUTRA-r16

}

},

reconnectCellId-r16                  CHOICE {

nrReconnectCellId-r16                CGI-Info-Logging-r16,

eutraReconnectCellId-r16             CGI-InfoEUTRALogging

}                                                                                        OPTIONAL,

RLF-Report field descriptions

[……]

c-RNTI This field indicates the C-RNTI used in the PCell upon detecting radio link failure or the C-RNTI used in the source PCell upon handover failure.

It is noticed that there is only one C-RNTI provided in the RLF report. The C-RNTI will indicate different information depending on the connection failure type. For HOF case, the UE provides the source C-RNTI used in source PCell where the UE was connected before the HOF. For RLF case, the UE provides the C-RNTI used in the PCell upon detecting RLF. For example, the UE provides the C-RNTI used in the failure cell (aka source cell) upon detecting RLF in case of too late LTM. The UE provides the target C-RNTI used in the target cell upon detecting RLF in case of too early LTM or LTM to wrong cell.

Observation 1: Only one C-RNTI is provided in the RLF report.

Observation 2: The C-RNTI will be set to:

- Source C-RNTI for:

  - RLF in case of too late LTM; or

  - HOF in case of too early LTM or LTM to wrong cell

- target C-RNTI for RLF in case of too early LTM or LTM to wrong cell

In the agreed TP [2] on TS 38.473, the CU can forward the RLF Report Failure Type and source C-RNTI to the DU.

9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |

As analyzed in the above section, it is worth noting that the UE will provide the source C-RNTI in RLF report for too late LTM, or HOF in case of too early LTM/LTM to wrong cell. Therefore, sending this information separately over the F1 interface to the source DU would be redundant.

However, for the RLF in case of too early LTM or LTM to wrong cell, there is only target C-RNTI in the RLF report. This means the source DU, which initiated the LTM switch, lacks the source C-RNTI to effectively identify the UE context and analyze the failure.

To address the UE context retrieval at source DU and optimize information exchange, it is recommended for the CU to provide the source C-RNTI when the source C-RNTI is not provided in the RLF report. Correspondingly, new conditions to include the source C-RNTI as additional information should be introduced to address the related case.

Proposal 3: The CU forwards the source C-RNTI to the source DU:

- if the connection Failure type IE in RLF report is set to “RLF”; and

- if RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”.

As mentioned in proposal 1, the CU will forward the RLF report to target DU in case of random access information existing for HOF case. According to the descriptions in [1], the UE will provide the source C-RNTI in RLF report for HOF case.

To assist the target DU to identify the UE and retrieve UE context at target DU, it is beneficial for CU to provide target C-RNTI alongside RLF report to the target DU.

Proposal 4: The CU forwards the target C-RNTI to the target DU:

- if the connection Failure type IE in RLF report is set to “HOF”; and

- if there is random access information in RLF report.

Case 2: Reestablishment without RLF Report

In LTM MRO, there is the reestablishment case without RLF report.

During LTM procedure, UE detects connection failure with the source or target LTM cell belonging to the CU. The UE behaviors are described in the TS 38.331 [1]:

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

1>	else:

[……]

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

It is worth noting that the UE performs LTM recovery with the selected cell if the selected cell is one of the LTM candidate cells and the attemptLTM-Switch is configured. The field attemptLTM-Switch is optional present for LTM candidate configuration. If the field attemptLTM-Switch is absent, the UE falls back to legacy reestablishment procedure with the selected LTM candidate cell.

Observation 3: If the LTM attempt is not configured, the UE may perform legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

For Rel-18 LTM, it is introduced for intra-CU case. The source cell and all the candidate cells belong to the same CU. When the UE attempts to re-establish the radio link connection with the selected candidate cell, the UE may not provide the RLF report to the CU. Despite the lack of RLF report, the CU can still perform initial analysis, e.g., based on the information gathered in the reestablishment attempt, including failure cell PCI, failure C-RNTI and reestablishment cell CGI. After initial analysis, if the CU determines that the LTM failure is not due to inappropriate LTM candidate cell preparation at CU side, it forwards the failure related additional information to the source DU.

Proposal 5: For reestablishment without RLF report, the CU should forward additional information to source DU if the failure is not due to wrong selection of prepared LTM cell.

For reestablishment case without RLF report in existing MRO scheme, the node receiving the reestablishment request may initiate a failure indication to the failure node where the connection failure was detected. Correspondingly, the failure node performs UE verification and perform a thorough analysis of the failure.

The failure indication for reestablishment without RLF report defined in TS38.423 [3] can include the failure information as shown below:

9.1.3.16	FAILURE INDICATION

This message is sent by NG-RAN node2 to indicate an RRC re-establishment attempt or a reception of an RLF Report from a UE that suffered a connection failure at NG-RAN node1.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| CHOICE Initiating condition | M |  |  |  | YES | reject |
| >RRC Reestab |  |  |  |  |  |  |
| >>CHOICE RRC Reestab Initiated Reporting | M |  |  |  | – |  |
| >>>RRC Reestab Reporting without RLF Report |  |  |  |  |  |  |
| >>>>Failure cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | Physical Cell Identifier | – |  |
| >>>>Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | – |  |
| >>>>C-RNTI | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the c-RNTI contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>ShortMAC-I | M |  | BIT STRING (SIZE (16)) | Corresponds to information provided in the shortMAC-I contained either in the RRCReestablishmentRequest message (TS 38.331 [10]) or in the RRCConnectionReestablishmentRequest message (TS 36.331 [14]) | – |  |
| >>>>RRC Conn Reestab Indicator | O |  | ENUMERATED (reconfigurationFailure, handoverFailure, otherFailure, ...) | Corresponds to information provided in the reestablishmentCause contained in the RRCReestablishmentRequest message as defined in TS 38.331 [10] or in the RRCConnectionReestablishmentRequest message as defined in TS 36.331 [14]. | YES | ignore |

Therefore, for reestablishment case without RLF report for LTM MRO, when the CU receives reestablishment attempt from the UE, it first verifies the UE and determines the failure type. If needed, the CU can forward the source cell, target cell, reestablishment cell, source C-RNTI and LTM failure type to the source DU for root cause analysis to optimize the LTM related parameters.

Proposal 6: For reestablishment without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment cell information

- LTM failure type

### LTM recovery without RLF Report

For CHO and LTM mobility, when the UE detects connection failure and selects a CHO or LTM candidate cell with attempt configured, the UE can perform CHO or LTM recovery with the selected candidate cell.

For CHO, the UE decides the CHO target cell if the CHO conditions are satisfied and executes CHO with the target cell. This is UE-based decision. If connection failure occurs, the UE selects a CHO cell and initiates CHO recovery. The UE sends the RRC Reconfiguration Complete message to the selected candidate cell. If the UE does not provide the RLF report to the CHO recovery cell, the CHO recovery cell is able to identify whether the access is a normal CHO execution or a CHO recovery from CHO failure. Consequently, the CHO recovery cell will not initiate a failure indication to the failure RAN node where the connection failure was detected.

Observation 4: Without RLF report, the RAN node where the CHO recovery is performed will not identify whether there was a connection failure during CHO procedure.

The situation changes for LTM. Although the CU configures the candidate LTM cells to UE, the LTM cell switch decision is still initiated by the source DU based on the L1 measurement results. This is NW-driven decision. The source DU selects the target LTM cell and informs the CU of the target LTM cell information via DU-CU CELL SWITCH NOTIFICATION message. After a successful LTM cell switch, the target LTM cell sends an ACCESS SUCCESS message to CU, indicating the (first) access cell where the UE has successfully accessed for LTM. If LTM procedure fails and the UE performs LTM recovery with a candidate LTM cell, the LTM recovery cell also sends an ACCESS SUCCESS message to the CU to indicate the (second) access cell.

Therefore, if RLF report is not provide after successful LTM recovery, the CU still have the ability to detect the connection failure during LTM procedure. For example, based on the presence and comparison of the target LTM cell indicated by the source DU, the first access cell and the second access cell, the CU will identify whether there was a connection failure during LTM procedure.

Observation 5: CU can identify whether there was a connection failure during LTM procedure.

To collect all the failure information during LTM for performance optimization, it is highly expected to consider the LTM recovery case without RLF report for LTM MRO.

Proposal 7: RAN3 considers the LTM MRO case: LTM recovery without RLF report.

## LTM SHR

In RAN3#123bis meeting, it was agreed on the study of near failure LTM:

Work on scenarios of near failure LTM

For previous SHR of L3 mobility, timer-based triggering conditions are introduced, including the timers T304, T310 and T312. All of the timers are still used as before during LTM procedure. It is straightforward to reuse all the above timer-based triggering conditions for LTE SHR determination.

Proposal 8: Reuse timers T304, T310 and T312 related triggering conditions for LTM SHR.

When CU receives the LTM SHR, it performs initial analysis and identifies the triggering condition(s) which triggered the SHR. Correspondingly, the CU can decide the DU responsible for the near failure LTM procedure. For example, if the LTM SHR was triggered based on the T304 related triggering condition, the CU forwards the LTM SHR to the target DU; while for T310 or T312 related triggering conditions, the CU forwards the LTM SHR to the source DU.

Furthermore, the CU forwards the LTM SHR to the DU based on the triggering condition(s).

Proposal 9: CU forwards the LTM SHR to DU based on triggering conditions for LTM SHR:

- Source DU in case of T310 or T312 related triggering conditions

- Target DU in case of T304 related triggering condition

# Conclusion

In this contribution, we discussed the potential solutions for LTM MRO. The related observations and proposals are summarized below:

LTM RLF report:

Observation 1: Only one C-RNTI is provided in the RLF report.

Observation 2: The C-RNTI will be set to:

- Source C-RNTI for

  - RLF in case of too late LTM; or

  - HOF in case of too early LTM or LTM to wrong cell

- target C-RNTI for RLF in case of too early LTM or LTM to wrong cell

Observation 3: If the LTM attempt is not configured, the UE may perform legacy reestablishment procedure with the selected LTM candidate cell after LTM failure.

Observation 4: The RAN node where the CHO recovery is performed will not identify whether there was a connection failure during CHO procedure.

Observation 5: CU can identify whether there was a connection failure during LTM procedure.

Proposal 1: CU forwards RLF report to the correct DU based on LTM failure type:

- The failure DU in case of too late LTM

- The source DU in case of too early LTM, or LTM to wrong cell

- The target DU if random access information exists in RLF report for HOF case

Proposal 2: DU performs root cause analysis to optimize the LTM related parameters.

Proposal 3: The CU forwards the source C-RNTI to the source DU:

- if the connection Failure type IE in RLF report is set to “RLF”; and

- if RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”.

Proposal 4: The CU forwards the target C-RNTI to the target DU:

- if the connection Failure type IE in RLF report is set to “HOF”; and

- if there is random access information in RLF report.

Proposal 5: For reestablishment without RLF report, the CU should forward additional information to source DU if the failure is not due to wrong selection of prepared LTM cell.

Proposal 6: For reestablishment without RLF report, the CU forwards to the source DU the following additional information:

- Source cell information

- Source C-RNTI

- Target cell information in case of too early LTM or LTM to wrong cell

- Reestablishment cell information

- LTM failure type

Proposal 7: RAN3 considers the LTM MRO case: LTM recovery without RLF report.

LTM SHR:

Proposal 8: Reuse timers T304, T310 and T312 related triggering conditions for LTM SHR.

Proposal 9: CU forwards the LTM SHR to DU based on triggering conditions for LTM SHR:

- Source DU in case of T310 or T312 related triggering conditions

- Target DU in case of T304 related triggering condition

# Reference

- TS 38.331 v18.3.0

- R3-245832, (TP for BLCR for SON for 38.473) MRO enhancement for R18 mobility mechanisms

- TS 38.423 v18.3.0

# Annex - TP for LTM MRO for TS 38.473

Start of Change

#### 9.2.10.1	ACCESS AND MOBILITY INDICATION

This message is sent by gNB-CU to gNB-DU to provide access and mobility information to the gNB-DU.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| RA Report List |  | 0..1 |  |  | YES | ignore |
| >RA Report Item |  | 1 .. <maxnoofRAReports> |  |  | - |  |
| >>RA Report Container | M |  | OCTET STRING | Includes the RA-ReportList-r16 IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| RLF Report Information List |  | 0..1 |  |  | YES | ignore |
| >RLF Report Information Item |  | 1 .. <maxnoofRLFReports> |  |  | - |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | Includes the nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [8]. | - |  |
| >>UE Assistant Identifier | O |  | gNB-DU UE F1AP ID 9.3.1.5 |  | - |  |
| >>C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU if the connection Failure type IE in RLF report is set to “RLF” and RLF Report Failure Type IE is set to value “too early LTM” or “LTM to wrong cell”; or C-RNTI allocated at the target gNB-DU | YES | ignore |
| >>RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| RRC Reestab Reporting without RLF report |  |  |  |  |  |  |
| >Source cell PCI | M |  | NG-RAN Cell PCI 9.2.2.10 | Physical Cell Identifier | - |  |
| >Target cell PCI | O |  | NG-RAN Cell PCI 9.2.2.10 | Physical Cell Identifier | - |  |
| >Re-establishment cell CGI | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  | - |  |
| >C-RNTI | O |  | 9.3.1.32 | C-RNTI allocated at the source gNB-DU | - |  |
| >RLF Report Failure Type | O |  | ENUMERATED (too late LTM, too early LTM, LTM to wrong cell,...) |  | YES | ignore |
| Successful HO Report Information List |  | 0..1 |  |  | YES | ignore |
| >Successful HO Report Information Item |  | 1 .. <maxnoofSuccessfulHOReports> |  |  | - |  |
| >>Successful HO Report Container | M |  | OCTET STRING | Includes the SuccessHO-Report IE as defined in subclause 6.2.2 in TS 38.331 [8]. | - |  |
| [……] |  |  |  |  |  |  |

End of Change