3GPP TSG-RAN WG3 Meeting #126	R3-247316

Orlando, US, 18th – 22nd November 2024

Agenda item:	14.3

Source:	Xiaomi, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, China Telecom, Lenovo

Title:	Support of Inactive UE mobility in NTN

Document for:	Other

# 1	Introduction

Inactive UE support

RAN3 assumes that current mechanisms to support UEs performing RRC connection resume and RRC connection re-establishment may be reused in regenerative payload architecture, subject to implementation and deployment.

The topic of inactive UE support in NTN was discussed but not fully discussed in previous RAN3 meetings, although RAN3 has the following agreement, companies in [1]-[5] believe that the current mechanism is not sufficient.

In this contribution, we’d like to further discuss the support of inactive UE in NTN on the issues and potential solutions.

# 2	Issues for support RRC inactive UE in NTN

In case a UE is sent to RRC_INACTIVE state by a gNB deployed on a non-GEO satellite, when UE resumes to a new satellite-gNB, the retrieve UE context procedure is performed, successful execution of this procedure depends on the availability of an inter-satellite link or feeder link between the anchor gNB and the receiving gNB.

As shown the below Figure 1, at T1, the orange satellite-gNB sends UE to RRC_INACTIVE state and keeps UE context. The following cases may happen at T2 when the orange satellite-gNB that have UE context moves to the other side of the earth:

- Case 1, for UE initiated resume, the blue satellite-gNB receives UE’s RRC resume request but it may not able to communicate with the orange satellite.

- Case 2, for network-initiated DL data/signalling, if the orange satellite-gNB wants to page the UE, it may not be able to send the RAN paging to the current serving gNB of the UE (blue satellite-gNB) if the inter-satellite link or feeder link is unavailable.

- Case 3, for MT purposes, if the connection between orange satellite and blue satellite is available via inter-satellite link(s) and/or feeder link(s), it may take a long time to page the UE, the AMF/UPF transfers the DL signalling or data to the last serving gNB, and then the last serving gNB sends the RAN paging to the current serving gNB, after that, UE resumes, this will obviously have a long transmission delay.

Figure 1

If reusing existing mechanism, the following detail cases may occur.

For case 1, UE initiated resume, as stated in TS 38.300[7], if the new serving gNB cannot retrieve UE context, it can either reject the UE resume or setup new RRC connection, the call flows can be as follows:

For the reject case, UE will perform resume after a wait timer, the next resume may also be failed, the user experience is negatively impacted by the service latency.

For the RRC setup case, the serving gNB (LEO1) may store the UE context for a long time even through the UE has already connected to the network via other satellites, this wastes the storage space and network efficiency on the satellite.

For Case 2, the network-initiated resume, e.g., there may be DL signalling or DL data, as specified in TS 38.300[7], the call flow can be as follows:

If the last serving gNB receives DL data/signalling, it cannot send the RAN paging to other satellites if there’s no ISL or ISL is not available, resulting in a failure of the RAN paging. In such cases, as specified in TS 23.501[8], the last serving gNB may request the CN to release the UE context.

In addition, if NG removal is performed, the “last serving gNB” will release all the contexts of the UEs served by the AMF. There may be two options for gNB behaviour, option A, the gNB releases the UE context without notifying the UE, option B, the gNB pages all the inactive UEs and release the RRC connection. The drawback for the case is that the UE does not know network release its context or a large number of UEs needs to be paged to relocate the context before NG removal.

All above cases can be solved by existing system but they have drawbacks for UEs and networks. By re-using existing mechanism that designed for TN network does not fully and well support inactive UE mobility in context of NTN.

Observation 1, The following drawbacks may arise when applying existing mechanisms for RRC_INACTIVE UE mobility, which are likely to occur frequently in NTN systems:

- UE may have a poor experience if the receiving gNB rejects the RRC resume request due to retrieval UE context failure.

- if UE setup new connection with a receiving gNB due to the resume failure, the last serving gNB may not be aware that the UE has connected to a new gNB and continues to maintain an irrelevant UE context.

- RAN releases UE context due to the RAN paging failures, UE have different RRC states compared to the network, resulting in misalignment of their behaviour.

- The service may be delayed even through there’s inter-satellite link or feeder link for the inactive UE.

Observation 2, reusing existing mechanisms designed for TN networks does not adequately support inactive UE mobility within the context of NTN.

# 3	Potential Solutions

Multiple potential solutions have been discussed in [1]-[6], they can be summarized as below:

Option 1, UE context is forwarded to the next satellites

- Option 1-1, before Xn is about to be disconnected, page and relocate UE context to next the satellite.

- The UE's last serving gNB can send paging messages to candidate gNB(s) relevant to the UE's RNA, when the Xn interface is about to be disconnected between the last serving gNB and the candidate gNB relevant to the UE's RNA.

- Option 1-2 Pro-actively move UE context to next satellite

- the inactive UE context is proactively moved from the gNB leaving a particular geographical area to an incoming gNB covering the same geographical area. This way when the UE resumes, the UE context is available in the nearby gNB to retrieve the context.

Option 2, UE context is kept on a fixed node (on-ground gNB or AMF)

- Option 2-1, relocate the UE context to the AMF on ground.

- When the UE resumes, the context is retrieved from the AMF by the gNB where the UE resumes. For a mobile terminated data/signaling, the AMF can initiate RNA-based paging procedure, then the serving gNB retrieve UE context from AMF.

- Option 2-2, relocate the UE context to the gNB on ground.

- UE context can be proactively relocated to an on-ground gNB. UE resumes by using the information generated by the on-ground gNB, the UE context is retrieved from the on-ground gNB. For a mobile terminated data/signaling, the AMF/UPF can directly communicate with the on-ground gNB.

Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used

- the UE context remains in last serving gNB. When UE resumes, the current serving gNB initiate NG-based UE context retrieve procedure to retrieve the UE context from last serving gNB. For MT data/signaling, the AMF can initiate RNA-based paging procedure.

The above options may have pros and cons, which may need further discussion in RAN3.

Observation 3, there’re multiple solutions to support inactive UE mobility on the table.

Proposal 1, RAN3 discuss the following options to support inactive UE mobility in NTN regenerative payload.

- Option 1, UE context is forwarded to the next satellites.

- Option 2, UE context is kept on a fixed node (on-ground gNB or AMF).

- Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used.

# 3	Conclusion

Based on the discussion, we have the following observations and proposals.

Observation 1, The following drawbacks may arise when applying existing mechanisms for RRC_INACTIVE UE mobility, which are likely to occur frequently in NTN systems:

- UE may have a poor experience if the receiving gNB rejects the RRC resume request due to retrieval UE context failure.

- if UE setup new connection with a receiving gNB due to the resume failure, the last serving gNB may not be aware that the UE has connected to a new gNB and continues to maintain an irrelevant UE context.

- RAN releases UE context due to the RAN paging failures, UE have different RRC states compared to the network, resulting in misalignment of their behaviour.

- The service may be delayed even through there’s inter-satellite link or feeder link for the inactive UE.

Observation 2, reusing existing mechanisms designed for TN networks does not adequately support inactive UE mobility within the context of NTN.

Observation 3, there’re multiple solutions to support inactive UE mobility on the table.

Proposal 1, RAN3 discuss the following options to support inactive UE mobility in NTN regenerative payload.

- Option 1, UE context is forwarded to the next satellites.

- Option 2, UE context is kept on a fixed node (on-ground gNB or AMF).

- Option 3, UE context is kept in the last serving gNB and NG-based UE context retrieve procedure is used.

| Options | Detail solution | Possible Impacts or issues |
| --- | --- | --- |
| Option1, UE context is forwarded to the next satellites | Page and relocate UE context to next the satellite | Stage2 impacts, rely on the ISL availability, may have signalling storm. |
|  | Pro-actively move UE context to next satellite (without notifying UE) | Xn impacts, may need to check SA3 with the security issue. |
| Option2, UE context is kept on a fixed node | relocate the UE context to the AMF on ground. | NG impacts |
|  | relocate the UE context to the gNB on ground. | Xn impacts |
| Option3, UE context is kept in the last serving gNB | NG-based UE context retrieve procedure is used if ISL is not available | NG impacts, may not work if NG is removed. |

# 4	Reference

[1] R3-245178, (TP for TS 38.300) Discussion on the support of Regenerative payload (Nokia, Nokia Shanghai Bell)

[2] R3-245434, (TP for TS 38.300) Discussion on RAN Signaling impacts for NR NTN Regenerative Payload (Qualcomm Incorporated)

[3] R3-245451, Interface management for regenerative payload in NTN (Lenovo)

[4]R3-245571, Discussions on NG management and Inactive support (LG Electronics Inc.)

[5] R3-245085, Support of regenerative payload (Xiaomi)

[6] R3-245361, (TP for BLCR for TS 38.300) Support of regenerative payload (including LS responses to SA2), (Huawei)

[7] TS 38.300, NR; NR and NG-RAN Overall description; Stage-2

[8] TS 23.501, 5G; System Architecture for the 5G System