3GPP TSG-RAN WG3 Meeting #R3-126		           R3-247323

Orlando, USA, Nov 18-22, 2024

Agenda Item:	11.2

Source: 	Rakuten

Title:  	UE Performance Data for AI/ML based Network Slicing

Document for:	Discussion and decision

# Introduction

Related to UE performance feedback of the handed over UEs from the target node to the source node, we list below the relevant references:

From 38.743:

Following are the notes from RAN3#125-bis meeting:

Slice UE performance:

Observation based on current specs: Average packet delay UL/DL, average UE throughput (UL/DL), and average packet loss (DL) can be calculated per PLMN ID and per QoS level and per supported S-NSSAI.

The granularity of UE performance:

Op1: per S-NSSAI per PDU Session: ZTE, CMCC, Jio, TI, E///, LGE

Opt2: per S-NSSAI: Lenovo, SS, HW, NEC

Opt3: per-QoS flow groups: CATT, Nok

WA: UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node.

FFS if other finer granularity of UE performance is feasible.

# Discussion

In the previous meetings, it has been discussed and agreed that after successful HO completion from source to target gNB, the target gNB will send UE performance metrics to the source gNB. This will assist the source gNB to evaluate the merits of its mobility decisions and act as feedback to validate the accuracy of its AI/ML training and prediction.

## Granularity of UE performance metric

In RAN3#125-bis meeting, we concluded the discussion of UE performance with a working assumption that a UE performance per each PDU session associated to a requested S-NSSAI is reported from target node to source node. And it was left for further investigation if other finer granularity of UE performance is feasible.

If the UE performance after handover is compared at the PDU session layer associated with a slice, we believe that the comparison might not be equitable. This might happen because a PDU session with certain QoS flows in a source NG-RAN node might be handed over to the target NG-RAN node with some of the QoS flows being denied admission at the target NG RAN node, as supported in 38.423.

Therefore, the performance metrics at per-PDU session level might not fairly represent the UE performance at the target NG RAN node.

Observation 1: Due to the possibility of partial QoS flow admittance at the target NG RAN node during the handover procedure, the per-PDU session level UE performance might not result in a fair comparison of the handed over UE between the source and target NG RAN nodes.

For this reason, we propose that finer granularity of UE performance feedback be considered to support the validation of AI/ML prediction at the source NG RAN node.

Proposal 1: We propose the use of finer granularity of UE-level UE performance metrics at QoS flow level per slice.

Average DL UE throughput in gNB is defined in 28.558 (Section 6.3.1.4.1) and the following forms are supported as mentioned in field e:

In 28.552 Section 4.2, examples of possible filter values are given as 5QI, QCI, SNSSAI, and PLMN. Furthermore, in Section 4.2.2 Filters naming, the use of multiple filters is given as:

<Performance measurement>_<Filter1>_<Filter2>

Therefore, for supporting the current use case of Network Slicing, to get the UE performance metric at QoS flow level for the relevant S-NSSAI we propose to use

<Performance measurement>_<SNSSAI>_<QoS>

## Data Collection Request

In Data Collection Request (Section 9.1.3.26 in 38.423), Report Characteristics for Data Collection IE has been defined as a 32-bit string, and the bitmap set by the source NG RAN node indicates to the target NG RAN node which UE measurements are required. In the current standard, these metrics are defined at the per cell granularity by default. As of now, nine bits have been identified as shown in the figure below:

It is assumed that further bits will be identified to represent the new metrics or pre-existing metrics of different granularities to represent the requirements of the use cases like Network Slicing and Capacity and Coverage Optimization and future use cases. However, in our opinion, this approach is inefficient for supporting the growing and varied requirements. Therefore, we propose options for addressing this concern.

First and foremost, we propose the use of a filter to enhance the reporting procedure. This filter will identify the granularity at which the performance metrics need to be reported by NG-RAN node2. The filter range will be what has been defined and supported in 28.558. The selected filter can also be a combination of multiple filters to get the required sub counters at the source NG RAN node.

Option 1: Add a new IE, Report Filter for Data Collection, to Data Collection Request to identify the filter which can be applied to the metrics represented in the Report Characteristics for Data Collection. However, the problem with this approach is that the same filter will be applied to all the metrics identified in the Report Characteristics for Data Collection, and it might not be applicable or relevant.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Other bits are ignored by the NG-RAN node2. | YES | reject |
| Report Filter for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(16)) | Measurements may be reported in the required granularity by use of applicable filters defined in 28.552 First bit = default Second Bit = UE Third bit = 5QI Fourth Bit = QoS Fifth Bit = DRB Sixth Bit = PDU Seventh Bit = SNSSAI Eighth Bit = Cell Ninth Bit = PLMN Other bits are ignored by the NG-RAN node2 | YES |  |

For example, for the current use case of network slicing, if the report of the UE performance feedback is required at per PDU Session level then the bitmap for the filter would be 0000010000000000. And if the UE performance feedback is required at per DRB per PDU Session level, then the bitmap for the filter would be 0000110000000000.

The selected filter is applied to the metrics identified in Report Characteristics for Data Collection and reported to the source NG-RAN node1.

Option 2: To ensure that the filter is being applied only to the relevant metrics selected in the Report Characteristics for Data Collection, we propose the use of another bitmap which will indicate which metrics the filter is applied to. This option is being proposed to ensure backward compatibility while adding further accuracy.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Other bits are ignored by the NG-RAN node2. | YES | reject |
| Report Metric to Filter Map | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object to which the filter Report Filter for Data Collection should be applied to by the NG-RAN node2 while reporting. 0 will indicate the default value 1 will indicate the selected filter is applied |  |  |
| Report Filter for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(16)) | Measurements may be reported in the required granularity by use of applicable filters defined in 28.552 First bit = default Second Bit = UE Third bit = 5QI Fourth Bit = QoS Fifth Bit = DRB Sixth Bit = PDU Seventh Bit = SNSSAI Eighth Bit = Cell Ninth Bit = PLMN Other bits are ignored by the NG-RAN node2 | YES |  |

To explain the use of this, please consider the following example:

If the Report Characteristics for Data Collection has bits 2-7 selected for reporting:

Second Bit = Predicted Number of Active UEs,

Third Bit = Predicted RRC Connections

Fourth Bit = Average UE Throughput DL,

Fifth Bit = Average UE Throughput UL,

Sixth Bit = Average Packet Delay,

Seventh Bit = Average Packet Loss DL

and the NG-RAN has requested the information at the QoS level then the Report Filter for Data Collection IE will be 0001000000000000.

Since this filter is not applicable to all the metrics requested in Report Characteristics for Data Collection, the proposed IE Report Metric to Filter Map can be used to indicate which metrics (bits) of Report Characteristics for Data Collection it applies to. In the example above, if the metric is applicable only to the 4-7 bits of Report Characteristics for Data Collection, then the Report Metric to Filter Map bitmap will be 0001111000000000…..0000.

While this option overcomes the limitation of the method listed in Option 1, this has the limitation that different filters in Report Filter for Data Collection cannot be applied to different metrics from Report Characteristics for Data Collection.

Option 3: We propose the use of a list with two items, the first to indicate the metric of interest, and the second to indicate the applicable filter or granularity of reporting. This approach is future-proof and most accurate in representing the requirements from NG-RAN node1 to NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the NG-RAN node2 is requested to report. First Bit = Predicted Radio Resource Status, Second Bit = Predicted Number of Active UEs, Third Bit = Predicted RRC Connections Fourth Bit = Average UE Throughput DL, Fifth Bit = Average UE Throughput UL, Sixth Bit = Average Packet Delay, Seventh Bit = Average Packet Loss DL Eighth Bit = Energy Cost Ninth Bit = Measured UE Trajectory Other bits are ignored by the NG-RAN node2. | YES | reject |
| Measurement List for Data Collection |  | 0..1 |  | Measurement list to be reported for data collection | YES | ignore |
| >Measurement Item for Data Collection |  | 1 .. <maxnoofMeas> |  |  | – |  |
| >>Measurement ID | M |  | Meas Identifier 9.2.x | Indicates a measurement identifier for Data Collection | – |  |
| >> Report Filter for Data Collection | O |  | BITSTRING (SIZE(32)) | Measurements may be reported in the required granularity by use of applicable filters defined in 28.552 First bit = default Second Bit = UE Third bit = 5QI Fourth Bit = QoS Fifth Bit = DRB Sixth Bit = PDU Seventh Bit = SNSSAI Eighth Bit = Cell Ninth Bit = PLMN Other bits are ignored by the NG-RAN node2 | – |  |

The measurement list is a collection of measurement items, each of which shall contain the measurement identifier and a corresponding filter.

For example,

The measurements required to be reported at cell level are Predicted Number of Active UEs, Predicted RRC Connections, and the measurements required to be reported at DRB level are Average UE Throughput DL, Average Packet Delay, and Average Packet Loss DL. The resulting measurement list will look like:

<Predicted Number of Active UEs, 000000010….0000>

< Predicted RRC Connections, 000000010….0000>

<Average UE Throughput DL, 000010000…000>

<Average Packet Delay, 000010000…000>

<Average Packet Loss DL, 000010000…000>

The advantage of using this IE is that it gives the flexibility of choosing the granularity of UE performance metric feedback based on the requirement of the use case. Secondly, it allows for all future use cases without being limited in selection of number of metrics to report. If backward compatibility is essential, the Report Characteristics for Data Collection can be retained, however, we are open to discussing replacing Report Characteristics for Data Collection with the proposed Measurement List for Data Collection.

We propose the use of Option 3 to enhance DATA COLLECTION REQUEST because it is the most forward-looking and flexible.

Proposal 2: RAN3 to discuss the options above and select one to ensure that DATA COLLECTION REQUEST can accurately represent the requirements of current and future use cases of AI/ML.

# Conclusion

We have the following observations and proposals:

Observation 1: Due to the possibility of partial QoS flow admittance at the target NG RAN node during the handover procedure, the per-PDU session level UE performance might not result in a fair comparison of the handed over UE between the source and target NG RAN nodes.

Proposal 1: We propose the use of finer granularity of UE performance metrics at QoS flow level per slice.

Proposal 2: RAN3 to discuss the options above and select one to ensure that DATA COLLECTION REQUEST can accurately represent the requirements of current and future use cases of AI/ML

# Reference

[1] TS28.552 5G performance measurements

[2] TS25.558 UE level measurements for 5G system

[3] TS38.423 Xn application protocol (XnAP)