3GPP TSG-RAN3 Meeting #126	R3-247327

Orlando, US, Nov 18-22, 2024

Agenda Item:	11.4

Source: 	Qualcomm Incorporated, Nokia, Nokia Shanghai Bell

Title: 	Discussion on Continuous MDT

Document for:	Discussion/Decision

# Background

In RAN3#124 the problem statement of Continuous MDT was captured in the TR 38.743 as shown below. In RAN3#125bis it was agreed that OAM initiates and configures Management based MDT for Continuous MDT towards NG-RAN. Solutions were listed for Problem A and B, but the solutions were not down selected.

RAN3#124 - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. RAN3#125bis - RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN. - Further discussions are needed on how to define a temporary identifier for Continuous MDT. - Further discussions are needed on the solution for measurement correlation in Continuous MDT.

In this paper we discuss our views on solutions for Continuous MDT for Problem A and B.

# Discussion

## Problem Statement A – Measurement Continuity across RRC States

To get continuous MDT data from the same set of UE(s) across RRC States, RAN should be able to identify the group of UE(s) across state transitions. This way MDT measurement continuity can be achieved for the same group of UEs. There are 3 ways in which this can be achieved.

UE based solution:

When the UE goes to RRC_Idle, an identifier to identify the continuous MDT session can be passed from RAN on to the UE and when the UE goes to RRC Connected state, the UE reports the continuous MDT identifier back to the RAN. This will enable the RAN to identify the UE participating in Continuous MDT session.

Disadvantages:

- Has UE impacts and legacy UEs cannot participate in continuous MDT which will reduce the AI/ML data set.

- This might have security issues due to an identifier to identify the UE is passed to the UE over the air interface.

In UE based solution, continuous MDT session identifier is sent to the UE from RAN when it goes to RRC_idle, and UE reports it back to the RAN when it goes connected, for measurement continuity. Since this solution impacts the UEs, legacy UEs cannot participate in the continuous MDT which will drastically reduce the AI/ML training data set.

RAN3 to agree not to pursue UE based solution due to UE impacts and the legacy UE data cannot be used.

AMF based solution:

RAN can request AMF to allocate an identifier for the UE or a group of UEs participating in a Continuous MDT session. AMF provides an identifier to the RAN for the requested group of UEs and AMF stores this identifier in its database. When the UE goes to RRC_idle and moves to RRC Connected again, AMF identifies the UE and provides the Continuous MDT session identifier stored in its database to the RAN. This will enable the RAN to identify the UE participating in Continuous MDT session.

A sample call flow for continuous MDT session for AMF based is provided below for reference.

In AMF based solution, upon request from RAN, AMF allocates identifier for the group of UEs participating in a Continuous MDT session and assists the RAN/OAM in identification of the UEs across RRC states and measurement continuity by providing same Continuous MDT session identifier to the RAN during UE RRC state transition.

OAM based solution:

OAM can provide an identifier for Continuous MDT session in the Management Trace initiation to RAN. RAN can associate this identifier from OAM to the group of UEs participating in the Continuous MDT session. When the UE goes Idle, RAN can request AMF to store the Continuous MDT session identifier from OAM. When the UE moves to Connected state, AMF can provide the Continuous MDT session identifier to RAN. This will enable the RAN to identify the UE participating in Continuous MDT session.

A sample call flow for continuous MDT session for OAM based solution is provided below for reference.

In OAM based solution, OAM allocates identifier for a Continuous MDT session and sends it to RAN. RAN requests AMF to store this identifier when the UE goes idle and AMF provides it to RAN when the UE goes connected again for UE identification in RAN.

RAN3 to pursue AMF/OAM based solution for identification of the same UE(s) across RRC states for measurement continuity for Continuous MDT.

## Problem Statement B - MDT Report Correlation in OAM

When the UE provides logged and immediate MDT reports via RAN to OAM, OAM should be able to identify that the reports belong to the same UE or group of UEs across RRC states for a particular continuous MDT session. Correlation of MDT reports at OAM can be achieved by RAN sending a Continuous MDT session identifier in the logged and immediate MDT reports to OAM, for OAM to correlate the MDT reports from the same UE across RRC States.

Correlation of MDT reports in OAM from the same UE across RRC States can be achieved via a Continuous MDT session identifier sent by RAN in the logged and immediate MDT reports to OAM.

OAM/AMF based solution:

In the OAM/AMF based solution as detailed above in Clause 2.2, RAN can provide the Continuous MDT session identifier provided by AMF or OAM in the immediate and logged MDT reports to the OAM for immediate and logged MDT report correlation for same UE across RRC States.

RAN to report the Continuous MDT session identifier (e.g any reserved TR or any new identifier) allocated by AMF or OAM, while reporting immediate or logged MDT reports to OAM for correlation of MDT trace reports.

## User Consent for Continuous MDT

User consent is required for MDT report collection and user consent is provided for specific use cases for MDT report collection. It is unclear whether the user consent received for logged and immediate MDT can be used to collect data for AI/ML training as well or a separate user consent is needed. SA3 clarification is also needed to know if user consent received for logged and immediate MDT can be used for Continuous MDT collection.

RAN3 to discuss if user consent received for logged and immediate MDT report collection can be used for MDT report collection for AI/ML training and for continuous MDT collection.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	In UE based solution, continuous MDT session identifier is sent to the UE from RAN when it goes to RRC_idle, and UE reports it back to the RAN when it goes connected, for measurement continuity. Since this solution impacts the UEs, legacy UEs cannot participate in the continuous MDT which will drastically reduce the AI/ML training data set.

Observation 2.	In AMF based solution, upon request from RAN, AMF allocates identifier for the group of UEs participating in a Continuous MDT session and assists the RAN/OAM in identification of the UEs across RRC states and measurement continuity by providing same Continuous MDT session identifier to the RAN during UE RRC state transition.

Observation 3.	In OAM based solution, OAM allocates identifier for a Continuous MDT session and sends it to RAN. RAN requests AMF to store this identifier when the UE goes idle and AMF provides it to RAN when the UE goes connected again for UE identification in RAN.

Observation 4.	Correlation of MDT reports in OAM from the same UE across RRC States can be achieved via a Continuous MDT session identifier sent by RAN in the logged and immediate MDT reports to OAM.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	RAN3 to agree not to pursue UE based solution due to UE impacts and the legacy UE data cannot be used.

Proposal 2:	RAN3 to pursue AMF/OAM based solution for identification of the same UE(s) across RRC states for measurement continuity for Continuous MDT.

Proposal 3:	RAN to report the Continuous MDT session identifier (e.g any reserved TR or any new identifier) allocated by AMF or OAM, while reporting immediate or logged MDT reports to OAM for correlation of MDT trace reports.

Proposal 4:	RAN3 to discuss if user consent received for logged and immediate MDT report collection can be used for MDT report collection for AI/ML training and for continuous MDT collection.