3GPP TSG-RAN WG3 Meeting #126	R3-247346

# Orlando, USA, 18th - 22nd November 2024

Agenda item:	15.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support of IoT NTN

Document for:	Discussion and Decision

# 1	Introduction

The WID lists following objectives for RAN3:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

SA2 ([1]) agreed two architectures: a split MME architecture (i.e. architecture option 1), and an architecture with full CN on the satellite (i.e. architecture option 2).

This contribution analyses the high-level issues (e.g. procedure related issues) and the impact to RAN3.

# 2	Architecture Options

SA2 agreed to support two network architectures, i.e. a split MME architecture (i.e. architecture option 1), and an architecture with full CN on the satellite (i.e. architecture option 2). In last meeting, some companies propose to also consider the 3rd architecture option with only eNB on board the satellite and MME on earth. We propose to not consider this architecture option for following reasons:

- First, this architecture option is not considered by SA2.

- Second, The S1 connection is setup when feeder link is available, and the S1 connection becomes not available when the feeder link is unavailable. This architecture option impacts S1 interface, and requires SA2 support. Since SA2 does not support this architecture option in normative phase, there is no way for RAN3 to continue the work without SA2 support.

So we propose RAN3 shall focus the discussion on the architecture options selected by SA2 in Rel-19.

Proposal 1: RAN3 only work on the architecture options selected by SA2 in Rel-19.

# 3	general issues applied to both architecture options

There are some common issues that are applicable to both architecture options:

- Issue 1: Does the eNB need to know which architecture option is used?

In other words, does the eNB need to know whether it is connected with a split MME, or a full MME?

The eNB just considers it connects with a normal MME. The eNB does not need to know whether it is partial MME (in architecture option 1), or a full MME (in architecture option 2). By implementation, the eNB can know when the Feeder Link becomes available or not available. Based on this information, the eNB can broadcast the related S&F mode over the air interface based on RAN2 decision.  Even in case the eNB need to know the architecture option, this information may be configured in the eNB. There is no impact to S1AP.

Proposal 2-1: No need to enhance S1AP to inform eNB on which architecture option is used.

- Issue 2: S1 interface

In both architecture options, the eNB only setup S1 with the MME on the same satellite. There is no S1-Flex, thus NAS Node Selection function is not performed.

In case of network sharing MOCN, it is possible that there may be more than one logical MME functions on the same satellite and one MME function per PLMN. The eNB may connect with the MMEs on the same satellite. The eNB select the MME, which is similar as MME selection in normal MOCN in terrestrial network. Up to the implementation or configuration, the eNB setup S1 with MME(s) on the same satellite.

Unlike NR NTN that a gNB may need to remove the NG connection, e.g. when the satellite move out, the S&F eNB always connect with the MME on the same satellite. So there is no need to consider whether the S1 need to be removed.

Proposal 2-2: in S&SF system, an eNB only setup S1 with the MME(s) on the same satellite.

- Issue 3: feeder link switch over

In both architectures, the feeder link is not used by the eNB.

- In architecture option 1, the feeder link is used between the MME-onboard the satellite (MME-NT) and the MME-on ground (MME-T).

- In architecture option 2, the feeder link is used by a CN proxy onboard the satellite and the CN proxy on ground.

The feeder link switch over only affects the interface between MME-NT and MME-T (in architecture option 1), and the interface between proxies (in architecture option 2). SA2 agreed these interfaces are out of 3GPP scope. The eNB is not affected by the feeder link switch over.  There is no impact to RAN3.

Proposal 2-3: The feeder link switch over has no impact to RAN3.

# 4	Discussion on architecture option 1 (Split MME)

In this architecture option, the satellite host the eNB function and part/full MME function, but the S-GW and other CN nodes are on the earth.

Control Plane CIoT EPS optimisation

TS 36.300 defines following MO/MT procedures:

Figure 7.3b-1: MO-EDT for Control Plane CIoT EPS Optimisation

Figure 7.3c-1: MT-EDT for Control Plane CIoT EPS Optimisation

In CP CIOT EPS optimization, the DRB and AS security are not used. From RAN3 perspective, the eNB connects with the MME on the same satellite. The MO/MT data is buffered in MME, and does not affect eNB. Via the S1 interface, current MO/MT for CIOT-CP procedure can be reused.

Proposal 3-1: Current MO/MT for Control Plane CIOT EPS Optimisation can be reused in split MME architecture.

User Plane CIoT EPS optimisation

TS 36.300 defines following MO/MT procedures:

Figure 7.3a.3-1: RRC Connection Suspend procedure in EPS

Figure 7.3c-2: MT-EDT for User Plane CIoT EPS Optimisation

In case the S&F system only have one satellite (i.e. only one eNB), the UE suspend/resume uses the same satellite/eNB. Current procedure can be reused.

But in case the S&F system have more than one satellite, and the UE’s suspend and resume are performed via different satellite, it may have the issue. Current resume in different eNB requires to use X2 interface to retrieve the UE context from the last serving eNB.

Figure 7.3a.3-3: RRC Connection Resume procedure in different eNB in EPS

It is questionable on whether X2 is available between the last serving satellite/eNB and current serving satellite/eNB. Enhancement may be needed to transfer the UE Context via the S1 interface, or store the UE context in a network node on the ground.  In case the UE context is transferred via S1, or via a network node on the ground, it also have impact to other working groups, e.g. SA2, SA3, etc.

Proposal 3-2: RAN3 need to discuss whether support UP CIOT EPS optimisation. Input from other working group also need to be considered.

# 5	Discussion on architecture option 2 (full CN on satellite)

In Architecture option 2, the whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. The MO/MT for a specific UE is only performed via the same satellite.  In case multiple satellites are used, UE context/information need to be shared among the satellites, which will be in SA2 scope. From RAN3 perspective, the eNB connect with the MME/SGW on the same satellite. Current MO/MT procedure can be reused. It seems no impact to RAN3.

Proposal 4: There may be no RAN3 impact to support the architecture option with full CN on satellite.

# 6	Discussion on PWS support for NB-IoT NTN

RAN3 received a RAN2 LS ([3]) on the PWS support for NB-IoT NTN. The LS lists following actions to RAN3:

ACTION:	 RAN2 invites RAN3, CT1, SA1, SA2 to take the above agreements into account and to consider whether any alignment will be needed.

The PWS support is added to the latest WID RP-242397 (copied as below).

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In general, PWS is supported from RAN3 perspective. TS 36.300 v17.9.0 defines following:

## 23.21	Support for BL UEs, UEs in enhanced coverage and NB-IoT UEs over Non-Terrestrial Networks

### 23.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported in the following cases:

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

It is unclear whether RAN3 is impacted per the updated WID. For CMAS, the geo-fencing in the UE is supported. However, for ETWS, there is no geo-fencing. Considering a NTN cell may cover very large geographical area, but the ETWS warning message may be only related to UEs in a specific small area, it may be necessary to support the intended service area information for ETWS. The potential impact to RAN3 is unclear, e.g. whether it is possible to reuse the Warning Area Coordinates IE also for ETWS. This depends on RAN2 progress, there is no need to start RAN3 work before RAN2 decision.

Proposal 5: RAN3 can wait for RAN2 decision before RAN3 starts the discussion on PWS support for NB-IoT NTN.

# 7	Conclusion

In this contribution, we discussed how to support Store&Forward, and the PWS support. Our proposals are:

Proposal 1: RAN3 only work on the architecture options selected by SA2 in Rel-19.

Proposal 2-1: No need to enhance S1AP to inform eNB on which architecture option is used.

Proposal 2-2: in S&SF system, an eNB only setup S1 with the MME(s) on the same satellite.

Proposal 2-3: The feeder link switch over has no impact to RAN3.

For Split-MME:

Proposal 3-1: Current MO/MT for Control Plane CIOT EPS Optimisation can be reused in split MME architecture.

Proposal 3-2: RAN3 need to discuss whether support UP CIOT EPS optimisation. Input from other working group also need to be considered.

For full CN on satellite:

Proposal 4: There may be no RAN3 impact to support the architecture option with full CN on satellite.

Proposal 5: RAN3 can wait for RAN2 decision before RAN3 starts the discussion on PWS support for NB-IoT NTN.

# References

- TR 23.700-29

- TS 36.300

- R3-247008, LS on PWS support for NB-IoT NTN