3GPP TSG-RAN WG3 Meeting #126	R3-247377

Orlando, USA, 18 - 22 Nov 2024

Title:	Discussion on Continuous MDT measurement

Source:				China Telecom

Agenda item:		11.4

Document for:		Discussion and Decision

1. Introduction

In the study phase, we discussed use case for AI/ML based continuous MDT collection targeting the same UE across RRC states. The following problems for the data collection continuity was captured in TR 38.743 [1]:

-	Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states.

-	Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

In RAN3#125-bis meeting, the following assumption was made with several FFSs.

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

Further discussions are needed on how to define a temporary identifier for Continuous MDT.

Further discussions are needed on the solution for measurement correlation in Continuous MDT.

In this document, we continue discussing AI/ML based continuous MDT collection targeting the same UE across RRC states and provide proposals.

2. Discussion

To solve problem A (measurement continuity), it is needed to select again the same UE when the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive or when the UE is handed over between gNBs. In management-based MDT, UEs are not uniquely identified. Therefore, the identifier for continuous MDT needs to be considered. Based on the RAN3 assumption that the OAM configures management-based MDT for continuous MDT towards the NG-RAN, so we have following proposal:

Proposal 1: OAM configures the continuous MDT measurement indication for the management-based MDT in the MDT activation message.

Based on the continuous MDT measurement indication, gNB knows whether the UE should be selected for continuous MDT measurement. But there are several issues:

- Issue 1: When a UE transits to RRC_Idle state, the MDT configuration is erased by the UE. Therefore, when a UE transits from RRC_Idle to RRC_Connected state, the network cannot know the configuration that the UE had before it transits to RRC_Idle state.

- Issue 2: In legacy Management-based MDT, the MDT configuration does not propagate during handover. Therefore, the target node cannot know the configuration that the UE had in the source node.

Therefore, to address the issues above, we provide the following solutions:

-	Solution A1: The continuous MDT measurement indication is stored at AMF. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive, the AMF indicates to the gNB that the UE should be selected for continuous management-based immediate MDT

-	Solution A2: The continuous MDT measurement indication is stored at gNB. When the UE hands over from source gNB to target gNB, the source gNB indicates to the target gNB that the UE should be selected for continuous management-based immediate MDT.

Based on the solutions above, we have following proposals:

Proposal 2: gNB can obtain the continuous MDT measurement indication from AMF. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive, the gNB re-selects the UE for continuous MDT with this indication.

Proposal 3: Continuous MDT measurement indication should be transmitted from Source gNB to Target gNB. When the UE handover from Source gNB to Target gNB, the Target gNB re-selects the UE for continuous MDT with this indication.

The Figure 1 is the signal flow example of solution A1 and Figure 2 is the signal flow example of solution A2.

Figure 1 The UE transits across RRC state

Figure 2 The UE hands over from Source gNB to Target gNB

Apart from the continuous MDT measurement indication parameter, the continuous MDT measurement duration is also important. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive and/or when the UE is handed over between gNBs, the gNB should know whether the UE is configured the continuous MDT measurement indication parameter and whether the validity of continuous MDT measurement indication parameter is expired. If the validity of continuous MDT measurement indication parameter is expired, the gNB will not select the UE for continuous management-based immediate MDT even though the UE is configured the continuous MDT measurement indication parameter. Therefore, we have following proposal:

Proposal 4: The MDT configuration provided by OAM should include the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication.

3. Conclusion

We have following proposals:

Proposal 1: OAM configures the continuous MDT measurement indication for the management-based MDT in the MDT activation message.

Proposal 2: gNB can obtain the continuous MDT measurement indication from AMF. When the UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive, the gNB re-selects the UE for continuous MDT with this indication.

Proposal 3: Continuous MDT measurement indication should be transmitted from Source gNB to Target gNB. When the UE handover from Source gNB to Target gNB, the Target gNB re-selects the UE for continuous MDT with this indication.

Proposal 4: The MDT configuration provided by OAM should include the continuous MDT measurement duration. It defines the validity of continuous MDT measurement indication.

4. Reference

- TR 38.743, Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN (Release 19)