3GPP TSG-RAN WG3 Meeting #126	   				         R3-247379

Orlando, USA, 18th – 22nd Nov 2024

Agenda Item:		14.2

Source:			China Telecom

Title:		Support of MBS broadcast service for NTN

Document for:	 	Other

- Introduction

The following agreement has been made at previous RAN3 meeting:

Mapped Cell ID(s) could be used to describe the intended MBS service area for NTN.

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

In this contribution, we further provide some potential stage-2 description updates for MBS broadcast services based on RAN2&RAN3 agreements.

- TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<< Begin of the changes >>>>>>>>>>>>>>>>>>>

16.14.5	NG-RAN signalling

The Cell Identity, as defined in TS 38.413 [26] and TS 38.423 [50], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported:

-	The Cell Identity indicated by the gNB to the Core Network as part of the User Location Information;

-	The Cell Identity used for Paging Optimization in NG interface;

-	The Cell Identity used for Area of Interest;

-	The Cell Identity used for PWS;

-	The Cell Identity used for MBS broadcast service.

The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell. The cell identity used in the NG and Xn handover messages, Xn Setup and Xn NG-RAN Node Configuration Update procedures is expected to be Uu Cell ID.

The Cell Identities used in the RAN Paging Area during Xn RAN paging allow the identification of the correct target cells for RAN paging.

NOTE 1:	The Cell Identity used for RAN Paging is assumed to typically represent a Uu Cell ID.

The mapping between Mapped Cell IDs and geographical areas is configured in the RAN and Core Network.

NOTE 2:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

The gNB is responsible for constructing the Mapped Cell ID based on the UE location information received from the UE, if available. The mapping may be pre-configured (e.g., up to operator's policy) or up to implementation.

NOTE 3:	As described in TS 23.501 [3], the User Location Information may enable the AMF to determine whether the UE is allowed to operate at its present location. Special Mapped Cell IDs or TACs may be used to indicate areas outside the serving PLMN's country.

The gNB reports the broadcasted TAC(s) of the selected PLMN to the AMF as part of ULI. In case the gNB knows the UE's location information, the gNB may determine the TAI the UE is currently located in and provide that TAI to the AMF as part of ULI.

To support the intended service area of a broadcast service (e.g. MBS broadcast) in case the satellite footprint covers a larger area, the intended service area may be in a for ID list, TAI list, geographical area list or any combination as defined in TS 38.413 [26].

NOTE 4: The geographical area is assumed to be transmitted to Uu with the same format received from CN. If the intended service area is described by Mapped Cell ID or TAI, the gNB is responsible for converting them into a geographical area represented by a (set of) reference location and radius or by a (set of) polygon(s), and broadcasts over Uu.

Editor’s Note: The above paragraphs may be further reformulated.

<<<<<<<<<<<<<<<<<< End of the changes >>>>>>>>>>>>>>>>>>>