3GPP TSG RAN WG3#126	R3-247386

Orlando, USA, 18 – 22 Nov. 2024

Agenda item:	17.2

Source: 	China Telecom

Title: 	On support on-demand SSB SCell operation

Document for:	Discussion and decision

- Introduction

In RAN#105, the work item on “Enhancements of network energy savings for NR” was revised [1]. The three objectives in the work item were listed below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA;

- Specify support for on-demand SIB1 for UEs in idle/inactive mode;

- Specify adaptation of common signal/channel transmissions.

In this paper, we discuss on the OD-SSB SCell related issues based on the latest progress of RAN1 and RAN2.

- Discussion on OD-SSB SCell operation

2.1 Analysis on potential RAN3 impacts

The agreements related to RRC signaling from the most recent RAN2 and RAN3 meeting were listed below.

RAN1 agreements in RAN1#118bis

Agreement For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options Agreement For a cell supporting on-demand SSB SCell operation, support to provide at least the following parameters for on-demand SSB configuration by RRC at least for Case #1. - Sub-carrier spacing of the on-demand SSB - FFS if this can be absent - Physical Cell ID of the on-demand SSB - FFS: Time domain location of on-demand SSB burst such as SFN offset and half frame index - Downlink transmit power of on-demand SSB - FFS: The number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS whether the above parameters are configured by reusing legacy RRC parameters or new RRC parameters

RAN2 agreements in RAN2#127bis

Agreements on OD-SSB SCell - No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required. - Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A. - NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Based on the agreements of previous meetings, we observe that RRC signaling will be used to configure on-demand SSB transmission and MAC CE will be used to active the OD-SSB transmission for UE. The deactivation of OD-SSB has been agreed to support, but whether to use RRC signaling or MAC CE to indicate the deactivation of OD-SSB is still under discussion in RAN1.

But from RAN3 point of view, if RRC signaling has been supported to configure the OB-SSB, we need to discuss the specification impact on F1 in the case of a CU/DU split.

Observation 1: For CU/DU split case, as the RRC signaling of OD-SSB is related to F1 interface, the specification impact on F1 need to be discussed.

The inter-gNB scenario between Pcell and NES Scell has not been discussed yet, so there is currently no need to address the protocol impact of Xn interface. We can follow the progress of RAN1 and RAN2 to see there is any impact on Xn interface.

Observation 2: For now, there is no need to discuss the protocol impact on the Xn interface.

2.3 Analysis on potential F1 impacts

- Which node should configure SCell to use the on-demand SSB transmission mode

Currently, the CU configures the UE to use carrier aggregation (CA) to improve the traffic throughput as it has better knowledge of the transmission requirements and mobility state of the UE than the DU. Thus, CU should be responsible for deciding whether the SCell should stake the responsibility of deciding whether SCell should switch to the on-demand SSB operation. If the CU determines the transmission state of UEs does not require continuous SSB transmission, it can configure the UE to be ready for the on-demand SSB transmission via RRC signaling, and inform the DU that the cell can switch to the on-demand SSB operation mode.

Proposal 1: CU should decide whether the SCell can operate in the on-demand SSB transmission mode and inform the DU via F1 signaling.

- Which node makes the decision to trigger the on-demand SSB operation

The purpose of on-demand SSB operation is to reduce power consumption during SSB transmission. Unlike always-on SSB transmission, the on-demand SSB is only transmitted when required by UEs. If users can synchronize with the SCell or access it normally without the need for SSB transmission, the SSB will not be transmitted to reduce the gNB power consumption.

Since cell load changes frequently, the on-demand SSB operation needs to be performed quickly. Therefore, the DU should trigger the on-demand SSB operation by using MAC CE, which is agreed by RAN1 and RAN2 as MAC CE is used for faster configuration of UE operation.

Proposal 2: DU should trigger the activation/deactivation of the on-demand SSB operation using MAC CE and inform the CU of the SSB operation state via F1 signaling.

- Conclusion

Based on the above discussion regarding support for on-demand SSB operation, we have the following observations and proposals:

Observation 1: For CU/DU split case, as the RRC signaling of OD-SSB is related to F1 interface, the specification impact on F1 need to be discussed.

Observation 2: For now, there is no need to discuss the protocol impact on the Xn interface.

Proposal 1: CU should decide whether the SCell can operate in the on-demand SSB transmission mode and inform the DU via F1 signaling.

Proposal 2: DU should trigger the activation/deactivation of the on-demand SSB operation using MAC CE and inform the CU of the SSB operation state via F1 signaling.

- Reference

[1] RP-242354, New WID: Enhancements of network energy savings for NR, Ericsson.