3GPP TSG-RAN WG3 Meeting #126	R3-247415

Orlando, US, 16th – 22nd November, 2024

Title: 	Timing information and feedback for AI/ML-based Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	11.3

Document Type:	Discussion and Decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based CCO (Coverage and Capacity Optimization).

This contribution discusses the timing information and AI/ML feedback for the AI/ML-based CCO use case, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

Signalling details to enable the AI/ML-based CCO use case to be specified over Xn and F1 interfaces, instead, are discussed in a separate discussion paper [5].

# 2. Discussion

In the last RAN3 meeting the following agreements and open issues were achieved [4]:

Stage3 signalling design: Xn interface: Transferring Future Coverage State and the associated Coverage Modification Cause in the NG-RAN NODE CONFIGURATION UPDATE message. The Coverage Modification Cause reflects the predicted CCO issue. F1 interface: WA: Reusing the CU configuration update procedure to transfer the predicted CCO issue and reusing the DU configuration update procedure to transfer the Future Coverage State over F1. Timing information: Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen. Information to validate the prediction is needed.

We discuss here further the timing information and feedback information for the AI/ML-based CCO use case.

## 2.1 Timing information for AI/ML-based CCO

In the last meeting RAN3 agreed to specify separate timing information for predicted CCO issue and future CCO state, and also agreed definitions for them [4]:

Timing information: Introducing separate time information for future coverage state and predicted CCO issue. Time for future coverage state: The point in time when the future coverage state will be applied. Time for predicted CCO issue: The point in time when the CCO issue is predicted to happen.

In the RAN3 discussions in the last meeting there was a common understanding that the both the timing information for predicted CCO issue (gNB-CU to gNB-DU) and future CCO state (gNB-DU to gNB-CU) need to be transferred over F1. The timing information associated to the predicted CCO issue (i.e., Time for predicted CCO issue) is transferred from the gNB-CU to the gNB-DU so that the gNB-DU is made aware of the fact that the new CCO state to cope with the predicted CCO issue is to be applied in the future and not immediately because the CCO issue did not occur yet. Conversely, the timing information of the future CCO state (i.e., Time for future coverage state) is sent from the gNB-DU to the gNB-CU so that the gNB-CU will know when the future CCO state will be activated so to prevent (or limit at an early stage) the occurrence of the CCO issue.

Considering the proposals in [5] on re-using the gNB-CU/gNB-DU Configuration Update procedures to convey information for the AI/ML-based CCO use case, it is straightforward to include the “time for predicted CCO issue” within the new Predicted CCO Assistance Information IE (within GNB-CU CONFIGURATION UPDATE message), and the “time for future coverage state” in the new Future Coverage Modification Notification IE (within GNB-DU CONFIGURATION UPDATE message), respectively.

Proposal 1: RAN3 to agree to introduce the “time for predicted CCO issue” within the new Predicted CCO Assistance Information IE (within GNB-CU CONFIGURATION UPDATE message), and the “time for future coverage state” in the new Future Coverage Modification Notification IE (within GNB-DU CONFIGURATION UPDATE message), respectively.

The TP for AI/ML BLCR to TS 38.473 for AI/ML-based CCO in the Annex 2 of [5] also reflects the above Proposal 1.

Concerning the XnAP signalling, instead, RAN3 did not discuss in the last meeting on which timing information need to be sent over Xn. Let us assume that we have a split gNB1 (gNB-CU1 and gNB-DU1) where the CCO issue is predicted, and a neighbor split gNB2 (gNB-CU2 and gNB-DU2) that is also impacted by this predicted CCO issue. gNB-CU1 sends over F1 the predicted CCO issue and “time for predicted CCO issue” to gNB-DU1 which, in turn, generates the future CCO state together with the “time for future coverage state” and sends these pieces of information to the gNB-CU1. According to what has been captured in TR 38.743 clause 4.2.2.5 for the impacts on Xn interface, the gNB-CU1 sends over Xn the future CCO state (of gNB1) with an associated coverage modification cause and “timing information”. To understand what “timing information” over Xn is, we shall consider what has been captured in the same clause 4.2.2.5 of TR 38.743 concerning the impacts on F1 interface from the gNB2 perspective. In order to allow the gNB2 to take proper counter-measures following the future CCO state derived at gNB1 side, and based on the F1 impacts captured in TR 38.743, gNB-CU2 shall send to gNB-DU2 a predicted CCO issue (including predicted affected cells and SSBs of gNB1) and “time for predicted CCO issue”. It is worth noting that, same as in legacy CCO, the predicted CCO issue signalled by gNB-CU2 to gNB-DU2 is not a new issue but it is the same issue predicted by gNB-CU1 and signalled to gNB-CU2 as the coverage modification cause over Xn.

Therefore, gNB-CU2 needs to receive over Xn the predicted coverage modification cause (reflecting the predicted CCO issue at gNB1), the “time for predicted CCO issue” (indicating when the CCO issue is predicted to occur), the future CCO state and the “time for future coverage state” both at gNB1.

Proposal 2: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn both the “time for future coverage state” and the “time for predicted CCO issue”.

Upon receiving the future CCO state, the coverage modification cause (i.e., the predicted CCO issue), the “time for future coverage state” and the “time for predicted CCO issue” over Xn from gNB-CU1, the gNB-CU2 should signal to gNB-DU2 over F1: the predicted CCO issue (including affected cells and SSBs of gNB1), the “time for predicted CCO issue” and the “time for future coverage state”.

Proposal 3: RAN3 to agree that, upon receiving over Xn the future CCO state, the coverage modification cause, the “time for future coverage state” and “time for predicted CCO issue”, the neighbour gNB-CU signals to the neighbour gNB-DU over F1: predicted CCO issue (including affected cells and SSBs of the node predicting the CCO issue), “time for predicted CCO issue” and “time for future coverage state”.

The TP for AI/ML BLCR to TS 38.473 for AI/ML-based CCO in the Annex 2 of [5] also reflects the above Proposal 3.

## 2.2 Feedback information AI/ML-based CCO

In the last meeting RAN3 captured the following open point in the meeting minutes [4]:

Information to validate the prediction is needed.

The above open issue is related to the discussion RAN3 had on the feedback information for the AI/ML-based CCO use case, in particular on UE performance feedback which is one of the items captured in TR 38.743 clause 4.2.2.4 along with measured radio resource status and SON reports [3]. Before discussing which kind of measurements (e.g., legacy per UE UE performance, radio-level UE measurements, network-level UE performance, etc.) should the neighbour node report to the node hosting the AI/ML model, it should be clarified in RAN3 what is intended to be validated by such report. As can be seen from the text captured as open point, such text is very generic as it refers to “prediction” without clarifying whether it is the “CCO prediction”, the “future CCO state” (if derived via AI/ML in gNB-DU, which is possible only by implementation) or a combination of the two.

In our view, we should base the discussion according to the definition of “feedback” as captured in TR 37.817 [6]:

## 4.2 Functional Framework Figure 4.2-1. Functional Framework for RAN Intelligence This section introduces the common terminologies related to the functional framework for RAN intelligence illustrated in Figure 4.2-1. For the functions and data/information flows shown in the Figure 4.2-1, whether there is any standardization impact and what is the standardization impact are discussed in clause 5. **** Skip non-relevant text **** - Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself. - Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

Based on the above it is clear that the feedback information is strictly related to the AI/ML model, and for the AI/ML-based CCO the only AI/ML model which is supported by the standard is the one hosted by the gNB(-CU) which predicts the CCO issue. As a consequence, the feedback information that should be discussed in RAN3 should be for evaluating the CCO issue prediction.

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

With reference to the specific information that should be reported at the node hosting the model which predicted the CCO issue, we think it is not enough for the AI/ML model to obtain legacy (Rel-18) per UE UE performance metrics as feedback information. We believe that the local node should also receive node-/cell-/SSB-level UE performance metrics from the neighbour node, since this information may further help to monitor the AI/ML model.

Proposal 5: RAN3 to agree to introduce node-/cell-/SSB-level UE performance feedback over the Xn interface for the AI/ML-based CCO use case.

Moreover, we think it would be beneficial for AI/ML model’s monitoring purposes that the local node could receive from the neighbour node information related to its current CCO state just after the “time for predicted CCO issue”, that is, after the previously predicted CCO issue was expected to occur and the corresponding future CCO state was actually applied by the local node at the “time for future coverage state” to avoid such predicted CCO issue. This would allow the model in the gNB(-CU) to have a clearer view of the AI/ML-based CCO mechanism and to fine-tune the model accordingly.

Proposal 6: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information.

## 2.3 On recommended future CCO state from gNB-CU to gNB-DU

TR 38.743, clause 4.2.2.5, contains also a note (NOTE4) related to the possibility for the gNB-CU to generate a recommended/suggested future CCO state and send it to the gNB-DU as assistance information, so that the gNB-DU could take this information into account in order to determine the actual future CCO state in response of the predicted CCO issue. Below the note mentioned above is excerpted from in the TR [3]:

#### 4.2.2.5 Potential standard impacts Following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18 based on the CCO framework described in TS 38.300 [2] and TS 38.401 [3]. **** Skip non-relevant text **** F1 interface: **** Skip non-relevant text **** NOTE 4: Whether gNB-CU generates the suggested future CCO state and sends it to gNB-DU as assistance information is left to normative discussion. **** Skip non-relevant text ****

In our previous paper in [7] we already provided our view on such scenario. From the legacy CCO mechanism, it could be noted that the gNB-CU actually has visibility of the CCO state and corresponding coverage modification cause information from the gNB-DU (i.e., Coverage Modification Notification IE within the GNB-DU CONFIGURATION UPDATE message) and, if available, from the neighbour gNBs (i.e., Coverage Modification List IE within the NG-RAN NODE CONFIGURATION UPDATE message) related to either another CCO issue detected in the neighbour gNB itself or as a consequence of a coverage configuration change in the concerned gNB-CU. In our understanding, this makes it possible for the AI/ML model in the gNB-CU to infer not only the predicted CCO issue but also the corresponding future CCO state based on historical information such as the previous CCO state from the gNB-DU and neighbour gNB. In this way, the gNB-DU could obtain richer and more meaningful information from the gNB-CU (i.e., not only the predicted CCO issue but also an indication of future CCO state) to solve or mitigate the predicted CCO issue in advance. It should be noted that we are not excluding the possibility that the gNB-DU determines the actual future CCO state for its own cells/SSBs, that is, the gNB-DU could either directly consider the recommended future CCO state as determined by the gNB-CU or determine the actual future CCO state on its own based on the predicted CCO issue from the gNB-CU and by also taking the recommended future coverage state from the gNB-CU as a reference.

Proposal 7: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future CCO state.

# 3. Conclusion

Based on the above discussion, we have the following proposals:

On timing information

Proposal 1: RAN3 to agree to introduce the “time for predicted CCO issue” within the new Predicted CCO Assistance Information IE (within GNB-CU CONFIGURATION UPDATE message), and the “time for future coverage state” in the new Future Coverage Modification Notification IE (within GNB-DU CONFIGURATION UPDATE message), respectively.

Proposal 2: RAN3 to agree that, for the timing information, the gNB(-CU) which predicted the CCO issue sends to a neighbour gNB(-CU) over Xn both the “time for future coverage state” and the “time for predicted CCO issue”.

Proposal 3: RAN3 to agree that, upon receiving over Xn the future CCO state, the coverage modification cause, the “time for future coverage state” and “time for predicted CCO issue”, the neighbour gNB-CU signals to the neighbour gNB-DU over F1: predicted CCO issue (including affected cells and SSBs of the node predicting the CCO issue), “time for predicted CCO issue” and “time for future coverage state”.

The TP for AI/ML BLCR to TS 38.473 for AI/ML-based CCO in the Annex 2 of [5] also reflects the above Proposal 1 and Proposal 3.

On feedback information

Proposal 4: RAN3 to agree that feedback information is needed to evaluate the CCO issue prediction.

Proposal 5: RAN3 to agree to introduce node-/cell-/SSB-level UE performance feedback over the Xn interface for the AI/ML-based CCO use case.

Proposal 6: RAN3 to agree to transfer the current CCO state of the neighbour gNB to the local node over the Xn interface as a feedback information.

On recommended future CCO state from gNB-CU to gNB-DU

Proposal 7: RAN3 to agree that the gNB-CU could provide to the gNB-DU not only the predicted CCO issue but also the corresponding future CCO state as a recommendation: the gNB-DU could either confirm the recommendation from the gNB-CU or use it as a reference in determining the actual future CCO state.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-247416, (TPs for AI/ML BLCR to TS 38.423 and AI/ML BLCR to TS 38.473) Signalling details for AI/ML-based Coverage and Capacity Optimization, Huawei

- 3GPP TR 37.817 “Study on enhancement for Data Collection for NR and EN-DC”, Rel-17

- R3-243467, (TP to TR 38.743) AI/ML-based Coverage and Capacity Optimization, Huawei