3GPP TSG-RAN WG3 Meeting #126	R3-247418

Orlando, US, 16th – 22nd November, 2024

Title: 	(TP for AI/ML BLCR to TS 38.473) Split architecture support for Rel-18 “AI/ML for NG-RAN” use cases

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to split architecture support for Rel-18 use cases.

This contribution analyzes the technical enablers for specifying support of split architecture for Rel-18 use cases, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Discussion

## 2.1 On gNB-DU Energy Cost (EC) reporting over F1

In the last meeting, as part of the Rel-18 leftover related to split architecture support for Rel-18 use cases, RAN3 agreed to introduce the Data Collection Reporting Initiation and Data Collection Reporting procedures over F1, as done in Rel-18 for the Xn interface, to enable the reporting of the measured Energy Cost (EC) from the DU to the CU.

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

Similar to the procedure over Xn, the Data Collection Reporting Initiation procedure over F1 is used by the gNB-CU to request from the gNB-DU the reporting of information for, e.g., AI/ML in NG-RAN, by using non UE-associated signalling.

As part of the Successful Operation, the gNB-CU sends the DATA COLLECTION REQUEST message to the gNB-DU to start (or stop) the information reporting and, following the reception of this message, the gNB-DU shall initiate (or stop) the requested information reporting. To enable this mechanism the Registration Request for Data Collection IE needs to be included in the DATA COLLECTION REQUEST message.

Proposal 1: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Registration Request for Data Collection IE to start and stop the requested information reporting by the gNB-DU.

Proposal 2: The Registration Request for Data Collection IE is an extensible ENUMERATED type with “start” and “stop” codepoints.

To indicate the information that need to be reported by the gNB-DU to the gNB-CU, the Report Characteristics for Data Collection IE needs to be included in the DATA COLLECTION REQUEST message as well.

Proposal 3: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Report Characteristics for Data Collection IE to indicate the information that the gNB-DU reports to the gNB-CU.

In the last meeting it was agreed that the gNB-DU reports to the gNB-CU its own measured EC, which is a node-level measurement. For the Data Collection Reporting Initiation procedure over Xn the partial reporting mechanism was specified. This allows the reporting node to indicate in the DATA COLLECTION RESPONSE message the list of measurements that the reporting node cannot initiate, along with an appropriate cause value. Considering that RAN3 will discuss the reporting of UE performance measurements over F1, and hence multiple measurements could be requests to the be reported by the gNB-DU other than the measured EC, we think that partial reporting should also be supported for the Data Collection Reporting Initiation over F1 as well.

Proposal 4: RAN3 to agree that partial reporting is supported in the Data Collection Reporting Initiation procedure over F1.

If Proposal 4 above is agreed, we need to discuss the structure of the DATA COLLECTION RESPONSE message (still part of the Successful Operation). By recalling that the gNB-DU’s measured EC is a node-level measurement, and following the design of the DATA COLLECTION RESPONSE message over Xn, the DATA COLLECTION RESPONSE message sent by the gNB-DU to the gNB-CU should include the Node Measurement Initiation Result List IE indicating the list of measurements that failed to be initiated by the gNB-DU (with an appropriate cause value).

Proposal 5: RAN3 to agree to introduce the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

Proposal 6: RAN3 to agree to indicate the ‘Energy Cost’ as the first bit of the Node Measurement Failed Report Characteristics bitmap (within the Node Measurement Initiation Result List IE).

Still based on the design of the Data Collection Reporting Initiation procedure over Xn, the DATA COLLECTION REQUEST message allows to configure both one-time and periodic reporting of information, by means of the Reporting Periodicity for Data Collection IE. We think that similar design should also be followed for F1, to allow periodic reporting of the gNB-DU’s EC as well as of UE performance.

Proposal 7: RAN3 to agree to introduce the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message.

When it comes to the Data Collection Reporting procedure, instead, the DATA COLLECTION UPDATE message is used. In XnAP this message includes both cell-level and node-level measurements that could be reported based on the configuration in the associated DATA COLLECTION REQUEST message. For F1, and considering that RAN3 so far only agreed to report the gNB-DU’s measured EC (i.e., a node-level measurement), we think that at least the Node Associated Info Result IE should be included in the DATA COLLECTION UPDATE message over F1.

Proposal 8: RAN3 to agree to introduce the Node Associated Info Result IE in the DATA COLLECTION UPDATE message over F1.

Proposal 9: The Node Associated Info Result IE includes the measured Energy Cost of the gNB-DU.

It is well known that the Data Collection Reporting Initiation and Data Collection Reporting over Xn originate from the Resource Status Reporting Initiation and Resource Status Reporting procedures, respectively. For F1 it has been agreed in the last RAN3 meeting to basically mirror the Data Collection Reporting Initiation and Data Collection Reporting procedures specified over Xn. However, by checking F1AP non UE-associated procedures, e.g., the Resource Status Reporting Initiation and Resource Status Reporting, it can be noticed that messages used for these procedures include a mandatory IE which is termed as Transaction ID. This IE is determined by the initiating peer of a procedure and uniquely identifies a procedure among all ongoing parallel procedures of the same type initiated by the same protocol peer; moreover, messages belonging to the same procedure shall use the same Transaction ID IE. If the Transaction ID IE is missing from a certain non UE-associated procedure, this may cause problems in protocol error handling (refer to TS 38.473 clause 10). Based on that, we think that the Transaction ID IE should be introduced in messages used for Data Collection Reporting Initiation and Data Collection Reporting procedures over F1.

Proposal 10: RAN3 to agree to introduce the mandatory Transaction ID IE in DATA COLLECTION REQUEST, DATA COLLECTION RESPONSE, DATA COLLECTION FAILURE and DATA COLLECTION UPDATE messages.

The TP for the AI/ML BLCR to TS 38.473 in the Annex is based on the above proposals.

## 2.2 On reporting measured UE performance

During the Rel-19 SI, for split architecture support, the use case description and potential standard impacts have been investigated and reported in TR 38.743 clause 5.2 as follows:

## 5.2 Split architecture support for Rel-18 use cases ### 5.2.1 Use case description The split architecture should be enhanced to support the Rel-18 use cases, e.g, Load Balancing, Energy Saving, and Mobility Optimization. In case of CU-DU architecture, the following solutions are possible: - AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU(-CP); - AI/ML Model Training and Model Inference are both located in the gNB-CU(-CP). ### 5.2.2 Potential Standard impacts The following standard impacts are listed for subsequent Rel-19 normative work compared with what was specified during Rel-18: - The details of signaling measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need further discussion during normative phase. - Measured Energy Cost from gNB-DU to gNB-CU.

TR 38.743 captures that signalling details for transferring measured UE performance metrics from gNB-DU to gNB-CU-CP and/or from gNB-CU-UP to gNB-CU-CP need to be discussed during the Rel-19 WI phase.

The scenario we propose to consider relates to UE performance metrics collection at the gNB-CU-CP of a split target gNB from the gNB-CU-UP(s) and/or gNB-DU(s) controlled by the concerned gNB-CU-CP – such measurement collection is triggered after an AI/ML-based mobility decision (i.e., handover) taken by the source gNB is successfully executed. This scenario is illustrated in Figure 1.

Figure 1 - Scenario for reporting UE performance from split target gNB to source gNB.

RAN3 introduced in Rel-18 the Average UE Throughput DL, Average UE Throughput UL, Average Packet Delay and Average Packet Loss as the per-UE metrics representing the UE performance to be reported from target gNB to source gNB over Xn. In case the target gNB is a split node, we think it is important that the target gNB-CU-CP is able to collect UE performance information, so that the target gNB-CU-CP can report the aggregated UE performance to the source gNB for, e.g., AI/ML model monitoring in the source node.

As per TS 28.558, the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss are measured in the gNB-DU.

While, for the Average Packet Delay, TS 28.558 specified the delay components for both DL and UL – refer to Figure 2 and Figure 3, respectively – some of which are measured by the gNB-DU and others by the gNB-CU-UP.

Figure 2 - RAN part of DL packet delay measurement.

Figure 3 - RAN part (including UE) of UL packet delay measurement.

Table 1 – UE performance metrics, where they are measured.

|  | gNB-DU | gNB-CU-UP |
| --- | --- | --- |
| Average UE Throughput DL | X |  |
| Average UE Throughput UL | X |  |
| Average Packet Loss | X |  |
| DL packet delay components | D1, D2 | D3, D4 |
| UL packet delay components | D2.1, D2.2 | D2.3, D2.4 |

The above Table 1 shows where each of the UE performance metrics is measured, and it is clear that Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss should be reported to the gNB-CU-CP via F1 signalling.

- Proposal 11: RAN3 to agree on exchanging over F1 (gNB-DU to gNB-CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

The exchange of Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss can be performed via the new F1AP Data Collection Reporting Initiation and Data Collection Reporting procedures agreed in the last RAN3 meeting for the reporting of the measured gNB-DU’s EC.

For the Average Packet Delay, instead, since there are multiple components, some of them measured at gNB-DU level and others at gNB-CU-UP level, we think that the gNB-CU-CP could be made aware of the overall Average Packet Delay via one of the following options:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively; it is then up to gNB-CU-CP to aggregate the received measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1; with respect to Option 1, in this way it could still be possible for the gNB-CU-CP to have visibility of the different delay components in gNB-DU and gNB-CU-UP (same as in Option 2).

Proposal 12: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1.

To support the finer-grained measurement transfer, Option 2 and 3 can be adopted to inform the gNB-CU-CP about the specific component of the Average Packet Delay. Between them, Option 2 is more reasonable and efficient since the components are directly sent to gNB-CU-CP from the network entity where such components are measured.

Proposal 13: RAN3 to agree to select Option 2, i.e., gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement.

If Option 2 is agreed, the gNB-DU-measured Average Packet Delay components can be sent directly from gNB-DU to gNB-CU-CP by means of the new F1AP Data Collection Reporting Initiation and Data Collection Reporting procedures agreed in the last RAN3 meeting. While for the gNB-CU-UP-measured Average Packet Delay components a new procedure for E1 – also mirroring the XnAP’s Data Collection Reporting (Initiation) procedures – can be introduced.

Proposal 14: RAN3 to agree to introduce new E1AP procedures similar to the XnAP’s Data Collection Reporting Initiation and Data Collection Reporting.

In the last RAN3 meetings there were a lot of discussions related to the granularity of UE performance. For Rel-18, the granularity has been specified to be per-UE and it is still unclear how to derive such per-UE UE performance metrics to be reported to the source gNB over Xn from the corresponding per-DRB measurements as currently specified in TS 28.558/TS 38.314 and collected by the target gNB. Still referring to Figure 1, the focus of the discussion is on collecting at gNB-CU-CP of the target split gNB the UE performance metrics components reported by gNB-DU and gNB-CU-UP. In this case we think that per-DRB measurements reporting performed by gNB-DU and gNB-CU-UP as per current specifications is feasible. This would allow the gNB-CU-CP of the target split gNB to collect UE performance information at the finest granularity as currently possible. How the gNB-CU-CP of the target split gNB will report UE performance over Xn to the source gNB is pending the outcome of the RAN3 discussion on how to derive per-UE UE performance.

Proposal 15: RAN3 to agree that UE performance metrics components are reported by the gNB-DU and gNB-CU-UP on a per-DRB granularity to the gNB-CU-CP.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals:

On F1 signalling design for reporting measured EC from gNB-DU to gNB-CU

Proposal 1: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Registration Request for Data Collection IE to start and stop the requested information reporting by the gNB-DU.

Proposal 2: The Registration Request for Data Collection IE is an extensible ENUMERATED type with “start” and “stop” codepoints.

Proposal 3: RAN3 to agree that the DATA COLLECTION REQUEST message includes the Report Characteristics for Data Collection IE to indicate the information that the gNB-DU reports to the gNB-CU.

Proposal 4: RAN3 to agree that partial reporting is supported in the Data Collection Reporting Initiation procedure over F1.

Proposal 5: RAN3 to agree to introduce the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

Proposal 6: RAN3 to agree to indicate the ‘Energy Cost’ as the first bit of the Node Measurement Failed Report Characteristics bitmap (within the Node Measurement Initiation Result List IE).

Proposal 7: RAN3 to agree to introduce the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message.

Proposal 8: RAN3 to agree to introduce the Node Associated Info Result IE in the DATA COLLECTION UPDATE message over F1.

Proposal 9: The Node Associated Info Result IE includes the measured Energy Cost of the gNB-DU.

Proposal 10: RAN3 to agree to introduce the mandatory Transaction ID IE in DATA COLLECTION REQUEST, DATA COLLECTION RESPONSE, DATA COLLECTION FAILURE and DATA COLLECTION UPDATE messages.

The TP for the AI/ML BLCR to TS 38.473 in the Annex is based on the above proposals.

On reporting measured UE performance over F1 and E1

- Proposal 11: RAN3 to agree on exchanging over F1 (gNB-DU to gNB-CU-CP) the Average UE Throughput DL, Average UE Throughput UL and Average Packet Loss.

Proposal 12: RAN3 to decide which option to select for the Average Packet Delay measurement in split architecture; possible options are:

- Option 1: gNB-CU-UP aggregates all the different delay components, it derives the overall Average Packet Delay measurement and it sends a unique measurement to gNB-CU-CP;

- Option 2: gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement;

- Option 3: gNB-DU sends its own measurements of the Average Packet Delay components to gNB-CU-UP over F1 and gNB-CU-UP forwards such measurements to gNB-CU-CP transparently over E1.

Proposal 13: RAN3 to agree to select Option 2, i.e., gNB-DU and gNB-CU-UP independently send their own measurements of the Average Packet Delay components to gNB-CU-CP over F1 and E1, respectively, and gNB-CU-CP aggregates the measurements to derive the overall Average Packet Delay measurement.

Proposal 14: RAN3 to agree to introduce new E1AP procedures similar to the XnAP’s Data Collection Reporting Initiation and Data Collection Reporting.

Proposal 15: RAN3 to agree that UE performance metrics components are reported by the gNB-DU and gNB-CU-UP on a per-DRB granularity to the gNB-CU-CP.

# 4. Reference

- RP-242385, New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

# 5. Annex - TP for AI/ML BLCR to TS 38.473

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

## 8.X	Data Collection Reporting procedures

### 8.X.1	Data Collection Reporting Initiation

#### 8.X.1.1	General

This procedure is used by the gNB-CU to request from the gNB-DU node the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.1.2	Successful Operation

Figure 8.X.1.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU initiates the procedure by sending the DATA COLLECTION REQUEST message to the gNB-DU to start information reporting or to stop information reporting. Upon receipt, the gNB-DU:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to "start"; or

-	shall stop all measurements and terminate the reporting in case the Registration Request for Data Collection IE is set to "stop".

If the gNB-DU is capable of providing all of the requested information, it shall initiate the information reporting as requested by the gNB-CU and respond with the DATA COLLECTION RESPONSE message.

If the gNB-DU is capable of providing some but not all of the requested information, it shall initiate the information reporting for the admitted requested information and include the Node Measurement Initiation Result List IE in the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST message is present, this indicates the periodicity for the reporting of configured measurement objects. The gNB-DU shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

Interaction with the Data Collection Reporting procedure

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST message indicates the type of objects the gNB-DU performs measurements on. The gNB-DU shall include in the DATA COLLECTION UPDATE message:

-	the Energy Cost IE, if the first bit, "Energy Cost" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by the gNB-DU.

#### 8.X.1.3	Unsuccessful Operation

Figure 8.X.1.3-1: Data Collection Reporting Initiation, unsuccessful operation

If none of the requested information can be initiated, the gNB-DU shall send the DATA COLLECTION FAILURE message with an appropriate cause value.

#### 8.X.1.4	Abnormal Conditions

If the initiating gNB-CU does not receive either the DATA COLLECTION RESPONSE message or the DATA COLLECTION FAILURE message, the gNB-CU may reinitiate the Data Collection Reporting Initiation procedure towards the same gNB-DU, provided that the content of the new DATA COLLECTION REQUEST message is identical to the content of the previously unacknowledged DATA COLLECTION REQUEST message with the same Transaction ID.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "stop" and if the gNB-DU Measurement ID value received in the DATA COLLECTION REQUEST message is not used, the gNB-DU shall initiate DATA COLLECTION FAILURE message with an appropriate cause value.

If in the Report Characteristics for Data Collection IE bitmap all bits are set to "0" in the DATA COLLECTION REQUEST message, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

If the gNB-DU receives a DATA COLLECTION REQUEST message which includes the Registration Request for Data Collection IE set to "start" and the gNB-CU Measurement ID IE corresponding to an existing on-going Data Collection reporting, for which a different Transaction ID is used, then the gNB-DU shall initiate a DATA COLLECTION FAILURE message with an appropriate cause value.

### 8.X.2	Data Collection Reporting

#### 8.X.2.1	General

This procedure is initiated by the gNB-DU to report information accepted by the gNB-DU following a successful Data Collection Reporting Initiation procedure for the purpose of, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.X.2.2	Successful Operation

Figure 8.X.2.2-1: Data Collection Reporting, successful operation

The gNB-DU shall report the accepted information in DATA COLLECTION UPDATE message. The accepted information is the information that was successfully initiated during the preceding Data Collection Reporting Initiation procedure.

#### 8.X.2.3	Unsuccessful Operation

Not applicable.

#### 8.X.2.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.2.X	Data Collection Reporting Messages

#### 9.2.X.1	DATA COLLECTION REQUEST

This message is sent by gNB-CU to gNB-DU to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestForDataCollectionStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-DU is requested to report. First Bit = Energy Cost. Other bits are ignored by the gNB-DU. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED(500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestForDataCollectionStop | This IE shall be present if the Registration Request for Data Collection IE is set to the value “stop”. |
| ifRegistrationRequestForDataCollectionStart | This IE shall be present if the Registration Request for Data Collection IE is set to the value “start”. |

#### 9.2.X.2	DATA COLLECTION RESPONSE

This message is sent by gNB-DU to gNB-CU to indicate that the requested information, for all or part of the measurement objects included in the reporting, is successfully initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | C-ifRegistrationRequestForDataCollectionStop |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Node Measurement Initiation Result List |  | 0..1 |  | List of measurement objects that failed to be initiated in the node. | YES | reject |
| >Node Measurement Initiation Result Item |  | 1 .. <maxFailedMeasPerNode> |  |  | – |  |
| >>Node Measurement Failed Report Characteristics | M |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement objects that failed to be initiated in the gNB-DU. First Bit = Energy Cost. Other bits are ignored by the gNB-CU. | – |  |
| >>Cause | M |  |  | Failure cause for measurement objects for which the measurement cannot be initiated. | – |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.3	DATA COLLECTION FAILURE

This message is sent by gNB-DU to the gNB-CU indicate that for all of the requested objects the reporting cannot be initiated.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.X.4	DATA COLLECTION UPDATE

This message is sent by gNB-DU to the gNB-CU to report the requested information.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-CU | YES | reject |
| gNB-DU Measurement ID | M |  | INTEGER (0..4095,...) | Allocated by gNB-DU | YES | ignore |
| Node Associated Info Result |  | 0..1 |  |  | YES | ignore |
| >Energy Cost | O |  | INTEGER (0..10000,…) | The node level measured Energy Consumption index. Value 0 indicates the minimum measured Energy Consumption and 10000 indicates the maximum measured Energy Consumption. | - | - |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>