3GPP TSG-RAN WG3 Meeting #126	R3-247419

Orlando, US, 16th – 22nd November, 2024

Title: 	On Continuous MDT for AI/ML

Source: 	Huawei

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In the last meeting RAN3 started discussing the objectives of the new Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1], which follows the SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [2] and whose outcome is summarized in TR 38.743 [3].

One of the objectives of the WI is to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to Continuous MDT collection targeting the same UE across RRC states.

This contribution analyzes the technical enablers for specifying support of Continuous MDT, and provides proposals for discussion based on the outcome from the last RAN3 meeting.

# 2. Background

The main motivation for introducing Continuous MDT arose during the Rel-18 WI “AI/ML for NG-RAN” [4], where the issue on how to collect measurements from UE(s) to be used as datasets for AI/ML models’ (re-)training was initially discussed. Back then it was argued that, for the AI/ML model (re-)training to be successful, the MDT data collected from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. It was claimed that, when the UE enters to connected state from idle/inactive or when the UE is subject to connected mode mobility (handover), consecutive (without time gaps) information would not be collected, with the consequence that the data gathered from the UE would not constitute a valid training dataset for AI/ML models’ (re-)training. By means of the Continuous MDT, that is, an enhancement of the current MDT mechanism suited for AI/ML models’ (re-)training, the continuous collection of MDT data from the same UE during the same RRC state as well as across RRC states can be enabled.

During the Rel-19 SI, for Continuous MDT, the use case description and problems to be solved have been acknowledged and captured in TR 38.743 clause 5.4 as follows:

## 5.4 Continuous MDT collection targeting the same UE across RRC states ### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. The Data Collection continuity in this scenario can be split into two tasks as below: - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE. ### 5.4.2 Potential Standard impacts Potential solutions addressing the problems above can be discussed during normative phase.

In the last meeting RAN3 agreed the following [5]:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN. Further discussions are needed on how to define a temporary identifier for Continuous MDT. Further discussions are needed on the solution for measurement correlation in Continuous MDT.

In the next Section 3 we provide our views on how to properly address Problem A and Problem B as described in TR 38.743 clause 5.4.1 and as reflected in the above reported open issues from last RAN3 meeting, so to actually enable the Continuous MDT feature in Rel-19.

# 3. Discussion

In the last meeting RAN3 achieved a first high-level agreement for Continuous MDT:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

The above agreement reflects the fact that data collection serves the purpose of providing sufficient amount of data as input for AI/ML model (re-)training, meaning that data needs to be collected from as many UEs as possible, under different radio environments and RRC states. Fulfilling such requirements for proper data collection from UEs can be possible via management-based MDT. Not to mention that TR 38.743, clause 5.4.1, explicitly refers to management-based MDT for the Continuous MDT problem statement.

In the following we provide our views on the two open aspects as captured in the last RAN3 meeting minutes, that is [5]:

Further discussions are needed on how to define a temporary identifier for Continuous MDT. Further discussions are needed on the solution for measurement correlation in Continuous MDT.

## 3.1	Trace correlation issue

The main limitation with management-based MDT to be used as the baseline approach for Continuous MDT, however, is related to the MDT data anonymization requirement, which impacts Problem B (trace correlation) as described in clause 5.4.1 of TR 38.743. TS 32.422, in fact, states that anonymization of MDT data gathered via management-based MDT is mandatory for both MDT types (the so-called Job Types), i.e., Immediate MDT and Logged MDT. When the UE is configured to report both Immediate MDT and Logged MDT measurements, the UE performs measurements while in different RRC states (Immediate MDT measurements when in connected mode, and Logged MDT measurements when in idle/inactive mode) and reports them to the serving gNB in different time instances and anyway only when the UE is in connected mode. As a consequence, the gNB will provide such MDT reports to the Trace Collection Entity (TCE) within separate Trace Files and in different time instances as well.

In order to actually implement the Continuous MDT concept and, more specifically, to create datasets that are represented as a time series of continuous information and that can be used for proper (re-)training of AI/ML models – i.e., data series not affected by time interruptions due to e.g. stop of UE data collection and successive resuming of this process – it is needed that TCE identifies that the Immediate and Logged MDT measurement reports are from the same UE so that the TCE can correlate these MDT reports, hence creating a unique dataset.

However, as per the anonymization requirement of management-based MDT data, the MDT reports provided to the TCE are anonymized on purpose, hence it seems that currently there are no means for the TCE to recognize which MDT reports are provided by the same UE.

It is worth to highlight that there is no need for the TCE to identify the UE which provided the Immediate and Logged MDT reports, but what it is actually needed is that the TCE identifies which MDT reports are provided by the same UE so that the TCE can correlate these reports and hence create a unique dataset which can be used for AI/ML (re-)training purposes.

Proposal 1: RAN3 to agree that, for Continuous MDT, the TCE needs to identify which management-based MDT reports (Logged and Immediate) are provided by the same UE and there is no need for the TCE to identify/recognize that UE.

In Rel-18 discussions there was an attempt to check whether existing identifiers, e.g. IMEI-TAC, NR CGI, TR/TRSR could be used for trace correlation at TCE level [6]. In our previous contribution in [6] we provided an in-depth analysis of such identifiers and concluded that they are not suited for being used in the trace correlation in TCE.

One possibility to address this issue could be to introduce a new Job Type in SA5 specifications which allows to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR). This should be possible because from SA5 TS 32.422 §5.9a it is already possible to configure a Job Type for two modes – refer to, e.g., Job Type value 3 which is Immediate MDT and Trace. Hence, if a new Job Type such as “Immediate MDT and Logged MDT” is introduced, then the same TR could be assigned to the MDT session corresponding to this new Job Type. TS 32.422 states that the OAM sends a Trace Session activation request to the gNB with configuration parameters including the Job Type and the TR. Then the gNB activates the MDT functionality for the selected UE and assigns a Trace Recording Session Reference (TRSR) to the selected UE.

Therefore, in management-based MDT, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized), the TCE could perform such correlation by “collating” those MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence solving Problem B (trace correlation) in [3].

Since the above solution has impacts on SA5, it is proposed to liaise SA5 asking for feedback on its feasibility.

Proposal 2: RAN3 to agree on the solution based on a new Job Type “Immediate MDT and Logged MDT” with a single session-specific TR and UE-specific TRSR for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex 1 of this contribution.

## 3.2 Measurement continuity across UE’s RRC state changes and during the same RRC state

Aside Problem B (trace correlation), a proper Continuous MDT solution needs to address also Problem A (measurement continuity) in [3], that is, how to ensure that the same UE keeps on collecting MDT measurements both across different RRC states (i.e., after idle/inactive to connected state transition) as well as during the same RRC state (that is, after handover from source gNB to target gNB).

For the first case (i.e., after idle/inactive to connected state transition), the scenario – detailed in Annex 2 – involves a UE which has been configured with Immediate and Logged MDT and, after having performed Logged MDT measurements while in idle/inactive state, it re-establishes/resumes its RRC connection in a new cell and the UE reports the availability of Logged MDT measurements to the (potentially new) serving gNB. In order to avoid time interruptions in the measurement process, the UE should be re-configured to perform Immediate MDT immediately after it re-establishes/resumes the RRC connection, i.e., just after it moves to connected state.

Considering that if the OAM provides a management-based MDT configuration with a specific Job Type, i.e., “Immediate MDT and Logged MDT”, to gNBs in a certain area, this means that UEs selected by the NG-RAN and configured according to that particular OAM configuration are the only ones for which the Continuous MDT feature is requested to be enabled. This is in line with the agreement from the last RAN3 meeting [5]:

RAN3 assumes that the OAM configures Management Based MDT for Continuous MDT towards the NG-RAN.

To ensure measurement continuity across different RRC states it could be possible to rely on a network-based solution where the gNB serving a Continuous MDT UE sends parts of the MDT configuration to the AMF (e.g., within the NG: UE CONTEXT RELEASE REQUEST) which stores such information. These parts of the MDT configuration can be stored within the UE context in the 5GC/AMF by default only for those UEs selected for Continuous MDT, i.e., configured with a management-based MDT configuration with Job Type “Immediate MDT and Logged MDT”. In this manner, once the UE re-establishes/resumes its RRC connection, the (potentially new) serving gNB will be provided by the 5GC/AMF with UE context data including user consent information as well as the stored parts of the MDT configuration (e.g., within the NG: INITIAL UE CONTEXT SETUP REQUEST). These stored parts of the MDT configuration can be e.g. the Job Type “Immediate MDT and Logged MDT” and the TRSR previously allocated to the Continuous MDT UE by the (old) serving gNB, and these pieces of information can be used as an indication that the UE was previously selected for Continuous MDT, so that the (potentially new) serving gNB can re-select the same UE and re-configure the UE to continue performing the Immediate MDT measurements in connected mode. This removes the burden of the (potentially new) serving gNB to recognize which UE has been previously selected and configured with the Continuous MDT by the (old) serving gNB, because it will become aware of this by simply checking the presence of the parts of the MDT configuration in the UE context data provided by the 5GC/AMF. If present, this means that the UE was previously selected for Continuous MDT by the (old) serving gNB and needs to be re-selected and re-configured again.

Proposal 3: RAN3 to discuss and agree on a network-based solution for Continuous MDT for measurement continuity across RRC state changes (i.e., after idle/inactive to connected state transition) based on storing in AMF/5GC at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned to the Continuous MDT UE. The stored information is then provided by the AMF/5GC (pending availability of the user consent) to the (potentially new) serving gNB so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

Let’s now discuss the second scenario for Problem A (measurement continuity) in [3], that is, how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state (that is, after handover from source gNB to target gNB). This scenario, detailed in Annex 3, also addresses the TRSR allocation conflict issue between source and target gNB for the handed over UE, whose solution is also applicable for the scenario previously described and detailed in Annex 2.

To ensure measurement continuity during the same RRC state (i.e., connected mode) it could still be possible to rely on the same network-based solution. Due to radio conditions the UE is handed over to a target gNB, therefore the source gNB initiates handover to the target gNB and, as per current TS 32.422, the UE stops the ongoing Immediate MDT measurements performed while under the coverage of the source gNB. As part of the Handover Preparation procedure, the source gNB sends to target gNB the user consent information for the UE and at least the Job Type “Immediate MDT and Logged MDT” and the TRSR previously allocated to the UE by the source gNB. After handover completion, the target gNB re-selects and re-configures the UE to continue performing MDT measurements by simply checking the presence of the Job Type “Immediate MDT and Logged MDT” and the TRSR previously allocated by the source gNB. By assuming that the TRSR previously allocated by the source cannot be re-assigned because this TRSR is already assigned to another UE served by target gNB, a new TRSR (different from the one allocated by the source gNB) is allocated to the UE. In doing so, and for the purpose of trace correlation at TCE, it is important to keep the mapping relationship of TRSRs allocated to the UE, and we think that this mapping relationship can be kept by the target gNB. After the UE reports the Immediate MDT measurements to the target gNB, the target gNB sends these measurement report to the TCE including the couple TR/TRSR and the mapping relationship of TRSRs allocated to the concerned UE. In this way, the TCE correlates the MDT reports from the source gNB marked with the couple TR/TRSR with the MDT reports from the target marked with a different couple TR/TRSR (note that the TR is the same in source and target, while the TRSR is different): this is possible because TCE also receives the mapping relationship of TRSRs allocated to UE by the source and target gNBs.

Proposal 4: RAN3 to discuss and agree on a network-based solution for Continuous MDT for measurement continuity during the same RRC state (i.e., connected mode mobility) by providing from source gNB to target gNB in the Handover Preparation at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB to the Continuous MDT UE. The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

To address the issue of TRSR allocation conflicts, the same approach as described above can be reused in case of measurement continuity across RRC states where, in case the new serving gNB of the UE cannot re-assign the same TSRS allocated by the old serving gNB (and received as part of the with UE context data provided by the 5GC/AMF), the new serving gNB assigns a new TRSR and keeps the mapping relationship of TRSRs allocated to the UE for the purpose of trace correlation at the TCE.

Proposal 5: RAN3 to discuss and agree on the new serving gNB (or target gNB in case of handover) to keep the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between old serving gNB (or source gNB) and new serving gNB (or target gNB). The new serving gNB (or target gNB) provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation.

# 4. Conclusion

Based on the discussion in this paper, we provide the following proposals:

Trace correlation issue

Proposal 1: RAN3 to agree that, for Continuous MDT, the TCE needs to identify which management-based MDT reports (Logged and Immediate) are provided by the same UE and there is no need for the TCE to identify/recognize that UE.

Proposal 2: RAN3 to agree on the solution based on a new Job Type “Immediate MDT and Logged MDT” with a single session-specific TR and UE-specific TRSR for solving Problem B (trace correlation). A LS to SA5 should be sent asking for feedback on the feasibility of such solution in Rel-19.

A draft LS to SA5 can be found in the Annex 1 of this contribution.

Measurement continuity across UE’s RRC state changes and during the same RRC state

Proposal 3: RAN3 to discuss and agree on a network-based solution for Continuous MDT for measurement continuity across RRC state changes (i.e., after idle/inactive to connected state transition) based on storing in AMF/5GC at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned to the Continuous MDT UE. The stored information is then provided by the AMF/5GC (pending availability of the user consent) to the (potentially new) serving gNB so that the same UE can be re-selected and re-configured with management-based MDT immediately after it re-establishes/resumes its RRC connection.

Proposal 4: RAN3 to discuss and agree on a network-based solution for Continuous MDT for measurement continuity during the same RRC state (i.e., connected mode mobility) by providing from source gNB to target gNB in the Handover Preparation at least Job Type “Immediate MDT and Logged MDT” and TRSR assigned by the source gNB to the Continuous MDT UE. The provided information, along with availability of the user consent, can be used by the target gNB in order for the same UE to be re-selected and re-configured with management-based MDT immediately after handover completion.

Proposal 5: RAN3 to discuss and agree on the new serving gNB (or target gNB in case of handover) to keep the mapping relationship of TRSRs allocated to the Continuous MDT UE in case of TRSR allocation conflicts between old serving gNB (or source gNB) and new serving gNB (or target gNB). The new serving gNB (or target gNB) provides the mapping relationship of TRSRs to the TCE for the sake of trace correlation.

# 5. Reference

- RP-242385, “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#105

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-233441, “Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (CMCC, Ericsson), 3GPP RAN#102

- 3GPP RAN3#125b Chairman’s notes (RAN3_125bis_agenda_20241018_1430_EOM.doc)

- R3-243471, On Continuous MDT for AI/ML, Huawei

# 6. Annex 1 – Draft LS to SA5

3GPP TSG-RAN3 Meeting #126	R3-24abcd

Orlando, US, 16th – 22nd November, 2024

Title:	[Draft] LS on Continuous MDT for AI/ML model training in OAM

Response to:

Release:	Release 19

Work Item:	NR_AIML_NGRAN_enh-core

Source:	Huawei [to be RAN3]

To:	SA5

Cc:	RAN2

Contact person:	Damiano Rapone

damiano.rapone@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

As part of the Rel-19 Work Item on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN, RAN3 agreed to specify an enhancement of the MDT framework termed as Continuous MDT in order to continuously collect measurements from the same UE during the same RRC state (i.e., during connected mode mobility) and across RRC states (i.e., after idle/inactive to connected mode transition) that could be used for the AI/ML model (re-)training in OAM.

In order for the AI/ML model (re-)training to be successful the collected data from the UE should be consecutive – i.e., data series not affected by time interruptions due to, e.g., stop of UE data collection and successive resuming – and a time series of information with a certain granularity. RAN3 discussed solutions based on management-based MDT to tackle the following requirements as in TR 38.743:

- measurement continuity: how to ensure that the same UE keeps on collecting MDT measurements during the same RRC state and across different RRC states;

- trace correlation: how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

The above requirements could be fulfilled if a new Job Type is introduced in SA5 specifications, which allows the OAM to configure a MDT session encompassing Immediate MDT and Logged MDT simultaneously, and assigning to this MDT session the same Trace Reference (TR), while the Trace Recording Session Reference (TRSR) is assigned by the gNB on a per-UE basis within the same MDT session identified by the specific TR. In this manner, a single TR could be assigned to an MDT session with a Job Type equal to “Immediate MDT and Logged MDT”, and this configuration is valid for all UEs selected by the NG-RAN to perform management-based MDT. In order to then correlate the Immediate and Logged MDT reports from the same UE (properly anonymized, e.g., via IMEI-TAC), the TCE performs such correlation by “collating” the MDT reports having the same couple TR/TRSR: the TR links the MDT reports one to each other (as they both belong to the same MDT session) while the TRSR links the MDT reports to a specific UE, hence realizing the Continuous MDT for the UE.

# 2	Actions

To SA5

ACTION: 	RAN3 respectfully asks SA5 to take the above solution description into consideration and provide feedback to RAN3 on its feasibility in Rel-19.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127				2025-02-17 – 2025-02-21				Athens, GR

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN

# 7. Annex 2 – Continuous MDT UE handling across different RRC states

How to handle a Continuous MDT UE across different RRC states (and TCE correlation as well) is summarized in the following Figure 1, where user consent of the UE is also handled based on current specifications (e.g., TS 32.422), and changes with respect to legacy management-based MDT are highlighted in yellow.

Figure 1 - Network-based solution for management-based Continuous MDT: TCE correlation and measurement continuity across RRC state changes (from idle/inactive to connected transition).

- As part of the UE Context Setup procedure the AMF – which previously stored user consent information for UE1 from the UDM, not shown in Figure 1 – provides the gNB1 with user consent information if the user is within its home operator’s PLMNs and the user has given its consent, and user consent information of UE1 are stored in gNB1;

- OAM provides the gNB1 (and other gNBs in the Area scope, i.e., gNB2) with a management-based MDT configuration having Job Type “Immediate MDT and Logged MDT” and a corresponding TR “TR#1”;

- gNB1 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 1., UE1 is selected for Continuous MDT

- gNB1 configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM and assigns a UE-specific TRSR “TRSR#1”;

- UE1 is in connected mode, hence it performs and reports to gNB1 the Immediate MDT measurements according to the management-based MDT configuration provided by the gNB1;

- gNB1 saves the received Immediate MDT measurements into a Trace Record file;

- The Trace Record file containing the Immediate MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- We now consider the case where UE1 moves to idle mode: in this RRC state UE1 performs (and logs) Logged MDT measurements according to the management-based MDT configuration provided by the gNB1 in Step 4.;

- As part of the UE Context Release procedure the gNB1 send at least the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 to the AMF which stores such information;

- We now assume that UE1 triggers transition from idle to connected mode under a new serving gNB, i.e., gNB2; at this point gNB2 is not aware that UE1 was previously selected by gNB1 for Continuous MDT measurements;

- At the end of the RRC Setup procedure involving UE1 and gNB2, gNB2 receives the availability indication from UE1 related to Logged MDT measurements available in the UE1;

- As part of the UE Context Setup procedure the AMF prepares UE context data of UE1 (e.g., PDU session context, security Key, UE Radio Capability and UE Security Capabilities, etc.) and sends it to gNB2; AMF also checks if the user is within its home operator’s PLMNs and the user has given its consent: if so, the AMF provides gNB2 with user consent information of UE1 as well as Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 by gNB1 and stored in AMF in Step 9.;

- UE context information of UE1, including user consent, Job Type “Immediate MDT and Logged MDT” and “TRSR#1”, are stored in gNB2;

- UE1 is now in connected mode under the coverage of gNB2;

- gNB2 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 12., UE1 can be a candidate. The presence of the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” is an indication for gNB2 that UE1 was previously selected by gNB1 for Continuous MDT, implying that UE1 needs to be re-selected and re-configured by gNB2 to continue performing MDT measurements;

- As part of the UE RRC Reconfiguration towards UE1, gNB2 re-configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM in Step 2. and assigns a UE-specific TRSR: for simplicity, it is assumed that the same “TRSR#1” previously assigned to UE1 by gNB1 is also re-assigned by gNB2 – this is possible if “TRSR#1” was not already assigned to another UE served by gNB2, and how to handle TRSR assignment conflicts is discussed below as part of the measurement continuity during the same RRC state (i.e., at handover); UE1 can re-start Immediate MDT measurements;

- gNB2 retrieves the Logged MDT measurements from UE1;

- gNB2 saves the received Logged MDT measurements of UE1 into another Trace Record file;

- The Trace Record file containing the Logged MDT measurements is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- The TCE correlates the Trace Record files received from gNB1 in Step 7. (with Immediate MDT measurements of UE1) and from gNB2 in Step 19. (with Logged MDT measurements of UE1) as both Trace Record files are “marked with” (and hence identifiable by) the same couple TR/TRSR, i.e., “TR#1 / TRSR#1”.

# 8. Annex 3 – Continuous MDT UE handling in connected mode

How to handle a Continuous MDT UE while in connected mode (and TCE correlation as well) is summarized in Figure 2, where, same as in Figure 1 of Annex 2, changes with respect to legacy management-based MDT to enable the Continuous MDT feature are highlighted in yellow:

Figure 2 - Network-based solution for management-based Continuous MDT: TCE correlation and measurement continuity during the same RRC state (connected mode mobility).

Step 1. to Step 6. are exactly as for Figure 1 in Annex 2.

- UE1 is in connected mode under the coverage of gNB1 (source gNB), hence it performs and reports to gNB1 the Immediate MDT measurements according to the management-based MDT configuration provided by the gNB1. Due to radio conditions UE1 needs to be handed over to gNB2, therefore gNB1 initiates handover to gNB2;

- As per current TS 32.422 clause 4.10, UE1 stops the ongoing Immediate MDT measurements performed while under the coverage of gNB1;

- gNB1 saves the received Immediate MDT measurements into a Trace Record file which is sent to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#1”;

- As part of the Handover Preparation procedure, gNB1 sends to gNB2 the user consent information for UE1 and at least the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” allocated to UE1 by gNB1;

- UE context information of UE1, including user consent, Job Type “Immediate MDT and Logged MDT” and “TRSR#1”, are stored in gNB2;

- The Handover procedure for UE1 from gNB1 to gNB2 takes place, at the completion of which UE1 accessed the target cell in gNB2;

- UE1 is now in connected mode under the coverage of gNB2;

- gNB2 performs user selection among consented users (or users for which consent is not required) and, since it has received user consent information for UE1 in Step 10., UE1 can be a candidate. The presence of the Job Type “Immediate MDT and Logged MDT” and “TRSR#1” is an indication for gNB2 that UE1 was previously selected by gNB1 for Continuous MDT, implying that UE1 needs to be re-selected and re-configured by gNB2 to continue performing MDT measurements;

- As part of the UE RRC Reconfiguration towards UE1, gNB2 re-configures the UE1 to perform MDT measurements based on the management-based MDT configuration previously provided by the OAM in Step 2. and assigns a UE-specific TRSR: it is assumed that the “TRSR#1” previously assigned to UE1 by gNB1 cannot be re-assigned by gNB2 to UE1 because “TRSR#1” was already assigned to another UE served by gNB2, therefore a new TRSR “TRSR#2” (different from “TRSR#1”) is allocated to UE1 by gNB2;

- Since UE1 has been allocated with a new TRSR (“TRSR#2”) it is important to keep the mapping relationship of TRSRs allocated to UE1 for the sake of trace correlation at TCE level – this mapping relationship can be kept by the target gNB (gNB2 in Figure 2);

- UE1 can re-start Immediate MDT measurements and report them to gNB2;

- gNB2 saves the received Immediate MDT measurements into another Trace Record file;

- The Trace Record file is sent by gNB2 to the TCE including the couple TR/TRSR, i.e., “TR#1 / TRSR#2”, as well as the mapping relationship of TRSRs allocated to UE1;

- The TCE correlates the Trace Record file received from gNB1 in Step 9. containing Immediate MDT measurements of UE1 performed under the coverage gNB1 and marked with the couple “TR#1 / TRSR#1” with the Trace Record file from gNB2 in Step 19. containing Immediate MDT measurements of UE1 performed under the coverage gNB2 and marked with the couple “TR#1 / TRSR#2”; this is possible because TCE also received the mapping relationship of TRSRs allocated to UE1 by the gNBs.