3GPP TSG-RAN WG3 Meeting #126	R3-247439

Orlando, USA, 18 - 22 Nov, 2024

Agenda item:	16.3

Source: 	Lenovo

Title: 	Radio resource allocation and UE reader selection for topology 2

Document for:	Discussion and Approval

- Introduction

This paper discusses radio resource allocation and UE reader selection for topology 2.

- Discussion

Radio Resource Allocation

In RAN2#127bis meeting, regarding radio resource allocation, RAN2 made the following agreements:

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

To assist the NW or gNB to control the A-IoT air interface resource, e.g. how many and for how long the resource needs to be allocated, some assistance information needs to be provided. The assistance information could be e.g. what is the device id type that will be indicated in the A-IoT paging message, the approximate device number or Q value, message size that is to be transmitted by A-IoT device(s) etc. The assistance information could be provided to the gNB by the following options, but we are still open for that in this stage.

-	Option 1 (CN request): A-IOT enabled CN node indicates information to A-IOT enabled RAN node, A-IOT enabled RAN node determines time/frequency resource and sends to A-IOT enabled UE. It can happen before A-IOT enabled CN node sending inventory request or can piggyback in the inventory request message, which depends on the solutions of protocol stack.

-	Option 2 (UE request variant 1): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE determines the radio resource request and reports to A-IOT enabled RAN node, A-IOT enabled RAN node determines the final resource and sends to A-IOT enabled UE.

-	Option 3 (UE request variant 2): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE forwards such information to A-IOT enabled RAN node, and A-IOT enabled RAN node determines the resource and sends to A-IOT enabled UE.

Besides above all options, in addition to receive the information from the CN, UE reader may also receive the assistance information from the A-IoT device, e.g., the message size that is to be transmitted. If such information e.g., message size reporting from A-IoT devices is received, UE reader could report such information to the gNB, and gNB determines the final resource and sends it to UE reader.

Proposal 1: Assistance information is needed for A-IOT enabled RAN node to control the A-IoT air interface resource between A-IoT device(s) and UE reader.

Proposal 2: Assistance information could be provided to the A-IOT enabled RAN node by the following options:

-	Option 1 (CN request): A-IOT enabled CN node indicates information to A-IOT enabled RAN node, A-IOT enabled RAN node determines time/frequency resource and sends to A-IOT enabled UE. It can happen before A-IOT enabled CN node sending inventory request or can piggyback in the inventory request message, which depends on the solutions of protocol stack.

-	Option 2 (UE request variant 1): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE determines the radio resource request and reports to A-IOT enabled RAN node, A-IOT enabled RAN node determines the final resource and sends to A-IOT enabled UE.

-	Option 3 (UE request variant 2): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE forwards such information to A-IOT enabled RAN node, and A-IOT enabled RAN node determines the resource and sends to A-IOT enabled UE.

Proposal 3: A-IOT enabled UE may report information received from A-IoT device to A-IOT enabled RAN node to assist A-IOT enabled RAN node to determine the A-IoT air interface resource.

UE reader selection

For topology 2, the UE reader firstly needs to perform the registration procedure.

Another issue related to topology 2 is the UE reader selection, i.e. how to select suitable UE reader for target A-IoT device(s). One solution is that the UE reader is selected based on the registration associated with the area, which is like a “static” solution. For example, the AIOTF performs UE reader selection according to ‘inventory area’ provided by the AF and UE registration information provided by AMF. Another assumption is that AIOTF may be aware of topology of UE readers.  This solution should be defined by SA2.

Other solutions which are more “dynamic” may rely on study on other issues e.g. proximity determination in RAN1 or UE reader mobility including radio conditions in RAN2. For example, the UE reader may be selected according to the proximity determination results. For another example, when UE reader moves out of service area or UE reader suffers to radio link failure, UE reader re-selection may be needed. In this kind of solutions, gNB should be involved since radio conditions of UE reader will be considered.

Proposal 4: How to perform UE reader selection in topology 2 is pending to other WGs.

3. Conclusion

This paper discusses radio resource allocation and UE reader selection for topology 2. We propose:

Proposal 1: Assistance information is needed for A-IOT enabled RAN node to control the A-IoT air interface resource between A-IoT device(s) and UE reader.

Proposal 2: Assistance information could be provided to the A-IOT enabled RAN node by the following options:

-	Option 1 (CN request): A-IOT enabled CN node indicates information to A-IOT enabled RAN node, A-IOT enabled RAN node determines time/frequency resource and sends to A-IOT enabled UE. It can happen before A-IOT enabled CN node sending inventory request or can piggyback in the inventory request message, which depends on the solutions of protocol stack.

-	Option 2 (UE request variant 1): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE determines the radio resource request and reports to A-IOT enabled RAN node, A-IOT enabled RAN node determines the final resource and sends to A-IOT enabled UE.

-	Option 3 (UE request variant 2): A-IOT enabled CN node indicates information to A-IOT enabled UE, A-IOT enabled UE forwards such information to A-IOT enabled RAN node, and A-IOT enabled RAN node determines the resource and sends to A-IOT enabled UE.

Proposal 3: A-IOT enabled UE may report information received from A-IoT device to A-IOT enabled RAN node to assist A-IOT enabled RAN node to determine the A-IoT air interface resource.

Proposal 4: How to perform UE reader selection in topology 2 is pending to other WGs.