3GPP TSG-RAN WG3 Meeting #126	R3-247459

Orlando, FL, USA, November 18 – 22, 2024

Agenda item:	20.2

Source: 	Qualcomm Incorporated

Title: 	AI/ML based positioning accuracy enhancements

Document for:		Discussion and Decision

# 1.	Introduction

At RAN3#125bis, some initial agreements on the support for NG-RAN node assisted positioning with gNB-side model (AI/ML assisted positioning (Case 3a)) and on the support for NG-RAN node assisted positioning with LMF-side model (direct AI/ML positioning (Case 3b)) were made:

Consider aggregated gNB case first, then split architecture. The existing LCS framework is re-used for AI/ML positioning. Case 3a agreements: - The AI/ML model (inference and training functions) is at the gNB. - LMF provides ground truth label and related data (i.e., part B information as called by RAN1) to NG-RAN. FFS on the content of the ground truth label and related data pending on RAN1 progress. FFS on the signalling design. - How the LMF obtains the ground truth label and related data information is outside of RAN3 scope. - NG-RAN obtains measurements from TRP (i.e., part A information as called by RAN1). The details of the measurements are still pending and subject to further discussion in RAN1. - WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model. Case 3b agreements: - Re-using the Measurement procedure defined in NRPPa to support requesting and reporting the related measurements to LMF for Case 3b. The details of the related measurements are still pending and subject to further discussion in RAN1. - NG-RAN gets measurements (i.e., part A information as called by RAN1) from TRP and provides them to LMF. The details of the measurements are still pending and subject to further discussion in RAN1.

In our previous contribution [1] we discussed the status in RAN1/RAN2/SA2 and summarized the RAN3 specification impacts.

In this contribution, we discuss further details on the procedures and signalling design to support network-based AI/ML assisted (Case 3a) and direct (Case 3b) positioning.

# 2.	Network-based AI/ML assisted positioning – Case 3a

## 2.1	Data Collection and Model Training

For Case 3a, the AI/ML model is deployed at the NG-RAN node (gNB/TRP). The model output are "SRS-based measurements", which may be AI/ML enhanced legacy measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements dependent on RAN1 progress. For training data generation, the measurements and related attributes (e.g., timestamp, etc.) are generated by the TRP/NG-RAN node. Therefore, the training data are directly available at the NG-RAN node (gNB/TRP).

Per RAN3#125bis agreement, the 'AI/ML Model Training Function' is located at the gNB. Required for 'Model Training' is the label generation for the training data. The 'ground truth labels' comprise the 'true' SRS based measurements in Case 3a (AI/ML assisted). This requires having knowledge of the actual UE/PRU location estimate available at the gNB when collecting the training data. In addition, the TRP/ARP location information may be needed, e.g., to determine 'true' UL RTOA or gNB Rx-Tx Time differences, etc.

The UE ground truth can be provided by an LMF to the NG-RAN. How an LMF obtains the UE ground truth does not need to be specified. Existing Location Services (LCS) allows an LMF obtaining the location of one or more target UEs using a variety of positioning methods. A desired UE (or PRU) location can be determined with any UE and NW supported method(s) (incl. RAT-dependent and RAT-independent methods, as well as network-based methods).

Observation 1:	For ground-truth label generation, existing LCS [2] and UE positioning [3] procedures can be used to obtain the UE/PRU location estimate by an LMF.

Therefore, for data collection the NG-RAN needs to obtain:

-	UL measurements together with quality information and timestamp (Part A information).
This requires UEs/PRUs transmitting a desired UL SRS.

-	UE ground truth label together with quality information and timestamp (Part B information).
This information can be provided on-demand by an LMF as "assistance data".

The training function at the NG-RAN node may determine that model (re-)training is required, e.g., based on the model monitoring result and initiates the procedures to collect the Part A and Part B information. The trigger for data collection and model training in NG-RAN can be up to implementation since the trigger may also need to consider implementation specific aspects such as current load at a gNB, etc.

Proposal 1:	The NG-RAN node determines when data collection is required to train a gNB-side AI/ML model for positioning.

Proposal 2:	The trigger for data collection and model training in NG-RAN is up to implementation.

For LMF-side models (Case 2b, Case 3b), an LMF may obtain a list of SUPIs for data collection in an area of interest from an AMF [2]. For gNB-side models (Case 3a), the same functionality could be used to provide the list of SUPIs in an area of interest to the NG-RAN when needed. However, since for Case 3a the role of UEs for data collection is only to transmit UL SRS to enable the data collection at the gNB, such a list of available SUPIs is not strictly necessary. I.e., a gNB may also determine an available UE for UL SRS transmission without assistance from an LMF.

A corresponding procedure for data collection for gNB-side model training in Case 3a is illustrated in Figure 1.

Figure 1: General data collection procedure for gNB-side model training in Case 3a.

1.	In the case the target is a UE, the LMF may subscribe to the AMF to retrieve the list of SUPIs located in an area of interest using Namf_EventExposure_Subscriber_Request operation. The AMF then sends a Namf_EventExposure_Subscriber_Response including the list of SUPIs in the area of interest, as defined in [2]. The LMF may also check the user privacy requirements for LCS (at this Step or at Step 5 below).

NOTE 1:	If the target is a PRU, this step may not be needed since the PRUs (and their locations) would usually be known at the LMF e.g., after the PRU registration procedure [2].

NOTE 2:	This Step has been defined by SA2 for Case 2b and 3b [2], which can also be applicable here to Case 3a, if needed. This Step also depends on further SA2 progress.

2.	The LMF may provide the list of available UEs/PRUs in an area of interest to the NG-RAN. This may be proactively pushed to the NG-RAN by an LMF (Option 1) or requested by the NG-RAN node on demand (Option 2). In both Options, this can be a new (non-UE associated) NRPPa message, which includes for each UE in the list an LMF assigned deferred Routing ID, which may be used later at Step 4. To enable an AMF to distinguish a Routing ID assigned by an LMF (used in this procedure) from a Routing ID assigned by the AMF (used as currently specified for NRPPa messages), the two types of Routing IDs could be selected from different ranges. 
If the UE is a stationary PRU, the location of the PRU may be provided in addition.

For Option 2, the request message could be the same procedure as in Step 4 but for a request for the list of UEs/PRUs (e.g., could be a generic NG-RAN node triggered NRPPa message). The request message may also include a desired area where available UEs are required (e.g., a list of Cell-IDs).

3.	The NG-RAN node determines that model (re-)training is required; e.g., based on the model monitoring result. If Step 2 was performed, the NG-RAN node may select a desired UE for model training from the list received at Step 2.

4.	The NG-RAN node may send a request to an LMF for the ground truth label of the UE selected at Step 3. 

If the UE/PRU is selected from the list of available UEs/PRUs received at Step 2 and if the selected UE/PRU is already CM_CONNECTED (RRC_CONNECTED or RRC_INACTIVE), this Step may also be performed after Step 8.

If the UE/PRU is selected from the list of available UEs/PRUs received at Step 2, the NG-RAN node uses the received deferred Routing ID for this UE when sending the message.

Otherwise, if Step 2 was not performed or if the desired UE/PRU was not in the list received at Step 2, the NG-RAN node assigns a Routing ID for this UE. The Routing ID could be a special/reserved value to indicate to the AMF that this is a NG-RAN triggered NRPPa message. A range of Routing ID values could be reserved for NG-RAN (and/or LMF triggered (e.g., like in Step 2)) NRPPa messages (i.e., a range of Routing IDs which cannot be assigned by an AMF would enable an AMF to distinguish a Routing ID assigned by a NG-RAN node or LMF from Routing IDs assigned by an AMF).

NOTE 3: 	If a PRU is selected from the list of available PRUs received at Step 2, and if the PRU is known to be stationary, and if the PRU location has already been provided at Step 2, and if the PRU is already CM_CONNECTED, this step may not be needed.

NOTE 4:	This message may be a generic NG-RAN node triggered NRPPa message, which may be used for example at Step 2 (Option 2), Step 4 to request ground-truth, for other LCM purposes (TBD), or simply for "requesting training", e.g., to bring a UE/PRU to CM_CONNECTED state.

5.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [2] and 38.305 [3] to obtain the UE location.
If the UE/PRU is in CM_IDLE state, the AMF initiates a network triggered Service Request procedure to establish a signalling connection with the UE/PRU, which then also allows the data collection at Step 7/8.

NOTE 5:	The LMF may check the LCS user privacy requirements at this Step or at Step 1.

6.	The LMF provides the UE/PRU location ('ground-truth label') to the serving gNB (via the serving AMF) in response to Step 4.

7.	The serving gNB configures a required SRS for model training in the UE/PRU.

NOTE 6:	The gNB may need to train models for a variety of different SRS configurations. I.e., the model output during inference may be dependent on the used SRS parameter (e.g., bandwidth, number of symbols, etc.).

8.	The NG-RAN performs the UL SRS measurements and derives any data required for model training.

9.	The UE/PRU may be released to idle mode.

From the above description, the following proposals are made:

Proposal 3:	The NG-RAN node selects suitable UEs for SRS transmission to train a gNB-side AI/ML model for positioning.

Proposal 4:	The NG-RAN node determines a required SRS for positioning and uses existing RRC procedures to configure the SRS transmission in the UE.

Proposal 5: 	Define a NG-RAN node triggered NRPPa procedure to allow a gNB to request assistance data from an LMF for gNB-side data collection and model training, where the assistance data include at least the UE ground-truth labels.

## 2.2.	Model Performance Monitoring

According to TR 38.843 [4], Model Monitoring is defined as follows:

Model monitoring: A procedure that monitors the inference performance of the AI/ML model.

Therefore, "model monitoring" is closely related to "model inference".

For model performance monitoring, RAN3 made the following working assumption:

WA: RAN3 will proceed with Option A: NG-RAN node performs monitoring metric calculation for its own model.

Since the AI/ML model inference and training functions are located at the gNB, it seems natural that the gNB also performs the monitoring metric calculation for its own model. The gNB could measure the SRS and derive the model input data and perform the model inference to obtain the "SRS-based measurements" (e.g., AI/ML enhanced legacy measurements (e.g., RTOA, UE Rx-Tx Time Difference Measurements) or any new measurements, dependent on RAN1 progress). To obtain the 'ground-truth label', the gNB would need the 'true' UE location and the 'true' gNB/ARP location (for example, to determine a 'true' RTOA, which depends on the distance between UE and gNB/ARP). Assuming the gNB has its own gNB/ARP location coordinates available, the procedure for gNB-side 'Model Performance Monitoring' could be the same as in Figure 1 with an additional final step to derive the model performance metric based on the model output and UE/gNB/ARP location. The model performance result may trigger the gNB to (re-)train the models as described in Figure 1. Therefore, no additional procedure would be needed for model performance monitoring in Case 3a.

Observation 2:	The model performance monitoring at gNB-side for Case 3a can share the same procedures as for model training.

Proposal 6:	Confirm the WA: For performance monitoring metric calculation for Case 3a in label-based monitoring, the NG-RAN node performs monitoring metric calculation for its own model.

## 2.3.	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [4] defines in section 7.2.4 the following:

- Management: o For UE-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the UE when the monitoring resides within the UE. o For gNB-side model, the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB. o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For gNB-side models (Case 3a), the functionality control can be inside the gNB. The gNB may decide on a particular model based on e.g., requested measurement type, currently used SRS configuration, etc. When requesting measurements from a gNB, the LMF specifies the measurements needed and provides all relevant information to the gNB in the NRPPa Measurement Request [5], incl. e.g., the SRS configuration. Therefore, a gNB should have all information available to perform the functionality management operation of activating/deactivating a particular model or fallback to non-AI/ML based measurements.

Note, a fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements can not be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

Enhancements may be needed to the particular messages (e.g., additional parameter to request AI/ML enhanced measurements, etc.) which, however, depend on further RAN1 progress. In any case, existing procedures can be used for the positioning operation (see also section 2.4 below). Therefore, it is proposed to confirm the study item conclusion:

Proposal 7:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

Observation 3:	A fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements cannot be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

## 2.4	Model Inference Operation

For Case 3a model inference, model input data are internally available at the gNB and the inference operation is running internally on the gNB. The model output for Case 3a are AI/ML enhanced SRS measurements (e.g., existing measurements and/or any additional measurements defined by RAN1) [6]:

Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

NG-RAN node measurements can be reported via the NRPPa Measurement Information Transfer procedures [5].

For example, Figure 2 below shows the typical UL-TDOA positioning procedure from [3]. The LMF may request a desired SRS at Step 2 and requests measurements at Step 6. A gNB may provide AI/ML enhanced measurements, if possible, or may fallback to 'normal' measurements. There may be small specification impacts to specifically request AI/ML enhanced measurements, report AI/ML enhanced measurements, allow/disallow fallback to non-AI/ML measurements by the gNB, etc. However, the overall positioning procedures do not require changes.

Proposal 8:	For Network-based AI/ML assisted positioning (Case 3a), existing positioning procedures for network-based positioning (e.g., UL-TOA/UL-AoA) are used.

Figure 2: Exemplary UL-TDOA positioning procedure

# 3. 	Network-based direct AI/ML positioning – Case 3b

## 3.1	Data collection and Model Training

For Case 3b, the AI/ML model is deployed at the LMF. The model output is the estimated UE location. The NG-RAN/TRP measurements may be the existing UL SRS measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements dependent on RAN1 progress.

For Case 3b (LMF-side models), the LMF performs the 'Data Collection and Model Training' [2]. This also implies that an LMF is collecting the necessary data from the TRPs/NG-RAN to perform the 'Model Training'. The trigger for data collection and model training by an LMF is up to implementation [2].

A general procedure for Case 3b data collection was agreed at SA2#165 in [7], which is repeated in Figure 3 below. The procedure has the following Editor's Note:

Editor's note: The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

Figure 3: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (according to [7]).

At Step 6 in Figure 3, the LMF performs the data collection from NG-RAN. This requires UEs/PRUs transmitting UL SRS for positioning. Existing NRPPa Positioning Information Exchange procedures [5] allow an LMF to request UL SRS transmission from a UE/PRU. Similar, existing NRPPa Measurement Information Transfer procedures [5] allow an LMF to obtain desired UL measurements from TRPs.

At Step 7 in Figure 3, the LMF obtains the UE/PRU 'ground-truth location'. As also discussed in [1], existing Location Services (LCS) allows an LMF obtaining the location of one or more target UEs using a variety of positioning methods. A desired UE (or PRU) location can be determined with any UE and NW supported method(s) (incl. RAT-dependent and RAT-independent methods, as well as network-based methods).

Based on the discussion above, an extension of the procedure in Figure 3 is shown in Figure 4.

Figure 4: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements (extension of Figure 3).

1.-5.	As defined by SA2 in [7].

6.	The LMF performs the NRPPa Positioning Information Exchange procedure [4] to request the serving NG-RAN node to configure a required SRS in the target (UE or PRU).

NOTE 1:	Additional parameter values for SRS configuration (if any) depend on RAN1.

7.	The LMF sends a NRPPa Measurement Request [5] to the serving and possibly neighbour NG-RAN nodes (TRPs) to request the SRS-based measurements for model training. This may be a request for any of the existing measurement types and/or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.).

8.	The TRPs provide the measurement results for model training to the LMF in a NRPPa Measurement Response message.

9.	The LMF may perform any of the UE positioning procedures defined in TS 23.273 [2] and 38.305 [3] to obtain the UE/PRU location.

NOTE 2:	The positioning method used for obtaining the UE/PRU location should not be based on the same SRS as used for steps 6-8, since the UE/PRU location should be independent on the actual data collected for model training. E.g., the UE/PRU location may be determined via GNSS, DL-TDOA, etc.

NOTE 3:	In case the target is a PRU, the LMF may already have the PRU location available (e.g., from the PRU registration [2]) and step 9 may not be needed.

After step 9, the LMF would have all the data available for 'Model Training', i.e., the measurement data (SRS-based measurements; Part A) together with the ground truth label (UE location; Part B).

Proposal 9:		For Case 3b ground-truth label generation for model training, existing LCS [2] and UE positioning [3] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 10:		Agree/confirm the general procedure in Figure 4 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

For LMF-side model training (Case 3b) the existing NRPPa Positioning Information Request with the Requested SRS Transmission Characteristics (Step 6a in Figure 4) can be used to request a desired SRS for positioning configuration for the UE/PRU. A LMF may need to train models for a variety of different SRS configurations. I.e., the model output during inference may be dependent on the used SRS parameter (e.g., bandwidth, number of symbols, etc.). For the inference phase, existing positioning procedures can be used (see also section 3.4 below). In these procedures, the gNB may provide a particular SRS configuration to the LMF (decided by the gNB) and the LMF would need to search for an applicable model in its model repository suitable to this particular SRS configuration. Therefore, when using the Requested SRS Transmission Characteristics for model training, a LMF may need to specify in more detail a required (and not desired) SRS configuration, such that the LMF can train models for a variety of SRS configurations. The NRPPa Validity Area Specific SRS Information [5] already allows an LMF to specify a more detailed requested SRS characteristics, but currently only when area-specific SRS is requested. This may need to be extended to the 'Model Training' use case, and the 'desired SRS configuration' may need to be defined as a 'required SRS configuration'.

Proposal 11:	For LMF-side model training, the NRPPa Requested SRS Transmission Characteristics need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

## 3.2	Model Performance Monitoring

For LMF-side models (Case 3b) the LMF is performing the 'Model Performance Monitoring' [2]. Therefore, the same procedure as for data collection could be used for performance monitoring. The procedure in Figure 4 would then have an additional 10th step to determine the desired model performance metric using the calculated UE position with the measurements from Step 8 in Figure 4 and comparing the result with the 'ground-truth label' from Step 9 in Figure 4. As discussed in section 2 above, a LMF may have several trained models corresponding to several different SRS configurations available. Therefore, also for Model Performance Monitoring, a LMF may need to be enabled to specify a more detailed required SRS Configuration in Step 6a of Figure 4 (see Proposal 11).

Observation 4:	The model performance monitoring at LMF-side can share the same basic procedures as for model training.

## 3.3	Functionality Management/Control

For functionality/model selection, activation, deactivation, switching, and fallback operation, which may be generally referred to as "functionality management" or "functionality control", TR 38.843 [4] defines in section 7.2.4 the following:

- Management: o For UE-side model, […] o For gNB-side model, […] o The model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) may be performed by the LMF when the monitoring resides within the LMF or UE.

For LMF-side model (Case 3b), the LMF can request measurements from the gNBs as usual (see also section 3.4 below). The requested measurements may be any of the existing measurement types (RTOA, AOA, gNB Rx-Tx Time Difference, etc.) or any new measurements defined by RAN1. The LMF performs the inference operation based on available models. All information required to perform model selection, activation, switching, fallback, etc. should be available at the LMF. Per the study item conclusion shown above, the model/functionality control may be performed by the LMF when the monitoring resides within the LMF, which is the case per SA2 agreements [2]. Therefore, it is proposed to confirm the study item conclusion:

Proposal 12:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

Note, a fallback to non-AI/ML based positioning does not necessarily imply a change of selected positioning method. E.g., if the LMF requested measurements are the existing UL SRS measurements (e.g., RTOA), a LMF could fallback to conventional position calculation (e.g., if no trained model is available for the used UL SRS or network conditions). However, the selected positioning method (e.g., UL-TDOA) does not need to change.

## 3.4	Model inference operation

For Case 3b, the NG-RAN nodes provide UL SRS measurements, which may be the existing UL SRS measurements (e.g., RTOA, gNB Rx-Tx Time Difference, etc.) or new measurements dependent on RAN1 progress, and the model inference is running at the LMF. To configure and obtain the desired measurements, existing positioning procedures can be used.

For example, Figure 2 in section 2.4 above shows the typical UL-TDOA positioning procedure from [3]. The LMF may request a required SRS at Step 2 and requests measurements at Step 6. A gNB may provide the requested measurements at Step 8. The LMF may run the model inference operation after Step 8.

There may be small specification impacts to support new UL SRS measurements (if any) or to request a specific UL SRS to be used for the measurements. However, the positioning procedures do not require changes and existing NRPPa/F1AP procedures can continue to be used for AI/ML positioning.

Proposal 13:	For Network-based direct AI/ML positioning (Case 3b), existing positioning procedures for network-based positioning (e.g., UL-TOA/UL-AoA) are used.

# 4.	Summary

In this contribution, we discussed the Network-based AI/ML assisted positioning (Case 3a) and Network-based direct AI/ML positioning (Case 3b). From the discussion we observe that these cases can largely be supported with existing positioning/NRPPa procedures. Enhancements (e.g., new measurement parameter, etc.) may be needed based on further RAN1 progress as usual (i.e., implementation of RAN1 parameter list).

The main RAN3 impact we identified is the need for a new NG-RAN node triggered NRPPa procedure towards an LMF to support the various functions related to model training and performance monitoring for gNB-side models (Case 3a).

The following Observations and Proposals were made.

Case 3a:

Observation 1:	For ground-truth label generation, existing LCS [2] and UE positioning [3] procedures can be used to obtain the UE/PRU location estimate by an LMF.

Proposal 1:	The NG-RAN node determines when data collection is required to train a gNB-side AI/ML model for positioning.

Proposal 2:	The trigger for data collection and model training in NG-RAN is up to implementation.

Proposal 3:	The NG-RAN node selects suitable UEs for SRS transmission to train a gNB-side AI/ML model for positioning.

Proposal 4:	The NG-RAN node determines a required SRS for positioning and uses existing RRC procedures to configure the SRS transmission in the UE.

Proposal 5: 	Define a NG-RAN node triggered NRPPa procedure to allow a gNB to request assistance data from an LMF for gNB-side data collection and model training, where the assistance data include at least the UE ground-truth labels.

Observation 2:	The model performance monitoring at gNB-side for Case 3a can share the same procedures as for model training.

Proposal 6:	Confirm the WA: For performance monitoring metric calculation for Case 3a in label-based monitoring, the NG-RAN node performs monitoring metric calculation for its own model.

Proposal 7:	For gNB-side model (Case 3a), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the gNB.

Observation 3:	A fallback to non-AI/ML based measurements does not necessarily imply a change of selected positioning method. E.g., if AI/ML enhanced RTOA measurements cannot be obtained, a TRP could report the 'normal' RTOA measurements and UL-TDOA positioning can still be possible.

Proposal 8:	For Network-based AI/ML assisted positioning (Case 3a), existing positioning procedures for network-based positioning (e.g., UL-TOA/UL-AoA) are used.

Case 3b:

Proposal 9:		For Case 3b ground-truth label generation for model training, existing LCS [2] and UE positioning [5] procedures are used to obtain the UE/PRU location estimate by an LMF.

Proposal 10:		Agree/confirm the general procedure in Figure 4 for data collection at LMF for 'Model Training' in Case 3b and inform SA2 about the RAN3 agreement.

Proposal 11:	For LMF-side model training, the NRPPa Requested SRS Transmission Characteristics need some enhancements to allow an LMF to request a more detailed SRS configuration required (similar to the Validity Area Specific SRS Information).

Observation 4:	The model performance monitoring at LMF-side can share the same basic procedures as for model training.

Proposal 12:	For LMF-side model (Case 3b), the model/functionality control (e.g., selection, (de)activation, switching, fallback, etc.) is performed by the LMF.

Proposal 13:	For Network-based direct AI/ML positioning (Case 3b), existing positioning procedures for network-based positioning (e.g., UL-TOA/UL-AoA) are used.

# References

[1]	R3-245461, "AI/ML based positioning accuracy enhancements", Qualcomm Incorporated.

[2]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[3]	3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[4]	3GPP TR 38.843: "Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface".

[5]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[6]	RP-240499, "Status Report to TSG", RAN#103.

[7]	S2-2411257, CR 0597r4 to 23.273: "KI#1 - Addressing Editor´s Note: Procedure for Data Collection for AI/ML based positioning from NG-RAN", SA2#165.