3GPP TSG-RAN WG3 Meeting #126	R3-247524

# Orlando, U.S.A, 18th – 22nd November, 2024

Title: 	Split architecture support for Release 18 use cases

Source: 	Ericsson, AT&T, Verizon

Agenda item:	11.4

Document Type:	Discussion and decision

# 1. Introduction

In this contribution, we discuss the details of split architecture support for Release 18 use-cases based on the conclusions reached during the SI [1], the new WID [2], and agreements reached in the last RAN3 meeting.

# 2. Discussion

The discussion and proposals in this paper are organized in terms of the different use-cases to which split architecture leftovers are applicable, followed by the required signalling enhancements.

## 2.1 Measured UE performance

#### 2.1.2 Delay

The following was agreed as part of the meeting outcomes from RAN3 #125:

The details of signaling measured UE performance metrics from DU to CU-CP and/or from CU-UP To CU-CP needs further discussion during normative phase.

The above overrides the agreement “Transferring Measured UE performance from CU-UP to CU-CP.” from RAN3#124.

During discussions in Rel-19 study item phase, different proposals for collecting UE performance feedback metrics, e.g., only from the gNB-CU-UP, from both the gNB-CU-UP and the gNB-DU, and other variants were discussed. RAN3 agreed that, for the metrics to be collected in accordance with the general principles of AI/ML data collection, i.e., where the requesting node can configure a reporting periodicity, where the responding node may accept/reject a request, etc., a part of the metrics would have to be collected from the gNB-CU-UP and another from the gNB-DU.

In the context of downlink UE delay measurements, this corresponds to exchange of the following metrics:

- D1: Average delay in DL over air-interface (available at gNB-DU)

- D2: Average delay in DL in gNB-DU (available at gNB-DU)

- D3: Average DL delay over F1-U (available at gNB-CU-UP)

- D4: Average DL delay in CU-UP (available at gNB-CU-UP)

In the uplink direction, the delay components consist of the following:

- D1: UL PDCP packet average delay (available at gNB-CU-CP, no spec impact)

- D2.1: Average over-the-air interface UL packet delay (available at gNB-DU)

- D2.2: Average UL RLC packet delay (available at gNB-DU)

- D2.3: Average UL delay over F1-U (available at gNB-CU-UP)

- D2.4: Average UL PDCP re-ordering delay (available at gNB-CU-UP)

Proposal 1: RAN3 to agree to the exchange of RAN delay measurements for UL and DL from the gNB-CU-UP and gNB-DU to the gNB-CU-CP.

In the context of collecting UE performance at the gNB-CU-CP, which is then sent to a neighbour gNB, the granular components of the delay measurements, e.g., delay over the air interface, delay in the gNB-DU, and others listed above are not individually relevant for the gNB-CU-CP or the UE performance measurements at the neighbour node. It is only the total end-to-end delay in the UL or DL per UE that is of interest in this context. It may therefore be efficient to signal a compound measurement that is generated by summing up the cumulative measurements from the gNB-CU-UP or the gNB-DU to the gNB-CU-CP. However, the delay measurements defined in TS 28.558 Sec. 6.3.1.1, and the corresponding MDT measurements defined in TS 37.320 are defined per QoS level, or per S-NSSAI, or at a granularity of per-DRB- per UE. UE performance, as the name suggests, is at the granularity of the UE. RAN3 in meeting #124 agreed that the UE performance per UE is derived by average over the per-DRB measurements of different metrics corresponding to the UE.

UE level performance is derived as the average value of per DRB level measurements of the UE.

With the above approach however, there exists different methods to average the different DRB level measurements corresponding to a metric. For example, individually generating an average delay in the gNB-CU-UP or the gNB-DU for each delay sub-component, or signalling the raw measurements to the gNB-CU-CP to average and generate the per-UE-level UE performance metric. Other options include averaging the individual delay metrics measured per-DRB across all the DRB sessions and generating an arithmetic average, or by summing all the numerator values, e.g., measuring the delay components received in a time-period at the CU-UP, CU-CP, or DU, before generating a per-UE performance metric for UL/DL delay by dividing by the number of samples or time duration. All the above listed values may represent a certain average value. However, when it comes to exchange of UE performance across gNBs, a single method must be defined and followed according to agreed principles such that the collected data and its interpretation is correctly understood. Based on the above discussion, we recommend that:

Proposal 2: RAN3 to send an LS to SA5 asking for feedback on how such per-UE measurement reports are performed/collected with a per-UE granularity. If such a functionality is not available, then ask SA5 how  such measurements are configured/collected and whether a per UE granularity can be introduced for such measurements.

In addition to ensuring that the averages are computed in a well-defined manner, it is also vital that the different component measurements measured at the UE (i.e. UL PDCP packet average delay already available at the gNB-CU), gNB-DU and the gNB-CU-UP follow the principles below:

- are measured over the same time-period, and

- are aligned with one another when a cumulative UL/DL performance measurement is generated at the gNB-CU-CP, i.e., the samples from the gNB-CU-UP and the gNB-DU are available at the gNB-CU-CP when a cumulative measurement is calculated.

For the alignment of measurements, the gNB-CU-CP, which configures the reporting of the metrics from the UE, gNB-DU and the gNB-CU-UP, must ensure that the measurements are configured with the same reporting periodicity. For measurements collected at the UE it might suffice that they are configured with a periodicity that is equal or smaller than the reporting periodicity for gNBDU and gNB-CU-UP measurements. Whilst to ensure that the measurements reported are measured within the same time-period, it is essential that a reported measurement shall be taken within the previous time period and not in time periods before or after it. With these conditions, the gNB-CU-CP would be able to collate delay measurements coming from the UE, gNB-DU and from the gNB-CU-UP and received within the same time period, thereby forming a total DL value representing the delay experienced across the configured time period.

Proposal 3: The gNB-CU-CP ensures that the measurements collected from gNB-DU and gNB-CU-UP for UE performance are configured with the same measurement and reporting periodicity. The gNB-CU-CP ensures that the measurements collected from the UE are configured with periodicity equal or smaller than the periodicity for measurements from the gNB-DU and gNB-CU-UP. The gNB-CU-CP receives the delay measurements (from the UE, gNB-CU-UP, and the gNB-DU) for the same reporting period and computes the overall UL/DL delay metric to report as UE performance information.

#### 2.1.3 Packet loss

In Rel18 it was agreed to include Packet Loss as a parameter for UE performance feedback. It is important to note that UE performance feedback is collected at a target node to then provide to the source RAN information about the service level performance with which a UE is served after a handover. This information may then be used to evaluate the performance of mobility actions derived from AI/ML-based inference, in terms of how well services are served at the target node. However, we observe and describe in detail in the subsequent paragraphs that the Packet Loss parameter introduced in Rel18 may not represent the information originally desired and that it is challenging to collect it in a way that is meaningful and such that it covers all cases.

TS 28.552 defines measurement of DL Drop rate in gNB-CU-UP as the fraction of DL user plane packets that are dropped in relation to the total number of packets that entered the queue in the corresponding DRB over a period of time. The above measurement may be performed either at the PDCP level or at the RLC level to derive the packet loss rate at different levels in the protocol stack. In the context of UE performance reporting for packet losses, a value derived at a higher protocol layer, i.e., the measurements on the PDCP level packet loss rate, is considered more relevant as it better reflects the performance at service-level. Indeed, the packet loss at e.g. MAC level does not necessarily carry information about the quality of the service because a dropped MAC PDU may end up being successfully retransmitted with no negative effects on the service. Besides, a dropped MAC PDU may be due to aggressive modulation and coding scheme, which is an implementation/operational choice and not a sign of good or bad performance.

It is also understood that, for a DRB delivered over RLC Unacknowledged mode, computing a PDCP packet loss metric as defined above is not possible. This is due to the fact that the PDCP entity cannot keep track of packets that were acknowledged in RLC UM but can only keep track of packets submitted to lower layers for transmission. Therefore, PDCP does not know whether a RLC UM packet has been successfully delivered or not. On the contrary, it is possible to compute packet ross rate under RLC unacknowledged mode at lower layers, i.e., at the RLC entity in the gNB-DU. However, as explained above, the service level performance is not necessarily reflected by packet loss at RLC layer.

In addition to the above, packet loss rate at RLC layer is not available for uplink traffic. To make this measurement available for UL traffic, a change at the UE would be needed, implying high impact on UEs and standard specifications and also implying that the measurement would not be available for legacy UEs.

In summary:

- UL Packet Loss Rate is NOT available.

- DL packet loss rate at RLC indicates loss of packets at lower layers, which may not at all reflect in application packet losses, hence this metric is not useful to drive the performance with which the UE bearers in use are served.  Note that DL packet losses might be purposely accounted for in implementations that adopt aggressive modulation and coding, hence such packet losses are not necessarily an indication of low performance.

- DL packet loss at PDCP cannot monitor for loses in RLC Unacknowledged Mode.

The table below summarizes the drawbacks of Packet Loss rate collection at different protocol levels:

|  | Advantages | Disadvantages |
| --- | --- | --- |
| Packet Loss Rate Calculation at PDCP | It represents service level performance as it reflects IP packet drop rate | It cannot be calculated for RLC UM |
| Packet Loss Rate Calculation at RLC | It could be derived for RLC UM (from H-ARQ retransmissions result) | It is not available for UL unless new UL packet loss measurement reporting is introduced (this has UE impacts). It does not necessarily map to Application packet losses and it might simply be the result of aggressive modulation and coding. |

As it can be seen from the above, the existing Packet Loss measurements are all affected by rather major drawbacks.

Observation 1: Computing packet loss rate may not always be possible, and it may not provide full information on service level performance.

Furthermore, there are some important distinctions to note when discussing the relationship between packet loss measurement at a lower layer and the service or application-level performance.

Firstly, compared to other performance metrics, such as throughput and delay, the packet loss rate measured in the RAN is not necessarily reflected by the packet loss rate experienced at a higher layer. For example, at RLC level PDUs can be dropped but they are retransmitted at TCP level, hence becoming available at application level.

Secondly, delay and throughput are metrics directly linked to service performance at the higher layers, i.e., the packet delay and throughput at application layer are directly connected with the measured delay and throughput. The same is not valid for packet drops at application layer. Based on the above, it may be argued that while delay and throughput are absolute measures of service level performance, packet loss is not.

Furthermore, it is also the case that compared to throughput and delay, packet loss is a much more volatile metric. 
In most situations, the choice of a modulation scheme is made in relation to a target BLER, and as a consequence a certain base packet loss rate is the consequence of a design choice of the BLER target. BLER targets are implementation specific and may change in relation to, e.g. the current SNR, the overall amount of resources available, the scheduling policy of a RAN node. Therefore, changes in the packet loss rate may not give information of whether the target cell is a good mobility target or not. The mobility target cell may be a perfectly good cell, but due to implementation choices the target RAN node may decide to e.g. apply an aggressive modulation and coding scheme to the UE and end up in higher packet loss rates, while the performance at the application layer is still good.

Hence the question is whether the impact to the standard of providing this partial and somewhat misleading parameter is worthy of the advantages that it may bring, given that the Packet Loss metric does not cover all use cases and it may not represent performance at service level. On the above basis, we propose the following:

Proposal 4: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Additionally, we reflect on the fact that release 18 discussions on packet loss and the inclusion of this IE in the UE performance feedback was largely derived from the QoS settings for packet loss rate. The specified range for the packet loss rate is an integer between 0 to 1000 with steps of 0.1%. In realistic scenarios, as specified for NR QoS parameters, the expected packet loss/error rate that the network is expected to offer to services is at most on the order of 10-2. Therefore, the Average Packet Loss DL IE, in its current standardised form, would not even be able to represent the measured packet loss rate in the most typical case (as the value range of the IE is too coarse). In other words, the Average Packet Loss DL IE introduced in TS38.423 is of limited use and ignoring this IE would bring no impact to the overall functionality.

On the basis of this, we conclude that fixing the shortfalls to enable measurements of the packet loss rate in all use cases implies big impacts to the standard and in the end, it does not deliver information that are crucial to understand whether a mobility target is good or bad. We therefore propose the following:

Proposal 5: RAN3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used, and it is ignored.

Proposal 6: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

We understand that there might be companies that still see Packet Drop Rate as a meaningful metric to consider. In that case, we would propose as a very minimum, that an LS is sent to SA5 and RAN2 tasking them to define an UL Packet Drop Rate, if feasible, given that such metric is the most critical when it comes to packet losses over the air.

Proposal 7: If RAN3 decides to maintain the Packet Drop Rate as a performance metric, send an LS to SA and RAN2 requesting for the definition of an UL Packet Drop Rate metric, if feasible

#### 2.1.4 Throughput

As discussed in the previous section, and in Ericsson’s contribution to R3#125 (R3-244417) any performance measure computed at a higher layer is generally considered closer to the application-level or service-level perceived performance. TS 28.552 defines throughput measurements, however, the Average DL and UL UE throughput in gNB is defined as the ratio of successfully delivered RLC SDU data volume over time. This measure is available at the gNB-DU.

An alternative to the above is to obtain the UE throughput measurements from the gNB-CU-UP, which is the termination point for PDCP. While there are no measurements defined for PDCP throughput measurements at the gNB-CU-UP, the specifications imply that such functionality is available at the gNB-CU-UP as part of the traffic policing functionality. This functionality is likely up to the implementation. However, if such a metric were to be used in inter-node signalling, this would require a more rigorous definition of the measurement. It is likely preferred to avoid such discussions as another defined metric is available at the gNB-DU already.

Based on the above, we make the following proposal:

Proposal 8: RAN3 to agree using downlink and uplink throughput as measured by the gNB-DU (defined in Sec. 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558) as the downlink and uplink throughput for UE performance feedback in split architecture.

## 2.2 Energy Saving

During RAN3 #121, the below agreement was reached, although further discussions in the context of split architecture were suspended to future releases, and the same is in scope in Release 19.

Work on the measured EC transmission from gNB-DU to gNB-CU over F1 in R18. Whether reusing the current F1AP procedures or defining new procedures needs to be further discussed.

The following in RAN3 #124 confirms the earlier agreement:

Measured Energy Cost can be transferred from DU to CU.

In RAN3 #125-bis, the following agreement was captured on transmission of Energy Cost from the DU to the CU

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

In addition to signalling the above, agreements in Rel-18 and Rel-19 do not mention exchange of EC from the gNB-CU-UP to the gNB-CU-CP or if it suffices to report EC from only the gNB-DU for use as node-level EC. Herein, we describe our views and argumentation in this matter.

It is common understanding that a significant portion of the energy consumed at a gNB in split architecture is expected to be at the different gNB-DUs, as it is the logical entity that hosts the baseband and radios. The energy consumed on a gNB-CU-UP or gNB-CU-CP, on the other hand, is expected to be negligible if compared to that of the gNB-DU. Another complication worth considering in the generation of an EC associated to the gNB-CU-UP or gNB-CU-CP is that these nodes may be realized on a virtual/cloud environment hosting multiple instances of gNB-CU-Ups and gNB-CU-CPs. In such cases, accurately determining the EC associated with a single instance of a gNB-CU-UP/gNB-CU-CP function may not be feasible.

Based on the above, we have the following proposal:

Proposal 9: There is no need to consider the EC of the gNB-CU-UP and gNB-CU-CP.

## 2.3 Signalling aspects

In light of what is discussed in this paper it is straight forward that some UE performance metrics in the split architecture case shall be signaled from the gNB-CU-UP to the gNB-CU-CP and some other metrics shall be signaled from the gNB-DU to gNB-CU-CP.

In this section, we tackle the signalling aspects of the aforementioned metrics.

#### 2.3.1 UE performance

In RAN3 #125-bis, the following agreement was captured on transmission of Energy Cost from the DU to the CU

Introducing Data Collection Reporting Initiation and Data Collection Reporting procedures over F1 interface to transfer measured EC from DU to CU.

Below we discuss the need for a similar procedure between the gNB-CU-UP and the gNB-CU-CP to exchange the other UE performance metrics.

Currently over the E1 interface, there already exists Resource Status Reporting procedure which is used by a gNB-CU to request from the gNB-CU-UP reporting of measurements which may be used for various purpose, e.g. as AI/ML inference inputs or feedback information. From an operational perspective, the gNB-CU-CP uses the information received over the Resource Status Reporting procedure for actions such as load balancing, resource planning, etc., and extending this procedure with UE performance metrics seems a viable option that does not violate the principles of the Resource Status Reporting procedure.

Another alternative, that is comparatively more disruptive, is to introduce Data Collection Reporting Initiation and Data Collection Reporting procedures over E1 interface to transfer UE performance metrics from the CU-UP to the CU-UP. This approach would mean that the same procedural design is preserved over the XN, F1, and E1 interfaces.

Proposal10: RAN3 to discuss and agree on procedure to signal UE performance metrics over E1 interface.

# 3. Conclusion

Based on the discussion in this paper, we make the following observation and proposals.

Proposal 2: RAN3 to send an LS to SA5 asking for feedback on how such per-UE measurement reports are performed/collected with a per-UE granularity. If such a functionality is not available, then ask SA5 how  such measurements are configured/collected and whether a per UE granularity can be introduced for such measurements.

Proposal 3: The gNB-CU-CP ensures that the measurements collected from gNB-DU and gNB-CU-UP for UE performance are configured with the same measurement and reporting periodicity. The gNB-CU-CP ensures that the measurements collected from the UE are configured with periodicity equal or smaller than the periodicity for measurements from the gNB-DU and gNB-CU-UP. The gNB-CU-CP receives the delay measurements (from the UE, gNB-CU-UP, and the gNB-DU) for the same reporting period and computes the overall UL/DL delay metric to report as UE performance information.Proposal 3: The gNB-CU ensures that the measurements for UE performance are configured with the same measurement and reporting periodicity across the different measuring entities. The gNB-CU-CP receives the delay measurements (from the UE, gNB-CU-UP, and the gNB-DU) for the same reporting period and computes the overall UL/DL delay metric to report as UE performance information based on the LS response from SA5 (proposal 2).

Proposal 4: RAN3 to agree that packet loss is not a suitable candidate metric for AI/ML UE performance feedback in split architecture use cases.

Proposal 5: RAN3 to agree on a correction to release 18 UE performance feedback, where the Average Packet Loss DL IE is not used, and it is ignored.

Proposal 6: RAN3 to not continue further discussion on Average Packet Loss for UE performance in the context of split architecture.

Proposal 7: If RAN3 decides to maintain the Packet Drop Rate as a performance metric, send an LS to SA and RAN2 requesting for the definition of an UL Packet Drop Rate metric, if feasible

Proposal 8: RAN3 to agree using downlink and uplink throughput as measured by the gNB-DU (defined in Sec. 6.3.1.4.1 and 6.3.1.4.2 in TS 28.558) as the downlink and uplink throughput for UE performance feedback in split architecture.

Proposal 9: There is no need to consider the EC of the gNB-CU-UP and gNB-CU-CP.

Proposal10: RAN3 to discuss and agree on procedure to signal UE performance metrics over E1 interface.

TPs to TS38.473, TS37.483 and TS38.423, mirroring the proposals above, are presented in R3-247525, R3-247526 and R3-247527

# 4. References

- R3-244810 (TP to 38.473) Conclusion on AI/ML NG-RAN Enhancements

- RP-242385 New WID on Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN