3GPP TSG-RAN WG3 Meeting #126	R3-247586

Orlando, Florida, U.S., 18th – 22nd November 2024

Agenda Item:	16.2

Source:	Ericsson

Title:	[TP for TR 38.769] on AIoT Architecture aspects

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-247585.

# 2	Text Proposal for TR 38.769 v1.1.0 based on R3-247036/R3-245849

------------Start of the First Change-------------

## 6.4	RAN architecture aspects

This clause attempts to identify and describe architectural elements necessary to define a RAN architecture for support of Ambient IoT embedded in the overall 5G system architecture in support of topology 1 and topology 2 (as defined in TR 38.848 [2]).

This chapter also attempts to identify a functional split between RAN and CN.

The logical system architecture for A-IoT consists of the following architectural elements:

A-IoT device:	Equipment with characteristics outlined e.g., in TS 22.369 [10] and TR 38.848 [2].

A-IoT RAN:	Hosts certain functions for A-IoT as part of the functional split between RAN and CN.

Editor’s Note 4: Further details regarding A-IoT functions hosted in the A-IoT RAN and the respective functional split to be decided by RAN2, RAN3 and SA2.

A-IoT radio:	Radio interface between A-IoT device and A-IoT RAN node in topology 1 and between A-IoT device and A-IoT-enabled UE in topology 2.

Editor’s Note 5: Further details on A-IoT radio to be discussed by RAN1 and RAN2.

A-IoT CN:	Hosts certain functions for A-IoT as of the functional split between RAN and CN.

NOTE: the details of A-IoT CN are subject to SA2.

Editor’s Note 6: Further details regarding A-IoT functions hosted in the A-IoT CN and the respective functional split to be decideed by RAN2, RAN3 and SA2.

XX interface:	Interface between the A-IoT RAN and the A-IoT CN on which certain A-IoT specific functions are performed.

Editor’s Note 7: The functions represented by the XX interfaces are FFS. It is also FFS whether this interface represents a new logical interface or is equal to NG. E.g., for topology 1 it may only represent a single interface instance, e.g., a new interface between A-IoT RAN and A-IoT CN, for topology 2 it might represent either 2 interface instances, one instance for NG and one instance “XX” for a new interface between A-IoT CN and A-IoT RAN, or one instance for NG alone.

Common reader function:	A function that communicates with the A-IoT device by means of A-IoT radio.

Editor’s Note 8: Further details on Common reader function is to be discussed by RAN1 and RAN2.

A-IoT RAN node function:	A function that contains e.g., the control of the A-IoT radio resources used towards the A-IoT device.

Editor’s Note 9: further details are FFS. Note that “control of A-IoT radio resources” does not necessarily imply dynamic configuration of resources but could also rely on static assignment of resources by means of OAM. Aspects concerning coordination of the Upper Layer functions (e.g., Inventory, Command) e.g., in case these functions have to be performed over a multitude of instances of the Common Reader Function are FFS.

AIoT upper layer information:	This term denotes communication means to transport AIoT transaction information to the AIoT device via the readers; for T2 it also contains control information for the AIoT transaction destined for the AIoT enabled UE.

Editor’s Note 10: further details are FFS.

AIoT sessioin resource request:	This term denotes denotes communication means to trigger allocation/control of AIoT radio resources by the AIoT RAN.

Editor’s Note 11: further details are FFS.

AIoT radio resource control:	This term denotes denotes communication means to trigger allocation/control of AIoT radio resources by the AIoT RAN.

Editor’s Note 12: further details are FFS.

### 6.4.1	Support of Topology 1

Figure 6.4.1-1 depicts a logical system architecture for topology 1, where the Common reader function and A-IoT RAN node function are deployed within an A-IoT RAN.

Figure 6.4.1-1 Logical system architecture for topology 1

For Toplogy 1, architecture and protocol aspects of split RAN architecture are not studied.

In Topology 1, the XX interface could be based on NG or a new interface carried over NG or a new interface.

Figure 6.4.1-2 shows the Protocol stack for Topology 1, assuming a SCTP-based transport:

Figure 6.4.1-2. Protocol Stack for Topology 1

NOTE:	The protocol stack in Figure 6.4.1-2 does not illustrate how A-IoT upper layer information, if any, is transported over XXAP. Details are subject to SA2 agreements.

For topology 1, the XXAP is terminated at an A-IoT RAN node.

The signalling transport for XXAP at the A-IoT RAN node is SCTP/IP. Other options of signalling transport for XXAP at the A-IoT RAN node (e.g., HTTP/2/TLS/TCP) were discussed, but will not be pursued.

Editor’s Note 3: The A-IoT CN may include AMF and A-IoT related functions which is up to SA2 decision.

In Topology 1, an A-IoT RAN node may serve one or more readers.

The A-IoT RAN node should enable the coordination of the usage of the A-IoT radio resources among readers.

Reader selection may need coordination between A-IoT RAN node and A-IoT CN.

### 6.4.2	Support of Topology 2

Figure 6.4.2-1 depicts a logical system architecture for topology 2, where the Common reader function is located at an A-IoT-enabled UE, and the A-IoT RAN node function is located at an A-IoT-enabled gNB.

The following definitions apply:

A-IoT-enabled gNB:	A gNB supporting A-IoT RAN node function in topology 2, which is able to communicate with the A-IoT-enabled UE via NR Uu interface.

A-IoT-enabled UE:	A UE supporting Common reader function, which is able to communicate with the A-IoTdevice via the A-IoT radio interface.

Figure 6.4.2-1 Logical system architecture for topology 2

Editor’s Note 1:	Figure 6.4.2-1 doesn’t illustrate the protocol between A-IoT enabled UE and A-IoT CN, if needed, the figure needs to be revised in case such is defined by SA2.

Editor’s Note 2:	In Topology 2, the XX interface could be based on NG or a new interface carried over NG or a new interface. XX signaling could be transported via XX-C or XX-U, which is FFS.

Editor’s Note 3:	The A-IoT CN could include AMF and A-IoT related functions, which is up to SA2 decision.

Editor’s Note 4:	The A-IoT enabled gNB performs radio resource management for A-IoT related radio resources, details are pending on RAN1 and RAN2 mechanisms.

While the AIoT CN may provide AIoT-enabled UE-level assistance information or geographically connoted (AIoT) service area information, the AIoT enabled gNB is ultimately responsible for determining the appropriate set of reader functions to perform the requested AIoT service.

Editor’s Note 5:	Further details on reader selection, for Topology 1 and Topology 2, needs coordination with SA2.

In Topology 2, the RAN architecture should enable the coordination of the usage of the A-IoT radio resources among readers.

An A-IoT-enabled gNB could support both topology 1 and topology 2, this is an implementation matter and supported by appropriate architectural and protocol design.

#### 6.4.2.1	Solutions for Topology 2

##### 6.4.2.1.0	General

To support Topology 2, the following solutions are to be studied for conveying A-IoT upper layer information:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

NOTE:	The protocol stack for each solution option does not necessarily illustrate all A-IoT CN internal architecture details and how A-IoT upper layer information is transported, if any. Details are subject to SA2 agreements and further studies.

##### 6.4.2.1.1	Solution 1: RRC based solution

A single communication paths is necessary for on the RAN-CN interface for the RRC based solution:

-	a per AIoT transaction execution of AIoT session resource control between the AIoT CN and the AIoT enabled gNB also carries AIoT upper layer information which is relayed to the involved AIoT enabled UEs by the AIoT enabled gNB.

The communication path is depicted below:

Figure 6.4.2.1.1-1: Communication path for the RRC based solution of Topology 2

Upon receiving XXAP: A-IoT related message from A-IoT CN, the A-IoT-enabled gNB transmits the related information towards the A-IoT-enabled UE via NR Uu RRC, and vice versa.

Figure 6.4.2.1.1-2: RRC based solution of Topology 2

##### 6.4.2.1.2	Solution 2: NAS based solution

Two communication paths are necessary on the RAN-CN interface for the NAS based solution:

1)	a per AIoT enabled UE provision of transparent provision of AIoT upper layer information to the AIoT enabled UE via the AIoT enabled gNB: A-IoT related messages between the A-IoTF and the A-IoT-enabled UE are carried via A-IoT-enabled UE’s DL/UL NAS packets, the A-IoT-enabled gNB handles the A-IoT-enabled UE’s NAS packets as legacy, i.e., using DL NAS Transport and UL NAS Transport procedures over NGAP.

2)	a per AIoT transacton execution of AIoT session resource control between the AIoTF and the AIoT enabled gNB by means of new signalling.

The communication paths are depicted below:

Figure 6.4.2.1.2-1: communication paths for the NAS based solution of Topology 2

The protocol stacks are depicted below:

Figure 6.4.2.1.2-2: Protocol stack for NAS based solution of Topology 2 – provision of AIoT upper layer information

Figure 6.4.2.1.2-3: Protocol Stack for NAS based solution of Topology 2 – execution of AIoT session resource control

Figure 6.4.2.1.2-2 assumes an “XXAP*” protocol, which, together with the assumed XXAP protocol functions for the AIoT session resource control results in the same set of protocol functions as the assumed XXAP protocol functions for the RRC option:

-	UE Reader specific protocol functions contained in XXAP for the RRC option (see Figure 6.4.2.1.2-2) are replicated in XXAP* for the NAS option

-	AIoT session resource control specific functions contained in XXAP for the RRC option can be found in XXAP for the NAS option (see Figure 6.4.2.1.2-3).

-	Due to the disassociation of communication paths of the NAS option, which is not given for the RRC option, additional protocol functions are needed in order to associate RRC, NAS and RAN-CN signalling to the same AIoT transaction.

Editor’s Note:	Whether the duplication of protocol functions for the NAS option in XXAP and XXAP* is desirable, or at least calls for a single point of specification (assuming that normative work is agreed) needs further consideration.

##### 6.4.2.1.3	Solution3: UP based solution

Two communication paths are necessary on the RAN-CN interface for the NAS based solution:

1)	transparent provision of AIoT upper layer information to the AIoT enabled UE via the AIoT enabled gNB: A-IoT related messages between the A-IoTF and the A-IoT-enabled UE are carried (inband) via A-IoT-enabled UE’s PDU Session resources.

2)	execution of AIoT session resource control between the AIoTF and the AIoT enabled gNB by means of new signalling.

The communication paths are depicted below (including also PDU Session Resource control entities):

Figure 6.4.2.1.3-1: communication paths for the UP based solution of Topology 2

The protocol stacks are depicted below:

Figure 6.4.2.1.3-2: Protocol stack for UP based solution of Topology 2 – provision of AIoT upper layer information

Figure 6.4.2.1.3-3: Protocol Stack for UP based solution of Topology 2 – execution of AIoT session resource control

Figure 6.4.2.1.3-2 assumes an “XXAP*” protocol, which, together with the assumed XXAP protocol functions for the AIoT session resource control results in the same set of protocol functions as the assumed XXAP protocol functions for the RRC option:

-	UE Reader specific protocol functions contained in XXAP for the RRC option (see Figure 6.4.2.1.1-2) are replicated in XXAP* for the UP option

-	AIoT session resource control specific functions contained in XXAP for the RRC option can be found in XXAP for the UP option (see Figure 6.4.2.1.3-3).

-	Due to the disassociation of communication paths of the UP option, which is not given for the RRC option, additional protocol functions are needed in order to associate RRC, UP and RAN-CN signalling to the same AIoT transaction.

Editor’s Note:	Whether the duplication of protocol functions for the NAS option in XXAP and XXAP* is desirable, or at least calls for a single point of specification (assuming that normative work is agreed) needs further consideration.

### 6.4.x	Support of DO-A communication

The following DO-A traffic characteristics are identified and need to be considered on top of DO-DTT related design to allow .

-	DO-A traffic is not triggered by the AIOT CN

-	DO-A traffic is typically required to be delivered immediately from RAN to the CN without any optimizing accumulation of DO-A traffic from other devices at AIoT RAN, resulting in a single-device transaction on XX.

-	Assuming no interaction with CN initiated AIoT procedures, providing DO-A caused AIoT application-level data from the device would require an UL/D2R communication step followed by an acknowledgment of the network in DL/R2D direction.

-	Such device triggered DO-A procedure on XX cannot be identified by a CN allocated transaction ID, but rather with by means of an AIoT RAN allocated transaction ID or a device ID.

-	After the successful execution of such simple 2-step communication, the DO-A procedure would terminate successfully.

Editor’s Note 1:	Further details and implications related to security aspects are FFS.

## 6.5	Impacts on CN-RAN interface

Editor’s Note: Corresponds to the first RAN3 objective in the SID, to identify necessary impacts on signaling and procedures for CN-RAN interface.

### 6.5.1	Information exchanged between A-IoT CN and A-IoT RAN

#### 6.5.1.1	Inventory

Inventory can be sent by the A-IoT CN for a single device, or a group of devices, or all devices.

The Inventory Request from the A-IoT CN to the A-IoT RAN, may include the following:

(1)	A-IoT Device Identification (to find a single device, a group of devices, or all devices)

Note 1:	The definition of this identification is out of RAN3 scope.

Editor’s Note 1: It is FFS whether A-IoT RAN needs to interpret/store/process it.

(2)	Scope of inventory request (e.g., a certain area in which the inventory is to be triggered)

Multiple individual A-IoT Device IDs (one ID per device) can be provided to the A-IoT CN via a single Inventory Report.

Editor’s Note 2: It is up to SA2 whether device ID is sent transparent or not.

#### 6.5.1.2	Command

Command can be sent by the A-IoT CN for a single device.

Editor’s Note 1: it is FFS for command on a group of devices, or all devices.

Editor’s Note 2: it is FFS whether A-IoT RAN can remain agnostic of the type of request from the A-IoT CN (need to differentiate command and inventory).

#### 6.5.1.3	A-IoT radio resource allocation in case of NAS/UP based solutions

In NAS/UP based solutions, A-IoT radio resources are requested in advance to the NAS/UP based communication with the A-IoT device upon CN request.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>