3GPP TSG-RAN WG3 Meeting #126	R3-247607

# Orlando, USA, November 18 – 22, 2024

Agenda item:	14.2

Source: 	Samsung

Title: 	Further discussion on support broadcast service for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of MBS broadcast service for NR NTN, and the agreement and open issues are captured as follows,

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

Check RAN2 progress on the geographical area information.

In this contribution, we further discuss these open issues.

Discussion

According to the R19 NR NTN WID, one of the objectives is to support MBS broadcast service. And the following details are provided in the WID,

Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]

- Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2]

- Specify the necessary signaling between CN and NG-RAN. [RAN3]

Regarding the first bullet, the motivation to work on enhancement is that for some cases the intended service area is expected to be smaller than the coverage of a Uu cell. So the SIB signalling needs to provide the intended service area more granular than Uu cell provided by NTN. According to the WID, it is up to RAN2 to determine the detailed content related to the intended service area in the SIB to be used. If a new SIB is determined to be introduced, RAN3 can focus on potential enhancement on new SIB coordination in case of split architecture.

More specifically, the open issues could be, e.g.

- Which node provides the intended service area related information contained in the new SIB?

- Which node encodes the new SIB, CU or DU?

- Whether the existing mechanism on SIB coordination over F1AP is enough for the new SIB?

According to RAN2 progress, the new SIB has not been introduced, and it is also possible to reuse the existing SIBs or RRC messages. Therefore, such open issues could be further checked once RAN2 has progress on the detailed contents of the SIB.

Proposal 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Regarding the second bullet, last several RAN3 meetings we have achieved the following agreements,

The mapped cell ID can be used to define MBS broadcast service in NTN.

RAN3 takes the assumption that geographical service area information can be transmitted from CN with the same format as defined in Uu.

According to the above agreements, during last several meetings, I think RAN3 has achieved a full package on the CN-RAN signalling to support MBS broadcast service. In addition, an LS in R3-245844 has been sent to SA2 and CT4 in order to trigger their work on such signalling design. So regarding the support of MBS broadcast service, we think what we can do is to wait for further progress from SA2 and CT4.

Recall that the objective of this WI is to specify signalling of the intended service area of a broadcast service. Based on the discussion last several meetings, companies still have different interpretation on whether the broadcast service is only limited to MBS broadcast service, or the broadcast service can also include emergency services. RAN3 has achieved consensus on to start with the work on MBS broadcast service. With the good progress of the support of MBS broadcast service, we think it is time to touch the discussion on how to support other broadcast services, especially the emergency services.

In our opinion, from the CN-RAN signalling perspective, the similar open issues can be discussed to support PWS broadcast services.

Recall that the in the current XnAP warning messages, a Warning Area List IE has already been defined as follows,

9.3.1.37	Warning Area List

This IE indicates the areas where the warning message needs to be broadcast or cancelled.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Warning Area | M |  |  |  |
| >E-UTRA Cell IDs |  |  |  |  |
| >>EUTRA CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>E-UTRA CGI | M |  | 9.3.1.9 |  |
| >NR Cell IDs |  |  |  |  |
| >>NR CGI List for Warning |  | 1..<maxnoofCellIDforWarning> |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  |
| >TAIs for Warning |  |  |  |  |
| >>TAI List for Warning |  | 1..<maxnoofTAIforWarning> |  |  |
| >>>TAI | M |  | 9.3.3.11 |  |
| >Emergency Area IDs |  |  |  |  |
| >>Emergency Area ID List |  | 1..<maxnoofEmergencyAreaID> |  |  |
| >>>Emergency Area ID | M |  | 9.3.1.48 |  |

Wherein the Emergency Area ID is defined as follows,

9.3.1.48	Emergency Area ID

This IE is used to indicate the area which has the emergency impact.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Emergency Area ID | M |  | OCTET STRING (SIZE(3)) | Emergency Area ID may consist of several cells. Emergency Area ID is defined by the operator. |

In current TS 38.300, it has clearly specified that the Cell Identity used for PWS corresponds to a Mapped Cell ID. So the first leftover issue to support PWS broadcast service is that whether the current mapped cell, TAI and emergency area ID is good enough to provide the exact broadcast area for emergency services.

Proposal 2: RAN3 is kindly asked to discuss how to support PWS broadcast services.

Conclusion

In this paper, we provide discussion on support MBS broadcast service for NR NTN. The following proposals are provided,

Proposal 1: Wait for RAN2 progress on the detailed contents to be introduced for the intended service area in order to further check whether the current mechanism on SIB coordination is enough in split architecture.

Proposal 2: RAN3 is kindly asked to discuss how to support PWS broadcast services.

References