3GPP TSG-RAN WG3 Meeting #126	R3-247608

# Orlando, USA, November 18 – 22, 2024

Agenda item:	14.3

Source: 	Samsung

Title: 	Further discussion on support of regenerative payload for NR NTN

Document for:	Discussion and decision

Introduction

Last meeting further discussed the support of regenerative payload for NR NTN, and the following consensus is achieved,

The supported TAIs of an on-board gNB could be provided to AMF by OAM/pre-configuration.

The current mechanisms including, e.g. multiple TNLA association, are sufficient for TNL management during feeder link switch.

Legacy NG HO procedure can be reused for inter AMF HO for NTN regenerative payload.

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

In this contribution, we further discuss the support of regenerative payload.

Discussion

Last RAN3 meeting continued to discuss the NG interface management. And the following open issue is captured,

RAN CONFIGURATION UPDATE procedure could be reused to support NG suspend/Resume, by introducing suspend/resume indicators to avoid frequent TNL disconnection and re-connection?

And another alternative is to introduce new NG suspend/resume procedures.

According to the discussion during last several RAN3 meetings, the proponents of NG suspend/resume mentioned that the introduction of NG suspend/resume will save transmission energy when recovering the connection between gNB and AMF since less information is expected to be transmitted compared to NG setup as well as the following up RAN/AMF Configuration Update procedures.

On the other hand, the introduction of NG suspend/resume will require much more storage on NG interface management related information for gNB on board, especially after several orbits, the gNB on board may need to store the NG interface related information with every AMF along the orbits. In addition, there’s also a concern that whether the original suspended information is still valid when the gNB on board connects back to a specific AMF.

In our understanding, considering that the supported TA list of an on-board gNB could be provided to AMF by OAM, the main information that is needed to be stored at the gNB onboard is the TNL association related information, which actually does not require too much storage space, even if the gNB onboard needs to maintain the interface related information with every AMF along the orbits.

In addition, we observe that the addition/update/removal of the TNLA is initiated by the AMF, and both AMF and gNB is aware of the ephemeris of the regenerative payload as well as the supported TA information for a gNB on board. As a consequence, we think it is also worth considering whether to introduce suspend/resume indicators to AMF Configuration Update procedure instead.

And we can immediately see a benefit when we introduce a resume indicator to AMF CONFIGURATION UPDATE message that it will potentially save the number of interface management messages to be exchanged when the feeder link recovers. More specifically,

- Reusing the NG Setup approach to recover the feeder link: Needs one Class 1 NG Setup procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Adding resume indicator in RAN CONFIGURATION UPDATE message approach to recover the feeder link: Needs one Class 1 RAN Configuration Update procedure + one Class 1 AMF Configuration Update procedure if there are TNLAs that are needed to be added/updated/removed.

- Our proposed approach: Only needs one Class 1 AMF Configuration Update procedure to add/update/remove TNLAs and recover the feeder link at the same time.

We think it is the simplest approach we can identify.

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

The next open issue is about whether the Xn interface can be established over feeder link, as well as whether the NG interface can be established over ISL.

And we take whether the NG interface can be established over ISL as an example.

From connection perspective, the current discussion only assumes that the feeder link is always available between NTN GW and gNB onboard. However, in potential deployment (which has also been mentioned by operators), it is not always guaranteed that the feeder link is always available for a specific operator. As a consequence, when the direct feeder link between the gNB onboard and the NTN GW is unavailable, we need to provide means to ensure that the NG interface can still be maintained in some cases e.g. when the atmospheric attenuation is too severe due to the bad weather. As an alternative approach, we can trmporarily use ISL to carry NGAP messages in order to maintain the connection.

Note that in our understanding, if the feeder link is available, it is more reasonable for the gNB onboard to use the direct feeder link to carry NGAP messages.

Proposal 2: It is possible to use ISL to carry NGAP messages.

The next open issue is about the LS received by RAN3 in R3-247016 which says that,

SA2 thanks for the LS on Support of Regenerative-based Satellite Access. SA2 identified that the LS from RAN3 referred only to “5GS” while in the last paragraph also referred to “AMF/MME”. SA2 also noticed that the RAN WI “Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” (RP-242397) did not contain objectives for Regenerative-based Satellite Access for E-UTRAN/EPC.

SA2 would like to inform RAN3 that the conclusions for FS_5GSAT_Ph3 for “KI#1: Support of Regenerative-based satellite access” without Store and Forward in clause 8.1 of TR 23.700-29 apply to both E-UTRAN/EPS and NG-RAN/5GS and therefore the related RAN3 impacts can affect both S1 and NG procedures.

It should be noted that we have introduced new NG Removal procedure in R19 NR NTN. And the LS from SA2 clearly shows that the similar S1 Removal procedure should also be discussed. Firstly, we see no issue to introduce S1 Removal procedure, the same reason to introduce NG Removal procedure can also be applied to the introduction of S1 Removal procedure. However, it is an open issue that in which WI we have such discussion, R19 IoT NTN WI or R19 NR NTN WI. I think RAN3 can make a decision during the online session.

Proposal 3: Introduce S1 Removal procedure to support the regenerative payload. FFS on which WI to handle potential new S1 procedures/mechanisms including S1 removal/suspend/resume.

Conclusion

In this paper, we provide primary discussion on support of regenerative payload for NR NTN. The following proposals are provided,

Proposal 1: Introduce Suspend/Resume indicators in AMF CONFIGURATION UPDATE message, to support NG suspend/resume.

Proposal 2: It is possible to use ISL to carry NGAP messages.

Proposal 3: Introduce S1 Removal procedure to support the regenerative payload. FFS on which WI to handle potential new S1 procedures/mechanisms including S1 removal/suspend/resume.

References