3GPP TSG-RAN WG3 Meeting #126	R3-247625

Orlando, USA, 18 – 22 November, 2024

Agenda item:	20.1

Source:	Nokia

Title:	Discussion on the LS on AI/ML Data Collection from SA2

Document for:	Discussion and Decision

# 1	Introduction

In this meeting, RAN3 received a reply LS from SA2 in [1]. This LS is a response to an LS that SA2 received from RAN in [2], capturing different solutions on UE side data collection that RAN2 has identified and analyzed for their work in AI/ML Air Interface. RAN also provided in their LS to SA2 a list of requirements for UE side data collection that the different solutions must meet. RAN asked SA2 to provide inputs on the various solutions identified by RAN2, indicating that input from SA is needed by RAN #106 in December 2024 when RAN will decide how to progress on this topic.

In [1], SA2 responded with actions both towards RAN2 and RAN3. In particular, SA2 asked RAN3 to consider the following question and provide feedback as soon as possible:

Q1: Are there any aspects of the UE-data collection controllability, that required NG-RAN involvement? If so, what is the involvement of NG-RAN in UE-data collection controllability, e.g., what aspects of MNO controllability would require NG-RAN involvement and what would such involvement be?

In this paper, we provide our views with respect to Q1. We also provide a draft Reply LS to SA2 in the annex of the paper.

# 2	Discussion

In [2], a number of solutions for UE side data collection that were identified and analyzed by RAN2 are summarized by means of a table shown below:

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |
| Impacted WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, SA2 |
| - Note 1: Full controllability: The MNO can manage data transfer to the server for UE-side data collection, without the need of SLA. This includes initiating, terminating, and fully managing data transfer. - Note 2: Visibility of data content signifies that the MNO can, at least, be aware of, access, and comprehend the data without the need of SLA. - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content (e.g. UE proprietary information can be included transparently together with the standardized data message). - No standardized visibility (e.g. only UE proprietary information can be included transparently). - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: RAN2 cannot reach consensus on the level of MNO controllability and visibility possible via solution 1b without input from SA groups. - Note 6: RAN2 has not concluded on the need for partial and no visibility options. - Note 7: RAN2 could not reach consensus on the feasibility of UP solution in options 2 and 3. |  |  |  |  |

In addition, RAN provided a number of requirements for UE side data collection in [2]:

RAN#105 way forward and requirements

RAN has agreed to the following requirements for data collection for UE sided model training for standardized solution (if standardized) (i.e. Option 1b, 2, 3). Option 1a is not precluded. - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is futureproof and extendable. FFS/study if and how to handle non-standardized data (i.e. partial visibility). FFS controllability on data collection Standardized Solutions should follow the principle of aiming to minimize air interface overhead and impact to NW operation

When it comes to Q1, NG-RAN involvement and controllability of the UE data collection is important for the efficient and reliable network operation and for keeping necessary control for the purpose of load management. Through NG-RAN controllability, MNO can control the amount of data to be collected from UEs to avoid disruption of normal network operation due to data collection, e.g., during overload. For example, NG-RAN data collection control on a per-cell granularity allows to stop or decrease data collection over highly loaded cells. NG-RAN controllability could also address overload scenarios through prioritization of different data collection sessions, e.g., similarly to QoE framework, by providing priority levels to different measurement configurations. NG-RAN controllability can further ensure that the collected data is provided to the MNO via OAM interface.

Observation 1: NG-RAN controllability can address overload scenarios during the data collection process, which is important for the efficient and reliable network operation and for keeping necessary control for load management.

From the listed Options, in Option 3 NG-RAN is involved in the configuration of the needed measurements to UE, such as RRC measurements. In addition, NG-RAN, knowing UE capabilities, may also select the most appropriate UEs for the data collection and optimize UE selection process.

Observation 2: Option 3 achieves the desired full controllability since NG-RAN is involved in configuring UEs with measurements, and may be involved in selecting the most appropriate UEs.

Based on the above, we propose to agree the Reply LS in the annex of this paper.

Proposal: Agree the Reply LS in the annex of this paper.

# 3	Conclusion

In this paper we make the following observations and proposal:

Observation 1: NG-RAN controllability can address overload scenarios during the data collection process, which is important for the efficient and reliable network operation and for keeping necessary control for load management.

Observation 2: Option 3 achieves the desired full controllability since NG-RAN is involved in configuring UEs with measurements, and may be involved in selecting the most appropriate UEs.

Proposal: Agree the Reply LS in the annex of this paper.

# References

[1]		R3-247017 - S2-2411191, Reply LS on AIML data collection, SA2

[2]		RP-242389 - S2-2409600, LS on AIML data collection, RAN

# Annex

3GPP TSG-RAN WG3 Meeting #126	R3-24xxxx

Orlando, USA, 18 – 22 November, 2024

Title:	[Draft] Reply LS on  AI/ML data collection

Response to:	R3-247017 - S2-2411191

Release:	Release 19

Work Item:	NR_AIML_Air

Source:	Nokia [to be RAN3]

To:	SA2, RAN2

CC :		TSG SA, TSG RAN, SA3, SA5

Contact Person:

#### Name:	Anna Pantelidou

E-mail Address:	anna.pantelidou@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN3 thanks SA2 for their LS.

NG-RAN involvement and controllability of the UE data collection is important for the efficient and reliable network operation and for keeping necessary control for the purpose of load management. Through NG-RAN controllability, MNO can control the amount of data to be collected from UEs to avoid disruption of normal network operation due to data collection, e.g., during overload. For example, NG-RAN data collection control on a per-cell granularity allows to stop or decrease data collection over highly loaded cells. NG-RAN controllability could also address overload scenarios through prioritization of different data collection sessions, e.g., similarly to QoE framework, by providing priority levels to different measurement configurations. NG-RAN controllability can further provide the collected data to the MNO via proprietary OAM interface.

Option 3 achieves the desired full controllability since NG-RAN is involved in configuring UEs with measurements, and may be involved in selecting the most appropriate UEs.

2. Actions:

To SA5 : 	RAN3 would like to ask SA2 to take the above into account and reply to RAN3, if needed.

3. Dates of Next TSG-RAN WG3 Meetings:

TSG-RAN WG3 Meeting #127				17 - 21    February 2025							Athens, Greece

TSG-RAN WG3 Meeting #127bis			  7 - 11 	April 2025								TBD, China